| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0468 |
| --- | --- |

| Title: | CE5-Related: Reducing multiplier count in CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Frank Bossen2, Andrew Segall1 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | 1Sharp Labs of America 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the Cross Component Adaptive Loop Filter (CC-ALF).  It is asserted that the changes  (i) reduce complexity and (ii)  further align CC-ALF with the regular ALF processes.  To maintain the coding efficiency of CC-ALF with these changes, this contribution further proposes to allow for four CC-ALF filters to be used per chroma component.

Here, complexity reduction is achieved by:

- Reducing the support of the filter to a 3x4 diamond shape with 8 unique coefficients.  This reduces the number of multiplications by 43% compared to the 5x6 diamond shape that was tested in CE5-2.1 (and that had 14 unique coefficients).

- Limiting the bit depth of the CC-ALF output by clipping it to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive

In addition, to better align CC-ALF with regular ALF, the following changes are also proposed in the contribution:

- Limit filter selection to signaling at a CTU level

- Removal of the temporal layer CC-ALF coefficient buffers to harmonize with the APS concept

- Use of symmetric line selection at virtual boundary to harmonize with ALF

The performance of the proposed changes are evaluated under the Common Test Conditions (CTC) and are reported to provide an average chroma gain of 8.93% and 11.46% for AI and RA cases, respectively.  Please note that a configuration that uses 6 unique coefficients combined with a zero-gain constraint will also be provided in a revision of this document as supplemental information.

# Introduction

In CE5-2.1 [1] and JVET-O0636 [2], the Cross-Component Adaptive Loop Filter (CC-ALF) was introduced and studied.  The filter uses a 5x6 diamond shape filter (see Fig. 1a) to filter luma sample values and generates a residual correction for the chroma channels from the co-located filtered output.  In the original CC-ALF design, the 5x6 diamond shape filter consisted of 14 unique coefficients.  This resulted in requiring 14 multiplication operation during filtering.

Here, we consider a solution that reduces the number of multiplications in the filtering process.  Specifically, we propose to use a 3x4 diamond shape filter (see fig. 1b) with 8 unique coefficients.  As the reduction in filter support leads to coding efficiency loss, we further propose to allow selecting the CC-ALF filter from four available filters for each chroma component.  By reducing the number of filters from 14 coefficients to 8 coefficients, we observe a 43% reduction in the number of multiplications.

| (a) | (b) |
| --- | --- |
| Fig. 1: (a) 5x6 Diamond Shape Filter with 14 unique coefficients (b) 3x4 Diamond Shape Filter with 8 unique coefficients |  |

To better align CC-ALF with regular ALF the following additional changes are proposed in the contribution:

- Limit filter selection to signaling at a CTU level

- Removal of the temporal layer CC-ALF coefficient buffers to harmonize with the APS concept

- Use of symmetric line selection at virtual boundary to harmonize with ALF

Finally, to limit the bit depth of storing the CC-ALF correction, the output is clipped to -2BitDepthC-1 to 2BitDepthC-1-1, inclusive.

The text changes to the VVC draft are included as an attachment to this contribution.

# Experimental Results

The proposed changes were implemented in VTM6 [3] and evaluated for the CTC [4].  A summary of the results is provided below.  As can be seen from the summary, the proposed approach provides coding efficiency improvements in the chroma channel for All Intra, Random Access, and Low Delay configurations.  Detailed results can be found in the attached Excel files.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | -5.33% | -5.61% | 101% | 103% |
| Class A2 | 0.18% | -10.01% | -14.85% | 102% | 104% |
| Class B | 0.19% | -15.86% | -12.05% | 100% | 102% |
| Class C | 0.19% | -4.31% | -3.96% | 100% | 101% |
| Class E | 0.29% | -11.64% | -2.13% | 101% | 102% |
| Overall | 0.21% | -9.86% | -7.99% | 101% | 102% |
| Class D | 0.09% | -3.43% | -1.87% | 101% | 100% |
| Class F | 0.19% | -6.30% | -6.70% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -5.96% | -6.42% | 100% | 100% |
| Class A2 | 0.25% | -12.45% | -18.80% | 99% | 101% |
| Class B | 0.19% | -20.45% | -15.33% | 100% | 100% |
| Class C | 0.11% | -4.61% | -3.84% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.17% | -11.73% | -11.18% | 100% | 100% |
| Class D | 0.10% | -4.51% | -3.11% | 98% | 99% |
| Class F | 0.20% | -4.67% | -4.70% | 100% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.07% | -5.63% | -4.53% | 101% | 101% |
| Class E | 0.07% | -15.79% | -3.85% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.09% | -4.50% | -2.72% | 100% | 100% |
| Class F | 0.25% | -8.82% | -6.76% | 101% | 103% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.17% | -5.62% | -4.41% | 99% | 101% |
| Class E | 0.25% | -16.47% | -4.05% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.15% | -4.63% | -3.05% | 99% | 99% |
| Class F | 0.26% | -8.18% | -6.51% | 99% | 101% |

# Supplemental Results

As supplemental information, we consider a configuration that further reduces the number of coefficients to six unique values.  In this scenario, we combine the filter shape shown in Figure 2 with a zero-gain constraint.

Fig. 2: Zero-gain filter with 6 unique coefficients

As noted above, results for this configuration will be provided in a revision of the document.

# Conclusion

The contribution proposes modifications to the CC-ALF process to (i) reduce the number of multiplications and (ii) further align the process with ALF.  In achieving these goals, the contribution also proposes to allow selection of the CC-ALF filter from four candidates for each chroma channel.  It was demonstrated that the proposed changes can provide average chroma gains of 8.93% and 11.46% for AI and RA cases, respectively.  Thus, the proposal has higher coding efficiency than reported in CE5.2.1, while asserted to have lower complexity.

It is recommended to adopt one of the reduced complexity filters proposed in this contribution to the VVC draft.

# References

[1] K. Misra, F. Bossen, A. Segall " CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter," JVET-O0636, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019

[2] K. Misra, F. Bossen, A. Segall "Cross-Component Adaptive Loop Filter for chroma," JVET-O0636, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019

[3] VVC reference software (VTM) version 6.0, available at: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).