| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0472 |
| --- | --- |

| Title: | Non-CE8: Palette Coding Encoder Improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Sunmi Yoo Seung Hwan Kim LG Electronics. | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com sunmi.yoo@lge.com seunghwan3.kim@lge.com |
| Source: |  |  |  |

_____________________________

# Abstract

This document proposes an encoder RD bug fix to palette coding in VTM6. Performances of the proposed bug fix are:

In Dual Tree Off Case:

AI: -3.46%, -7.43%, -8.26% (TGM1080: -6.29%	, -10.46%, -10.89%; TGM720: -4.82%, -8.36%,	-0.75%)

RA: -2.64%	, -6.62%, -7.28% (TGM1080: -4.53%, -8.81%, -9.06%; TGM720: -4.36%, -8.39%, -10.35%)

LD: - xxx%,  xxx%, xxx%  (TGM1080: - xxx%,  xxx%, xxx%;  TGM720: -3.16%,-6.18%,-7.98%)

In Dual tree On Case:

AI:  0.08%, -2.55%, -2.78% (TGM1080: 0.48%, -3.85%, -4.20%; TGM720: -0.61%, -3.17%, -3.58%)

RA: -0.30%	, -3.66%, -3.92% (TGM1080: -1.09%, -5.94%, -5.69%; TGM720: -0.63%; -3.97%; -5.01%)

LD: -0.90%	, -3.50%, -4.01% (TGM1080: -2.45%,-6.00%,-5.74%; TGM720: -1.30%,-3.82%,-5.41%)

There is no impact to the coding time.

# Introduction

VTM6 supports the palette mode to be used for screen content coding in 4:4:4 color format. If the palette mode is utilized, the sample values in the CU are represented by a small set of representative colour values. The set is referred to as the palette. For pixels with values close to the palette colors, the palette indices are signalled. For samples outside of the palette, they are signalled as escape values.

The palette table are determined by RD decision. In VTM6, in function derivePLTLossy where palette entries are determined, the palette distortion is scaled down to 8 bit scale regardless of the internal coding bit depth (PLT_ENCBITDEPTH is defined as 8 in VTM6). In VTm6, it’s derived as:

const int  pcmShiftRight_L = (channelBitDepth_L - PLT_ENCBITDEPTH);

const int  pcmShiftRight_C = (channelBitDepth_C - PLT_ENCBITDEPTH);

const int shift = (comp > 0) ? pcmShiftRight_C : pcmShiftRight_L;

bestCost += (err*err) / (1 << (2 * shift));

The palette distortion is scaled down to 8 bit scale. However, the Lagrangian cost parameter Lambda being used in calculating the bitCost is still determined by the internal coding bit depth which is 10 in CTC.

double bitCost = m_pcRdCost->getLambda()*numColorBits;

Therefore the rate and distortion to calculate the cost is not on the same scale. This mis-alignment of rate and distortion caused sub-optimal RD decision. This bug can be fixed by also dividing the lambda by (1 << (2 * shift)). Alternatively, it can also be done by not dividing the squared error by (1 << (2 * shift)).

# Experimental Result

Test are done on the top CE8 base software (VTM6 with some palette bug fixes), following CE8 test description [2] , using YUV444 sequences. Fixing this RD bug result in large RD gain.

Table 1. Result of Proposed RD fix, Dual Tree Off

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.29% | -10.46% | -10.89% | 100% | 94% |
| TGM 720p | -4.82% | -8.36% | -10.75% | 99% | 96% |
| Animation | -0.39% | -2.53% | -2.57% | 99% | 98% |
| Mixed content | -3.23% | -12.02% | -12.62% | 101% | 97% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -3.46% | -7.43% | -8.26% | 100% | 97% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -4.53% | -8.81% | -9.06% | 97% | 97% |
| TGM 720p | -4.36% | -8.39% | -10.35% | 98% | 99% |
| Animation | 0.34% | -1.15% | -1.25% | 100% | 99% |
| Mixed content | -2.54% | -11.21% | -11.69% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -2.64% | -6.62% | -7.28% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | -3.16% | -6.18% | -7.98% | 99% | 98% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | -2.15% | -11.30% | -12.24% | 100% | 99% |
| Camera-Captured content | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Result of Proposed RD fix, Dual Tree on

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.48% | -3.85% | -4.20% | 101% | 99% |
| TGM 720p | -0.61% | -3.17% | -3.58% | 101% | 101% |
| Animation | 0.21% | -1.16% | -0.90% | 101% | 100% |
| Mixed content | 0.41% | -3.08% | -3.53% | 101% | 99% |
| Camera-Captured content | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Overall | 0.08% | -2.55% | -2.78% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.09% | -5.94% | -5.69% | 98% | 98% |
| TGM 720p | -0.63% | -3.97% | -5.01% | 99% | 99% |
| Animation | 0.53% | -0.60% | -0.59% | 100% | 99% |
| Mixed content | 0.19% | -5.67% | -6.05% | 100% | 99% |
| Camera-Captured content | -0.02% | -0.02% | -0.01% | 100% | 99% |
| Overall | -0.30% | -3.66% | -3.92% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -2.45% | -6.00% | -5.74% | 99% | 97% |
| TGM 720p | -1.30% | -3.82% | -5.41% | 99% | 99% |
| Animation | 1.03% | -0.15% | -0.21% | 100% | 99% |
| Mixed content | -0.86% | -5.37% | -6.31% | 100% | 98% |
| Camera-Captured content | 0.01% | -0.12% | -0.03% | 99% | 99% |
| Overall | -0.90% | -3.50% | -4.01% | 99% | 98% |

# Conclusion

This contribution proposes encoder RD bug fix in VTM6 palette coding. It fixes the mis-alignment of rate and distortion during palette coding RD decision. It is proposed to include this bug fix into next VTM.

# Reference

- “Non-CE8: Minimum QP for Transform Skip Mode,” Document of Joint Video Experts Team (JVET), JVET-O0919, Gothenburg, SE, July 2019.

- “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools,” Document of Joint Video Experts Team (JVET), JVET-O2028, Gothenburg, SE, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-O0010, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).