| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0474 |
| --- | --- |

| Title: | Non-CE8: Minimum QP for escape mode in palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JVET-O0919, a minimum allowed quantization parameter is introduced for transform skip mode when the nearly lossless condition is reached. This document proposes to extend the idea for palette mode in coding the samples in escape mode. Specifically, the baseQP is clipped to the value 4 + 6*(internalBitDepth - inputBitDepth).

The results of the proposed bugfix on YUV444 sequences are as follows

Dual tree OFF:

Low QP (2, 7, 12, 17): xxx% AI, -0.33% RA, 0.03% LB

# Introduction

In the palette mode in VTM6.0, If the palette mode is utilized for a coding unit (CU), the pixels values in the CU are represented by a small set of representative colour values. The set is referred to as the palette. For pixels with values close to the palette colors, the palette indices are signalled. For pixels with values outside the palette, the pixel is denoted with an escape symbol and the quantized pixel values are signaled directly. In the escape mode quantization, the lossless condition holds when the Quantization Parameter (QP) is equal to:

4 – 6*(inputBitDepth-8)

When QP becomes smaller, the quantization stage introduces rounding errors. In JVET-O0919, a minimum QP clipped is proposed which clips the baseQP when the lossless condition is reached.

# Proposal

In this proposal, the method in JVET-O0919 is applied on the samples coded as escape mode in palette coded CU. The baseQP is clipped as follows:

baseQp = qpy + qpBdOffset;

baseQp = max (baseQp, 4 + 6* (internalBitDepth[channel] – inputBitDepth[channel]))

Since palette mode is allowed for chroma components in VTM6.0, additional minimum QP value for chroma is signaled in SPS.

# Simulation Results

The results are reported using the Low QPs test conditions defined in CE8 (QP = 2, 7, 12, 17 for the first 100 frames).

Table 1: Results of DualITree = 0 for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.08% | -0.94% | -0.98% | 101% | 97% |
| TGM 720p | 0.03% | -0.30% | -0.30% | 100% | 102% |
| Animation |  |  |  |  |  |
| Mixed content | 0.06% | -0.27% | -0.31% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall |  |  |  |  |  |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.57% | -0.55% | -0.54% | 101% | 101% |
| TGM 720p | -0.45% | -0.41% | -0.43% | 100% | 99% |
| Animation | 0.00% | -0.01% | -0.01% | 100% | 97% |
| Mixed content | -0.40% | -0.38% | -0.40% | 100% | 102% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 105% |
| Overall | -0.33% | -0.31% | -0.32% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | -0.30% | -0.29% | 100% | 100% |
| TGM 720p | 0.11% | -0.19% | -0.17% | 100% | 99% |
| Animation | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Mixed content | 0.05% | -0.33% | -0.34% | 100% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.03% | -0.18% | -0.18% | 100% | 100% |

# Reference

- M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban, “Non-CE8: Minimum QP for Transform Skip Mode”, JVET-O0919, 15th meeting, Gothenburg, SE, July 2019.

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou ,W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, “CE8-2.1: Palette mode in HEVC” , JVET-O0119, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)