| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0475 |
| --- | --- |

| Title: | Non-CE8: Simplification of palette predictor update for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes two methods to simplify the palette predictor update for small coding units (CU size < 8x8) to solve the issue of pipeline latency. In the first method, the elements in the palette predictor that are allowed to be used in predicting palette table is restricted to the first W x H elements (W and H stands for width and height of the CU). In the second method, palette predictor update is bypasses for CU size < 64.

The results on YUV4:4:4 sequences for dual tree OFF show only minor difference compared to VTM6.0:

Method 1:

standard QP (22, 27, 32, 37): 0.00% AI, -0.03% RA, 0.08% LB

Method 2:

standard QP (22, 27, 32, 37): 0.09% AI, -0.01% RA, 0.05% LB

# Introduction

In VTM6.0, If the palette mode is utilized, the pixels values in the CU are represented by a small set of representative colour values. The set is referred to as the palette table. For coding of the palette table, a palette predictor is maintained. For each entry in the palette predictor, a reuse flag is signaled to indicate whether it is part of the current palette. After encoding the a palette coded CU, the palette predictor will be updated using the current palette, and entries from the previous palette predictor which are not reused in the current palette will be added at the end of new palette predictor sequencitally until the maximum size allowed (63 in VTM6.0) is reached. The process is called palette stuffing. Figure 1 and 2 illustrate the palette table derivation and the update of palette predictor.

Figure 1: Palette table derivation from palette predictor (entries marked as blue) and new signaled colours

Figure 2: Palette predictor update with current palette table and un-used palette predictor entries (the ordering is maintained).

The palette stuffing, however, may takes multiple cycles to complete because the encoder/decoder needs to check sequentially whether each entry in the predictor is used to predict the palette table. After checking predictor entries, fill in the updated color entries will also be sequential process. Since the maximum palette predictor size currently set in VTM6.0 is 63 and the predictors have to be updated before encoding/decoding the next palette coded coding units, for small blocks such as 4x4, 4x8, or 8x4, the palette predictor update process can be the bottleneck and cause latency in the pipeline.

# Proposed Method

## Method 1

In the first method proposed, the elements in the palette predictor that are allowed to be used in predicting palette table is restricted to the first W x H elements (W and H stands for width and height of the CU). Figure 3 illustrates the process.

Figure 3: Illustration of palette stuffing when the prediction entries used for prediction is restricted to the first WxH elements

As a result, the number of predictor entries encoder/decoder needs to check (whether the element is used for prediction or not) for palette stuffing is reduced to WxH. For predictor entries after the WxH’th position (marked as grey in the figure), since the entries are not used for prediction, the elements can be directly copied until the maximum palette size is reached.

## Method 2

In the second method proposed, palette predictor update is bypasses for CU size < 64, namely, the palette predictor will stay the same as the previous palette predictor.

# Simulation Results

The results are reported using the test conditions defined in CE8 description [2]. IBC, BDPCM, HashME, and PLT are turned ON for all the YUV4:4:4 sequences. The results for method 1 and method 2 are shown in Table 1 and 2.

Table 1: Results of method 1(restricting elements used for predictor) for DualITree=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | -0.01% | -0.01% | 101% | 101% |
| TGM 720p | 0.00% | -0.01% | -0.07% | 101% | 100% |
| Animation | 0.03% | 0.03% | 0.02% | 101% | 100% |
| Mixed content | -0.01% | -0.09% | -0.13% | 103% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | -0.02% | -0.04% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | -0.11% | -0.02% | 101% | 101% |
| TGM 720p | -0.06% | 0.05% | 0.08% | 102% | 99% |
| Animation | 0.03% | 0.01% | -0.02% | 102% | 100% |
| Mixed content | -0.08% | -0.18% | -0.27% | 101% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall | -0.03% | -0.04% | -0.04% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.18% | 0.12% | 0.08% | 102% | 101% |
| TGM 720p | 0.14% | -0.10% | 0.39% | 103% | 102% |
| Animation | 0.00% | 0.01% | 0.02% | 100% | 99% |
| Mixed content | 0.00% | -0.02% | -0.37% | 103% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Overall | 0.08% | 0.00% | 0.05% | 102% | 100% |

Table 2: Results of method 2 (bypass predictor update) for DualITree=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.21% | 0.22% | 0.11% | 100% | 100% |
| TGM 720p | 0.03% | 0.10% | -0.11% | 101% | 102% |
| Animation | 0.09% | 0.09% | 0.15% | 101% | 99% |
| Mixed content | 0.06% | 0.00% | -0.07% | 102% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.09% | 0.10% | 0.01% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.02% | 0.03% | 100% | 100% |
| TGM 720p | 0.01% | -0.03% | 0.28% | 102% | 102% |
| Animation | 0.01% | -0.01% | -0.03% | 103% | 101% |
| Mixed content | -0.07% | 0.04% | -0.08% | 101% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Overall | -0.01% | 0.00% | 0.06% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.18% | 0.03% | 100% | 98% |
| TGM 720p | 0.16% | 0.24% | 0.40% | 105% | 100% |
| Animation | -0.01% | 0.06% | 0.05% | 103% | 98% |
| Mixed content | -0.04% | -0.16% | 0.00% | 99% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Overall | 0.05% | 0.09% | 0.12% | 102% | 99% |

# Reference

- Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou ,W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, “CE8-2.1: Palette mode in HEVC” , JVET-O0119, 15th meeting, Gothenburg, SE, July 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)