| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0477-v8 |
| --- | --- |

| Title: | AHG18: On Last Position Signaling for Lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hilmi E. Egilmez, Muhammed Coban, Marta Karczewicz Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang | Tel: Email: | analci@qti.qualcomm.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Qualcomm Incorporated, MediaTek Inc., and Kwai Inc., |  |  |

_____________________________

# Abstract

In AHG18 VTM-6.0-lossless branch, transquant bypass (QB) mode provides a lossless alternative in VVC. This can be achieved by setting a CU-level cu_transquant_bypass_flag=1. In the lossy version of VVC, last position of primary transform coefficients is explicitly signaled and used in various inference schemes for LFNST and signaling of significant flags/groups. In the lossless setting, last position signaling may carry signaling overhead. This is because transform coefficients are in the frequency domain and lossless residuals are in the spatial domain and lossless residuals are not necessarily clustered in the top-left corner of a transform block.

In proposal #1, we disable last position signaling for lossless. The simulation results show overall bit-rate savings of -0.59% AI, -0.47% RA, and -0.53% LDB.

In proposal #2, we disable last position signaling on top of JVET-P0082 Proposal #1. In this case BDPCM is enabled for lossless for both TS and transform residual coding. The simulation results show overall bit-rate savings of -1.83% AI, -0.76% RA, and -1.01% LDB.

In proposal #3, we disable last position signaling on top of JVET-P0082 Proposal #4. In this case BDPCM is enabled for lossless for both TS and transform residual coding, additionally chroma BDPCM is allowed. The simulation results show overall bit-rate savings of -2.68% AI, -0.92% RA, and -1.15% LDB.

In proposal #4, we disable last position signaling on top of JVET-P0258 Proposal #1. In this additional test we disable dependent quantization for lossless blocks.

# Proposal

In AHG18 VTM-6.0-lossless branch, trans quant bypass (QB) mode provides a lossless alternative in VVC  by setting cu_transquant_bypass_flag=1 [1, 2]. In this proposal we propose to disable last position signaling when a CU is lossless coded. Specifically, last position is assumed to be at the right bottom location of a transform block and hence is not signaled. As lossy mode of VVC uses an inference scheme for significant sub-blocks (coded sub-block flag) and significance flag we, always signal significant sub-blocks and significant flags in the proposed method. The only exception is the first sub-block for which significance is not signalled and inferred to be 1.0 as in VVC.

The following variants are proposed:

- Proposal 1: Disable Last Position Signaling for Lossless

- Proposal 2: Disable Last Position on top of JVET-P0082 Proposal #1

  - In JVET- P0082 Proposal #1, BDPCM is applied to both in TS residual coding and transform residual (TR) coding.

- Proposal 3: Disable Last Position on top of JVET- P0082 Proposal #4

  - JVET-P0082 Proposal #4, BDPCM is applied to both in TS residual coding and transform residual (TR) coding and chroma BDPCM is applied to only TS residual coding.

- Proposal 4: We provide additional results for disable last position signaling on top of JVET- P0258 Proposal #1 in which dependent quantization is disabled.

# Results

Anchor is AHG18 VTM-6.0-lossless branch. The following settings are used to run both the anchor and proposals: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. Moreover, QP value was also set as 0. Additionally, for proposals #2, BDPCM : 1 configuration was used for all sequences e.g. A1, A2,…TGM. For proposals #3 we used BDPCM:  2 configuration for all sequences.

Proposal 1: Disable Last Position Signaling for Lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #1 |  | AHG18 LL | Proposal #1 |  | AHG18 LL | Proposal #1 |  |
| Class A1 | 2.2 | 2.2 | -0.41% | 2.3 | 2.3 | -0.40% |  |  |  |
| Class A2 | 1.7 | 1.8 | -0.44% | 1.9 | 1.9 | -0.36% |  |  |  |
| Class B | 2.1 | 2.1 | -0.50% | 2.3 | 2.3 | -0.51% | 2.3 | 2.3 | -0.50% |
| Class C | 2.1 | 2.1 | -0.62% | 2.6 | 2.6 | -0.55% | 2.6 | 2.6 | -0.55% |
| Class D | 1.9 | 2.0 | -0.59% | 2.8 | 2.8 | -0.56% | 2.7 | 2.8 | -0.56% |
| Class E | 2.9 | 3.0 | -1.01% |  |  |  | 3.3 | 3.3 | -0.57% |
| Class F | 5.5 | 5.5 | -0.13% | 39.4 | 39.2 | 0.01% | 60.7 | 60.2 | 0.09% |
| TGM | 12.3 | 12.1 | 1.01% | 112.6 | 111.6 | 0.93% | 142.4 | 141.0 | 1.04% |
| Overall | 2.2 | 2.2 | -0.59% | 2.3 | 2.3 | -0.47% | 2.6 | 2.7 | -0.53% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 98% |  |  | 98% |  |  |

Table 1. Results after disabling last position signaling for lossless.

Proposal 2: Disable Last Position Signaling for Lossless on top of JVET-P0082 Proposal #1

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #2 |  | AHG18 LL | Proposal #2 |  | AHG18 LL | Proposal #2 |  |
| Class A1 | 2.2 | 2.3 | -1.27% | 2.3 | 2.3 | -0.76% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.56% | 1.9 | 1.9 | -0.62% |  |  |  |
| Class B | 2.1 | 2.1 | -1.80% | 2.3 | 2.3 | -0.76% | 2.3 | 2.3 | -0.78% |
| Class C | 2.1 | 2.1 | -2.00% | 2.6 | 2.6 | -0.88% | 2.6 | 2.6 | -0.92% |
| Class D | 1.9 | 2.0 | -3.12% | 2.8 | 2.8 | -1.06% | 2.7 | 2.8 | -1.02% |
| Class E | 2.9 | 3.0 | -2.49% |  |  |  | 3.3 | 3.4 | -1.51% |
| Class F | 5.5 | 5.7 | -3.96% | 39.4 | 41.2 | -2.86% | 60.7 | 64.2 | -3.26% |
| TGM | 12.3 | 12.6 | -2.83% | 112.6 | 115.2 | -2.14% | 142.4 | 145.6 | -2.04% |
| Overall | 2.2 | 2.2 | -1.83% | 2.3 | 2.3 | -0.76% | 2.6 | 2.7 | -1.01% |
| Enc Time[%] | 117% |  |  | 114% |  |  | 116% |  |  |
| Dec Time[%] | 89% |  |  | 98% |  |  | 98% |  |  |

Table 2. Results after disabling last position signaling for lossless on top of JVET-P0082 Proposal #1. BDPCM is enabled for both TS residual coding and transform residual coding.

Proposal 3: Disable Last Position Signaling for Lossless on top of JVET-P0082 Proposal #4

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #3 |  | AHG18 LL | Proposal #3 |  | AHG18 LL | Proposal #3 |  |
| Class A1 | 2.2 | 2.3 | -2.07% | 2.3 | 2.3 | -1.10% |  |  |  |
| Class A2 | 1.7 | 1.8 | -2.00% | 1.9 | 1.9 | -0.66% |  |  |  |
| Class B | 2.1 | 2.1 | -2.42% | 2.3 | 2.3 | -0.89% | 2.3 | 2.3 | -0.89% |
| Class C | 2.1 | 2.1 | -2.76% | 2.6 | 2.6 | -1.04% | 2.6 | 2.6 | -1.04% |
| Class D | 1.9 | 2.0 | -4.25% | 2.8 | 2.8 | -1.28% | 2.7 | 2.8 | -1.18% |
| Class E | 2.9 | 3.1 | -4.28% |  |  |  | 3.3 | 3.4 | -1.71% |
| Class F | 5.5 | 5.9 | -5.59% | 39.4 | 41.7 | -3.66% | 60.7 | 64.7 | -3.82% |
| TGM | 12.3 | 13.1 | -5.98% | 112.6 | 116.4 | -3.08% | 142.4 | 146.4 | -2.65% |
| Overall | 2.2 | 2.3 | -2.68% | 2.3 | 2.3 | -0.92% | 2.6 | 2.7 | -1.15% |
| Enc Time[%] | 121% |  |  | 115% |  |  | 118% |  |  |
| Dec Time[%] | 85% |  |  | 98% |  |  | 99% |  |  |

Table 3. Results after disabling last position signaling for lossless on top of JVET-P0082 Proposal (#4). BDPCM is enabled for both TS residual coding and transform residual coding and chroma BDPCM is enabled for transform skip residual coding.

Proposal 4: Disable Last Position Signaling for Lossless on top of JVET-P0258 Proposal #1 (Additional Results)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 LL | Proposal #4 |  | AHG18 LL | Proposal #4 |  | AHG18 LL | Proposal #4 |  |
| Class A1 | 2.2 | 0.0 | -300.00% | 2.3 | 0.0 | -300.00% |  |  |  |
| Class A2 | 1.7 | 0.0 | -300.00% | 1.9 | 0.0 | -300.00% |  |  |  |
| Class B | 2.1 | 2.1 | -0.64% | 2.3 | 2.2 | -400.88% | 2.3 | 0.0 | -500.00% |
| Class C | 2.1 | 2.1 | -0.80% | 2.6 | 2.6 | -0.97% | 2.6 | 2.4 | -300.82% |
| Class D | 1.9 | 2.0 | -0.80% | 2.8 | 2.8 | -1.05% | 2.7 | 2.8 | -1.04% |
| Class E | 2.9 | 3.0 | -1.11% |  |  |  | 3.3 | 0.0 | -300.00% |
| Class F | 5.5 | 5.5 | -0.49% | 39.4 | 50.6 | -33.76% | 60.7 | 116.6 | -100.06% |
| TGM | 12.3 | 12.2 | 0.49% | 112.6 | 112.4 | 0.21% | 142.4 | 186.0 | -33.26% |
| Overall | 2.2 | 1.5 | -33.87% | 2.3 | 0.9 | -66.98% | 2.6 | 0.2 | -91.73% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

Table 4. Results after disabling last position signaling for lossless on top of JVET-P0258 Proposal (#1). Dependent Quantization is disabled for lossless.

(*Additional results pending)

# Conclusion

In this contribution we proposed to disable last position signaling for lossless coding. We presented our results on top of JVET-P0082 Proposals #1 and #4. Our results indicate that disabling last position provides overall bit-rate savings.

It is recommended to disable last position signalling for lossless coding in VVC.

# Spec Changes

#### 7.3.8.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if ( cu_transquant_bypass_flag ) { |  |  |
| LastSignificantCoeffX = ( 1 << log2TbWidth ) - 1 |  |  |
| LastSignificantCoeffY = ( 1 << log2TbHeight ) - 1 |  |  |
| } |  |  |
| else{ |  |  |
| if( log2TbWidth > 0 ) |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| } |  |
| … |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ¦ ¦ cu_transquant_bypass_flag ) && ( i > 0 ) ) |  |
| { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| … |  |

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred as follows.

- If cu_transquant_bypass_flag is equal to 1, it is inferred to be ( 1 << log2TbWidth )  - 1.

- Otherwise, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred as follows.

- If cu_transquant_bypass_flag is equal to 1, it is inferred to be ( 1 << log2TbHeight ) - 1.

- Otherwise, it is inferred to be 0.

# References

- AHG18 VTM-6.0-lossless, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting, Gothenburg, SE, June 2019.

- JVET-P0082 “AHG18:  BDPCM for Lossless”, A. Nalci, H. Wang, H. E. Egilmez, M. Coban, and M. Karczewicz (Qualcomm)

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).