| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 2–12 October 2019 | Document: JVET-P0493_v2 |
| --- | --- |

| Title: | CE6-related: Further simplification with new LFNST transform basis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomonori Hashimoto Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is related to CE6-2 which pursues LFNST simplification by reducing matrix size to 16x36 (CE6-2.3a), reducing mode number from 4 to 3 (CE6-2.1) and reusing larger blocks’ matrix for smaller blocks (CE6-2.2). This contribution reports further simplification.

One aspect is table reduction.

- Test1: combination of three, CE6-2.1 and CE6-2.2a and CE6-2.3a

The other is further computation reduction by using 16x32 matrices instead of VVC6’s 16x48 and CE6-2.3a’s 16x36 matrices.

- Test2 is a combination of CE6-2.2a and 16x32 matrices.

- Test3 is a combination of CE6-2.2a and 16x32 matrices without switched scan order.

- Test4 is a combination of CE6-2.2a and CE6-2.1 and 16x32 matrices without switched scan order.

The following is summary of the three tests.

|  | Reduce mode | reuse matrix | reduce matrix | Memory cost (KB) | Computation for 8xN, Nx8 | IO | RA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM 6 | - | - | - | 8 | 16x48 | 0 | 0 |
| CE6-2.1 | x | - | - | 6 |  | 0.03 | -0.01 |
| CE6-2.2a | - | x | - | 6 |  | -0.02 | -0.02 |
| CE6-2.3a | - | - | x | 6.7 | 16x36 | 0.02 | 0.03 |
| CE6-2.2c | x | x |  | 4.6 | 16x48 | 0.01 | -0.01 |
| CE6-2.2d |  | x | x | 4.6 | 16x36 | -0.01 | -0.02 |
| Test1 | x | x | x | 3.5 | 16x36 | 0.02 | -0.01 |
| Test2 | - | x | x | 4.1 | 16x32 | 0.01 | 0.01 |
| Test3 | - | x | x *1) | 4.1 | 16x32 | 0.01 | 0.00 |
| Test4 | x | x | x *1) | 3.1 | 16x32 | 0.05 | 0.02 |

*1) without switched scan order

- Proposal

Details for each test’s table size is summarized below.

|  | LFNST kernel size | Memory cost |
| --- | --- | --- |
| VTM 6 | 16×16 | 2 KB (16×16×2×4) |
|  | 16×48 | 6 KB (16×48×2×4) |
|  | 8 KB |  |
| Test1 | 16×16 | remove |
|  | 16×36 | 4.1 KB (16×36×2×3) |
|  | 3.5 KB |  |
| Test2&Test3 | 16×16 | remove |
|  | 16×32 | 3.5 KB (16×32×2×4) |
|  | 4.1 KB |  |
| Test4 | 16×16 | remove |
|  | 16×32 | 3.5 KB (16×32×2×3) |
|  | 3.1 KB |  |

Test1 (combination of CE6-2.1 and CE6-2.2a and CE6-2.3a)

In Test1, the 16x36 kernels in CE6-2.3a [1] are also applied in all block sizes (both 4xN and Nx4 blocks in addition to larger blocks) by using CE6-2.2a [2] method, which reuses larger block kernels in 4xN and Nx4 blocks. The number of LFNST transform sets is reduced from four to three (i.e., removing transform set 2) by using CE6-2.1 [3] method.

Test2

The proposed method test2 is to replace 16x48 matrices with smaller 16x32 matrices to achieve table size and computation reduction. The LFNST matrices have been retrained using non-CTC sequences and the training method is in the same way as CE6-2.3a [1]. The new kernels in Test1 is also applied in all block sizes (both 4xN and Nx4 blocks in addition to larger blocks) by using CE6-2.2a method, which reuses larger block kernels in 4xN and Nx4 blocks. In total, 4.1 KB is used (49 % reduction).

Test2 chooses 32 coefficients depending on lfnstIdx. Specifically, if lfnstIdx is equal to 1, the pattern in Fig 2 (Type 1) is used while if lfnstIdx is equal to 2, the pattern as shown in Fig 3 (Type 2) is used.

7 5 5 5 4 4 1 1 {7, 5, 5, 5, 4, 4, 1, 1} Type 1

Fig. 1: 32 coefficients pattern (proposal, lfnstIdx == 1)

5 5 5 5 4 4 4 0 {5, 5, 5, 5, 4, 4, 4, 0} Type 2

Fig. 2: 32 coefficients pattern (proposal, lfnstIdx == 2)

Test3

In Test3, the new kernels in Test2 is also applied in all block sizes (both 4xN and Nx4 blocks in addition to larger blocks) by using CE6-2.2a method, which reuses larger block kernels in 4xN and Nx4 blocks. In total, 4.1 KB is used (49 % reduction). The corresponding 32 coefficients as shown in Fig 3 (Test3 has no switched scan).

5 5 5 5 4 4 4 0 {5, 5, 5, 5, 4, 4, 4, 0}

Fig. 3: 32 coefficients pattern

Test4 (combination of CE6-2.1 and CE6-2.2a and Test3)

In Test4, the new kernels in Test3 is also applied in all block sizes (both 4xN and Nx4 blocks in addition to larger blocks) by using CE6-2.2a [2] method, which reuses larger block kernels in 4xN and Nx4 blocks. The number of LFNST transform sets is reduced from four to three (i.e., removing transform set 2) by using CE6-2.1 [3] method. The corresponding 32 coefficients as shown in Fig 3 (Test4 has no switched scan).

- Training procedure (same as CE6-2.3a)

The training method is in the same way as CE6-2.3a [1].

The training procedure is as follows:

- Produce training data by encoding non-CTC training sequence set with VTM-6.0.

- Derive 32x32 kernels using PCA function in sklearn [6] decomposition

- Quantize and integerize 16x32 kernels with signed 7 bit precision.

|  | Tester |
| --- | --- |
| Training sequence set | 100 frame 1920x1080 10bit yuv sequences generated from DIV2K [5] |
| Training software | Python3 script using PCA function in sklearn decomposition |
| VTM sw version used for retrieving training data | VTM-6.0 |

- Experimental Results

The simulations were conducted in the condition of ‘All Intra’, ‘Random Access’ defined in JVET-N1010 [4]. Table 1 shows the CTC and Low QP (2, 7, 12, and 17) results of test1. Table 2 shows the CTC and Low QP (2, 7, 12, and 17) results of test2. Table 3 shows the CTC and Low QP (2, 7, 12, and 17) results of test3. Table 4 shows the CTC and Low QP (2, 7, 12, and 17) results of test4.

Table 1. Result of proposed Test1 (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.31% | 0.36% | 99% | 100% |
| Class A2 | 0.01% | 0.38% | 0.36% | 99% | 100% |
| Class B | 0.02% | 0.03% | 0.17% | 100% | 100% |
| Class C | -0.01% | 0.00% | 0.26% | 100% | 100% |
| Class E | 0.04% | 0.55% | 0.46% | 100% | 100% |
| Overall | 0.02% | 0.22% | 0.30% | 100% | 100% |
| Class D | -0.02% | 0.24% | 0.19% | 100% | 99% |
| Class F | 0.03% | 0.16% | 0.15% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.18% | 0.17% | 100% | 100% |
| Class A2 | -0.01% | 0.09% | 0.02% | 100% | 100% |
| Class B | -0.01% | 0.06% | 0.08% | 100% | 100% |
| Class C | -0.05% | 0.11% | 0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.10% | 0.11% | 100% | 100% |
| Class D | -0.05% | 0.08% | 0.32% | 100% | 100% |
| Class F | 0.01% | 0.07% | 0.21% | 100% | 100% |

Result of proposed Test1 (Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.04% | 99% | 100% |
| Class A2 | 0.01% | 0.00% | -0.01% | 99% | 100% |
| Class B | 0.05% | -0.04% | 0.03% | 98% | 100% |
| Class C | 0.05% | -0.04% | 0.01% | 98% | 100% |
| Class E | 0.13% | 0.01% | 0.04% | 98% | 100% |
| Overall | 0.05% | -0.01% | 0.02% | 98% | 100% |
| Class D | 0.06% | -0.05% | 0.03% | 99% | 100% |
| Class F | 0.12% | 0.03% | 0.09% | 99% | 100% |

Table 2. Result of proposed Test2 (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.29% | 0.28% | 99% | 100% |
| Class A2 | -0.01% | 0.14% | 0.16% | 99% | 100% |
| Class B | 0.00% | 0.08% | 0.13% | 99% | 100% |
| Class C | -0.02% | -0.02% | 0.22% | 100% | 100% |
| Class E | 0.02% | 0.10% | 0.01% | 100% | 100% |
| Overall | 0.01% | 0.11% | 0.16% | 100% | 100% |
| Class D | -0.02% | 0.33% | 0.23% | 101% | 99% |
| Class F | -0.01% | 0.05% | -0.07% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.12% | 0.12% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.12% | 100% | 100% |
| Class B | 0.01% | 0.06% | 0.12% | 100% | 100% |
| Class C | -0.02% | -0.05% | 0.31% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.17% | 100% | 100% |
| Class D | -0.07% | 0.09% | 0.20% | 100% | 100% |
| Class F | -0.01% | 0.00% | 0.12% | 100% | 100% |

Result of proposed Test2 (Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.12% | 0.10% | 98% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.03% | 0.01% | 0.03% | 98% | 100% |
| Class C | 0.03% | -0.01% | 0.03% | 97% | 100% |
| Class E | 0.08% | 0.10% | 0.09% | 98% | 100% |
| Overall | 0.03% | 0.04% | 0.04% | 98% | 100% |
| Class D | 0.03% | -0.03% | 0.03% | 98% | 100% |
| Class F | 0.09% | 0.02% | 0.06% | 99% | 100% |

Table 3. Result of proposed Test3 (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.28% | 0.29% | 100% | 100% |
| Class A2 | -0.02% | 0.22% | 0.14% | 99% | 100% |
| Class B | 0.01% | 0.09% | 0.07% | 99% | 100% |
| Class C | -0.03% | -0.03% | 0.28% | 100% | 100% |
| Class E | 0.03% | 0.14% | 0.07% | 100% | 100% |
| Overall | 0.01% | 0.12% | 0.16% | 100% | 100% |
| Class D | -0.03% | 0.19% | -0.01% | 100% | 99% |
| Class F | -0.03% | 0.10% | -0.01% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.07% | 0.10% | 100% | 100% |
| Class A2 | -0.01% | 0.11% | 0.12% | 100% | 100% |
| Class B | -0.01% | 0.11% | -0.10% | 100% | 100% |
| Class C | -0.04% | 0.10% | 0.16% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.10% | 0.05% | 100% | 100% |
| Class D | -0.04% | -0.33% | -0.04% | 100% | 100% |
| Class F | 0.00% | -0.06% | 0.10% | 100% | 100% |

Result of proposed Test3 (Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | 0.09% | 98% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.03% | 0.02% | 0.03% | 98% | 100% |
| Class C | 0.03% | -0.01% | 0.01% | 98% | 100% |
| Class E | 0.09% | 0.09% | 0.09% | 98% | 99% |
| Overall | 0.03% | 0.03% | 0.04% | 98% | 100% |
| Class D | 0.03% | -0.03% | 0.07% | 98% | 100% |
| Class F | 0.03% | 0.01% | 0.01% | 99% | 100% |

Table 4. Result of proposed Test4 (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.38% | 0.41% | 99% | 100% |
| Class A2 | 0.02% | 0.29% | 0.34% | 100% | 100% |
| Class B | 0.04% | 0.06% | 0.13% | 99% | 100% |
| Class C | 0.00% | 0.07% | 0.23% | 100% | 100% |
| Class E | 0.06% | 0.59% | 0.37% | 100% | 100% |
| Overall | 0.05% | 0.24% | 0.27% | 100% | 100% |
| Class D | -0.02% | 0.18% | 0.24% | 100% | 99% |
| Class F | 0.04% | 0.16% | 0.10% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.21% | 0.25% | 100% | 100% |
| Class A2 | 0.01% | 0.16% | 0.10% | 100% | 100% |
| Class B | 0.02% | 0.08% | 0.03% | 100% | 100% |
| Class C | -0.01% | 0.13% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.14% | 0.10% | 100% | 100% |
| Class D | -0.03% | 0.12% | -0.01% | 100% | 100% |
| Class F | -0.02% | 0.15% | 0.13% | 100% | 100% |

Result of proposed Test4 (Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.00% | 0.04% | 98% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 98% | 100% |
| Class B | 0.05% | -0.03% | 0.01% | 98% | 100% |
| Class C | 0.04% | -0.05% | 0.01% | 98% | 100% |
| Class E | 0.14% | -0.02% | 0.03% | 98% | 100% |
| Overall | 0.05% | -0.02% | 0.02% | 98% | 100% |
| Class D | 0.05% | 0.01% | 0.04% | 98% | 100% |
| Class F | 0.05% | 0.03% | 0.08% | 99% | 100% |

- Orthogonality analysis

We used 16x36 kernels out of 32x32 kernels and it is with 7 bit integer approximation, so the trained kernels don’t have perfect orthogonality. We evaluated orthogonality with two criteria C1 and C2 and concluded  the new transform kernel has a similar evaluation value with VVC6, i.e. average energy loss are 0.04 and 1.46 in VVC6 while those are 0.04 and 1.45 in Test1 and Test2.

The following criteria C1 and C2 are our definition of energy loss of the kernel matrix.

Let A be 16xN transform matrix. AT・A is NxN matrix and should be equal to identity matrix in perfect orthogonality (it’s not possible 16xN matrix rather than NxN matrix must have energy loss). We evaluate this orthogonality with C1. A・AT is 16x16 matrix and should be equal to identity matrix in perfect orthogonality. We evaluate this orthogonality with C2.

C1= AT・A - IN1IN1                  (1)

C1= A・AT - I161I161                  (2)

where IN = 214 * NxN identical matrix. N = 32 or 36 in CE6-2.3a or 48 in VVC6.

|  |  | Mode=0 |  | Mode=1 |  | Mode=2 |  | Mode=3 |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | lfnstIdx=1 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |  |
| C2 (16x16) | Test1&Test2&Test3&Test4 | 0.04 | 0.03 | 0.04 | 0.03 | 0.04 | 0.04 | 0.03 | 0.04 | 0.04 |
|  | VTM-6.0 | 0.04 | 0.04 | 0.04 | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 | 0.04 |
| C1 (NxN) | Test1&Test2&Test3&Test4 | 1.13 | 1.14 | 1.65 | 1.98 | 0.98 | 1.48 | 1.63 | 1.58 | 1.45 |
|  | VTM-6.0 | 1.21 | 1.22 | 1.59 | 1.91 | 1.10 | 1.49 | 1.60 | 1.57 | 1.46 |

- Reference

[1]	T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis”, JVET-P0065

[2]	X. Zhao, X. Li, S. Liu, “CE6-2.2a/c: LFNST with unified kernels”, JVET-P0051

[3]	Y. Zhao, H. Yang, “CE6-2.1: LFNST with 3 transform sets”, JVET-P0044

[4]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[5] “NTIRE 2017 Challenge on Single Image Super-Resolution: Dataset and Study”, July, 2017

https://data.vision.ee.ethz.ch/cvl/DIV2K/

[6] scikit-learn, Machine Learning in Python

https://scikit-learn.org/stable/

- Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).