| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0499 |
| --- | --- |

| Title: | Non-CE4: Removal of MVD scaling process in MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the MVD scaling process in bi-predicted MMVD mode. Specifically, when both reference pictures have larger or smaller POC than the current picture, the signalled MVD is used for both reference lists; otherwise, the signalled MVD is used for reference list 0 and its opposite value is used for reference list 1. Simulation results reportedly show that the BD-rate changes are 0.01% for RA, and 0.08% for LB with almost no impact on encoding/decoding time compared to VTM-6.0.

# Introduction

In VTM-6.0, a complicated derivation process for motion vector difference in MMVD is used.

If the selected base merge candidate in MMVD mode has a bi-directional MV, the following applies in order:

Firstly, if the POC distance between the reference picture of L0 and the current picture (denoted as PocDisL0) and the POC distance between the reference picture of L1 and the current picture (denoted as PocDisL1) are same, MVD of L0 and L1 are both set equal to the signalled MVD directly.

Otherwise, if the absolute value of PocDisL0 is greater than or equal to the absolute value of PocDisL1, the following applies:

- MVD of L0 is set equal to the signalled MVD directly.

- If both reference pictures are short-term reference pictures, MVD of L1 is derived by scaling the signalled MVD according to the POC distances between the two reference pictures and the current picture.

- Otherwise,

  - If the two reference pictures both have larger or smaller POC than the current picture, MVD of L1 is set equal to the signalled MVD.

  - Otherwise, if one reference picture has larger POC than the current picture and the other reference picture has smaller POC than the current picture, MVD of L1 is set equal to the opposite value of the signalled MVD.

Otherwise, if the absolute value of PocDisL0 is less than the absolute value of PocDisL1, the similar operations are performed.

- MVD of L1 is set equal to the signalled MVD directly.

- If both reference pictures are short-term reference pictures, MVD of L0 is derived by scaling the signalled MVD according to the POC distances between the two reference pictures and the current picture.

- Otherwise,

  - If the two reference pictures both have larger or smaller POC than the current picture, MVD of L0 is set equal to the signalled MVD.

  - Otherwise, if one reference picture has larger POC than the current picture and the other reference picture has smaller POC than the current picture, MVD of L0 is set equal to the opposite value of the signalled MVD.

To summarize, the MVD derivation process in MMVD is complicated mainly because of:

- The MVD scaling process is required when the absolute POC distances between the two reference pictures and the current picture are different.

- Whether the signalled MVD is used for reference list 0 or 1 depends on the absolute POC distances between the two reference pictures and the current picture.

- Proposed methods

This contribution proposes to remove the MVD scaling process in bi-predicted MMVD mode. If the selected base merge candidate in MMVD mode has a bi-directional MV, MVD of reference picture list 0 (L0) is always set equal to the signalled MVD directly and MVD of reference picture list 1 (L1) is set equal to the signalled MVD or the opposite value of the signalled MVD.

More specifically, the followings are applied:

- If the two reference pictures both have larger or smaller POC than the current picture, MVD of L1 is set equal to the signalled MVD.

- Otherwise (one reference picture has larger POC than the current picture and the other reference picture has smaller POC than the current picture), MVD of L1 is set equal to the opposite value of the signalled MVD.

# Experimental Results

The proposed method has been implemented on top of VTM-6.0. Results under RA/LDB configurations are summarized in Table 1. The simulations were performed following the the common test conditions [1]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.08% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.01% | 0.03% | 0.02% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.09% | 0.01% | 100% | 100% | 0.10% | 0.14% | -0.07% | 100% | 99% |
| Class C | 0.02% | 0.12% | 0.00% | 100% | 100% | 0.07% | 0.12% | 0.05% | 100% | 98% |
| Class E |  |  |  |  |  | 0.06% | 0.24% | 0.40% | 100% | 99% |
| Overall | 0.01% | 0.08% | 0.02% | 100% | 100% | 0.08% | 0.16% | 0.09% | 100% | 99% |
| Class D | 0.06% | 0.08% | -0.02% | 100% | 100% | 0.18% | 0.47% | 0.41% | 101% | 100% |
| Class F | 0.01% | 0.02% | -0.01% | 100% | 100% | -0.04% | -0.26% | 0.19% | 100% | 99% |
| Class SCC | -0.02% | -0.03% | -0.02% | 100% | 100% | -0.10% | -0.05% | -0.12% | 98% | 99% |

- Conclusions

This contribution removes the MVD scaling process in bi-predicted MMVD mode. There is minor overall coding performance change with almost no impact on encoding/decoding time compared to VTM-6.0. It is recommended to adopt this method into VVC.

- Suggested changes to working draft

The proposed changes on top of JVET-O2001-vE are specified as below, with deleted text highlighted in turquoise strikethrough.

8.5.2.7	Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] )	(8-382)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] )	(8-383)

- If currPocDiffL0 is equal to currPocDiffL1, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-384)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-385)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-386)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-387)

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-388)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-389)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-390)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-391)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-392)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-393)

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 0 ] + 	(8-394)
	128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) )  >>  8 )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 1 ] + 	(8-395)
	128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) )  >>  8 )

    - Otherwise, the following applies:

mMvdL1[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL0[ 0 ]  :  −mMvdL0[ 0 ]	(8-396)

mMvdL1[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL0[ 1 ]  :  −mMvdL0[ 1 ]	(8-397)

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-398)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-399)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-400)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-401)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-402)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-403)

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL1[ 0 ] +	(8-404)
	 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) )  >>  8 )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, , (distScaleFactor * mMvdL1[ 1 ] +	(8-405)
	 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) )  >>  8 ) )

    - Otherwise, the following applies:

mMvdL0[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL1[ 0 ]  :  −mMvdL1[ 0 ]	(8-406)

mMvdL0[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL1[ 1 ]  :  −mMvdL1[ 1 ]	(8-407)

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 0 ]  :  0	(8-408)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 1 ]  :  0	(8-409)

- References

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).