| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0508 |
| --- | --- |

| Title: | CE7-related: unification for joint chroma residue coding modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xiang Li, Xin Zhao, Shan Liu | Tel: Email: | {leolzhao, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VVC Draft 6, there are three different joint chroma residual coding (JCCR) modes, wherein JCCR mode 2 is applied to both intra and inter coded CUs, but mode 1 and mode 3 are only applied to intra coded CU. It is reported that this is not a consistent design. In this contribution, the restrictions for JCCR mode 1 and mode 3 are removed so that these three JCCR modes can be consistently applied to both intra and inter coded CUs. To compensate the performance loss, for JCCR mode 1 and 3, the context for signaling tu_joint_cbcr_residual_flag is shared and dependent on intra/inter mode flag of current block, and context number is kept unchanged. It is reported that the proposed method achieves 0.01%, 0.00%, and -0.03% luma BD-rate changes for AI, RA, and LDB configurations, respectively.

Introduction

VVC Draft 6 supports several modes where chroma residuals are coded jointly, as known as joint chroma residual coding (JCCR) modes. The usage (activation) of a joint chroma coding mode is indicated by a TU-level flag tu_joint_cbcr_residual_flag and the selected mode is implicitly indicated by the chroma CBFs. The flag tu_joint_cbcr_residual_flag is present if any chroma CBF for a TU is equal to 1. The reconstruction process of chroma residuals (resCb and resCr) from the transmitted transform blocks is depicted in Table 1-1. When this mode is used, one single joint chroma residual block (resJointC[x][y] in Table 1-1) is signalled, and residual block for Cb (resCb) and residual block for Cr (resCr) are derived according to tu_cbf_cb, tu_cbf_cr, and CSign, which is a sign value specified in the slice header.

Table 1-1 Reconstruction of chroma residuals. The value CSign is a sign value (+1 or –1), which is specified in the slice header, resJointC[ ][ ] is the transmitted residual.

| tu_cbf_cb | tu_cbf_cr | reconstruction of Cb and Cr residuals | mode |
| --- | --- | --- | --- |
| 1 | 0 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 | 1 |
| 1 | 1 | resCb[ x ][ y ] = resJointC[ x ][ y ] resCr[ x ][ y ] = CSign * resJointC[ x ][ y ] | 2 |
| 0 | 1 | resCb[ x ][ y ] = ( CSign * resJointC[ x ][ y ] ) >> 1 resCr[ x ][ y ] = resJointC[ x ][ y ] | 3 |

The three JCCR modes described above are all supported in intra coded CU. In inter-coded CU, only mode 2 is supported. Hence, for inter coded CU, the syntax element tu_joint_cbcr_residual_flag is only present if both chroma CBFs are 1.

Proposed method

In this contribution, the restrictions for JCCR mode 1 and mode 3 are removed so that these three joint Cb and Cr residual coding modes can be applied to both intra and inter coded CUs for a consistent design. To compensate the performance loss, for JCCR mode 1 and mode 3, the context design of flag tu_joint_cbcr_residual_flag is changed as shown in Table 2-1 while the number of contexts is kept unchanged.

Table 2-1: Contexts for tu_joint_cbcr_residual_flag in VTM-6 and the proposed method

| Context | VTM-6 | Proposed method |
| --- | --- | --- |
| 0 | JCCR mode 2 | JCCR mode 2 |
| 1 | JCCR mode 1 | JCCR mode 1 and mode 3 in intra-coded CU |
| 2 | JCCR mode 3 | JCCR mode 1 and mode 3 in inter-coded CU |

Experimental results

The proposed method in this contribution is implemented on top of VTM6.0 software. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-N1010 [1]. One test is conducted to evaluate the performance of the proposed method.

Table 3-1: Results of the proposed method over VTM-6.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.09% | -0.03% | 100% | 100% |
| Class B | 0.00% | -0.11% | -0.02% | 100% | 99% |
| Class C | 0.02% | -0.20% | 0.17% | 101% | 99% |
| Class E | 0.00% | -0.03% | 0.20% | 100% | 101% |
| Overall | 0.01% | -0.07% | 0.07% | 100% | 100% |
| Class D | -0.01% | 0.04% | 0.16% | 100% | 103% |
| Class F | 0.00% | 0.01% | 0.08% | 99% | 100% |

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.18% | 0.24% | 101% | 100% |
| Class A2 | 0.02% | -0.39% | -0.04% | 101% | 100% |
| Class B | 0.00% | -0.07% | 0.11% | 101% | 99% |
| Class C | 0.02% | -0.31% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.22% | 0.05% | 101% | 100% |
| Class D | -0.03% | -0.78% | -0.38% | 100% | 103% |
| Class F | -0.01% | -0.16% | -0.10% | 100% | 102% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.32% | -0.88% | 101% | 99% |
| Class C | -0.02% | -0.99% | -0.68% | 101% | 98% |
| Class E | -0.17% | -0.16% | 0.13% | 101% | 99% |
| Overall | -0.03% | -0.50% | -0.56% | 101% | 99% |
| Class D | -0.03% | -1.16% | 0.43% | 101% | 104% |
| Class F | 0.05% | -0.26% | -0.39% | 101% | 100% |

Spec text

The following proposed text modifications of the proposed method are based on JVET-O2001-vE. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

#### 9.3.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

#### Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| bcw_idx[ ][ ] NoBackwardPredFlag = = 1 | 0 | bypass | bypass | bypass | na | na |
| cu_cbf | 0 | na | na | na | na | na |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | na | na | na | na | na |
| cu_sbt_quad_flag | 0 | na | na | na | na | na |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | na | na | na | na | na |
| cu_sbt_pos_flag | 0 | na | na | na | na | na |
| lfnst_idx[ ][ ] | ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| tu_cbf_luma[ ][ ][ ] | 0,1,2,3 (clause 9.3.4.2.5) | na | na | na | na | na |
| tu_cbf_cb[ ][ ][ ] | 0 | na | na | na | na | na |
| tu_cbf_cr[ ][ ][ ] | tu_cbf_cb[ ][ ][ ] | na | na | na | na | na |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| cu_chroma_qp_offset_flag | 0 | na | na | na | na | na |
| cu_chroma_qp_offset_idx | 0 | 0 | 0 | 0 | 0 | na |
| transform_skip_flag[ ][ ] | 0 | na | na | na | na | na |
| tu_mts_idx[ ][ ] | 0 | 1 | 2 | 3 | na | na |
| tu_joint_cbcr_residual_flag[ ][ ] | (2*tu_cbf_cb[ ][ ] + tu_cbf_cr[ ][ ] == 3)? 0: (pred_mode_flag == MODE_INTRA ? 1: 2) − 1 | na | na | na | na | na |

Conclusion

This contribution proposed to remove the restrictions for JCCR mode 1 and mode 3 so that these three joint Cb and Cr residual coding modes can be applied to both intra and inter coded CUs for better design consistency. In addition, for JCCR mode 1 and mode 3, the context of flag tu_joint_cbcr_residual_flag for entropy coding is shared and dependent on intra/inter mode flag of current block, and context number is kept unchanged. It is reported that the proposed method achieves 0.01%, 0.00%, and -0.03% BD-rate changes for AI, RA, and LDB configurations, respectively. It is proposed to adopt the proposed method.

References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).