| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0509 |
| --- | --- |

| Title: | CE3-related: Simplification for MIP matrix multiplication |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In current VVC Draft 6, the matrix multiplicaition in MIP prediction process is Aij⋅inputred[j], wherein Aij is 7-bit unsigned matrix coefficient and inputredj is 11-bit signed residues of reduced boundary samples (when internal bit depth is set to 10). Since the bit depth of inputredj is greater than 8-bit, 16-bit SIMD are required for software implementation of multiplications used in MIP prediction process, which is less efficient than 8-bit multiplications. In this contribution, inputred[j] is right shifted to 8-bit value before the matrix multiplication process so that MIP matrix multiplication process can be implemented with 8-bit SIMD multiplications. It is reported that the proposed method has 0.02% and 0.01% BD-rate impact for AI and RA configurations, respectively.

Introduction

The generation of the prediction signal for MIP process is based on the following three steps.

In Step 1, after averaging boundary samples, one additional step is employed. If idxW,H=0 or idxW,H=1, then

inputred0=bdryred0-(1≪bitDepth-1),                            (1-1)

inputredj= bdryredj-bdryred0,     j=1,…,sizebdryred-1,            (1-2)

Otherwise, if idxW,H=2, 
                    inputredj= bdryredj+1-bdryred0,     j=0,…,sizebdryred-2,         (1-3)

Here, bitDepth denotes the luma bit-depth. When internal bit-depth is set equal to 10, the bit depth of bdryredi is 10, and the bit depth of inputredj is 11 since it’s generated by subtracting one 10-bit value from another 10-bit value.

In Step 2, if  Wred and Hred denote the width and the height of the reduced prediction signal and if sW is the shift corresponding to the prediction mode, the reduced prediction signal predred  is computed as follows

predredi=j=0inSize(Aij-fW)⋅inputred[j]+1≪sW-1≫sW+bdryred0 ,                                                                      (1-4)

In Step 3, the prediction signal arises from the reduced prediction signal predred on Wred×Hred by linear interpolation. Depending on the block shape, linear interpolation is done in vertical, horizontal or both directions. If linear interpolation is to be applied in both directions, it is first applied in horizontal direction when W<H and it is first applied in vertical direction.

Proposes method

It is proposed that, the residues of reduced boundary samples inputredj in equation (1-4) is right shifted to signed 8-bit integers before the matrix multiplication process.

For software SIMD using Streaming SIMD Extensions (SSE) based on x86 architecture, two instructions PMADDUBSW/VPMADDUBSW are available which can apply parallel multiplications of 8-bit signed and 8-bit unsigned integers and adding adjacent pairs of intermediate results in one instruction. With the proposed method, inputred becomes signed 8-bit integers, so these SIMD instructions can be applied to double the throughput of multiplications per instruction.

Experimental results

The proposed method in this contribution is implemented on top of VTM6.0 software. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition (CTC) described in JVET-N1010 [1]. One test is conducted to evaluate the performance of the proposed methods which is listed below:

Table 1: Simulation results of the proposed method under CTC.

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.08% | 0.10% | 100% | 100% | 0.02% | -0.09% | 0.05% | 100% | 100% |
| Class A2 | 0.03% | 0.02% | -0.01% | 99% | 100% | 0.01% | 0.13% | -0.01% | 100% | 100% |
| Class B | 0.01% | 0.03% | -0.06% | 101% | 100% | 0.00% | 0.01% | -0.05% | 101% | 104% |
| Class C | 0.01% | -0.11% | 0.03% | 100% | 99% | 0.01% | 0.17% | 0.02% | 100% | 97% |
| Class E | 0.05% | 0.05% | 0.14% | 100% | 98% |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.03% | 100% | 100% | 0.01% | 0.06% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.02% | -0.12% | 100% | 95% | -0.01% | -0.35% | 0.05% | 100% | 96% |
| Class F | -0.01% | 0.07% | -0.02% | 100% | 103% | 0.05% | -0.09% | 0.04% | 100% | 100% |

Spec text

The following proposed text modifications of the proposed method are based on JVET-O2001-vE. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

#### 8.4.5.2.1	Matrix-based intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

Variables numModes, boundarySize, predW, predH and predC are derived using MipSizeId[ xTbCmp ][ yTbCmp ] as specified in Table 8-4.

Table 8-4 – Specification of number of prediction modes numModes, boundary size boundarySize, and prediction sizes predW, predH and predC using MipSizeId

| MipSizeId | numModes | boundarySize | predW | predH | predC |
| --- | --- | --- | --- | --- | --- |
| 0 | 35 | 2 | 4 | 4 | 4 |
| 1 | 19 | 4 | 4 | 4 | 4 |
| 2 | 11 | 4 | Min( nTbW, 8 ) | Min( nTbH, 8 ) | 8 |

The flag isTransposed is derived as follows:

isTransposed = ( predModeIntra > ( numModes / 2 ) )  ?  TRUE  :  FALSE	(8-55)

The variable inSize is derived as follows:

inSize = ( 2 * boundarySize ) − ( MipSizeId[ xTbCmp ][ yTbCmp ]  = =  2 )  ?  1  :  0	(8-56)

The variables mipW and mipH are derived as follows:

mipW = isTransposed  ?  predH  :  predW		(8-57)

mipH = isTransposed  ?  predW  :  predH		(8-58)

For the generation of the reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1, the following applies:

- The reference sample availability marking process is invoked with the sample location ( xTbCmp, yTbCmp ), reference line index equal to 0, the reference sample width nTbW, the reference sample height nTbH, colour component index equal to 0 as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1.. nTbH − 1 and x = 0.. nTbW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process is invoked with reference line index 0, the reference sample width nTbW, the reference sample height nTbH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1, and colour component index 0 as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

- The reference samples refT[ x ] with x = 0..nTbW − 1 and refL[ y ] with y = 0..nTbH − 1 are assigned as follows:

refT[ x ] = refUnfilt[ x ][ −1 ]		(8-59)

refL[ y ] = refUnfilt[ −1 ][ y ]		(8-60)

For the generation of the input samples p[ x ] with x = 0..2 * inSize − 1, the following applies:

- The MIP boundary downsampling process is invoked for the top reference samples with the block size nTbW, the reference samples refT[ x ] with x = 0..nTbW − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redT[ x ] with x = 0..boundarySize − 1 as outputs.

- The MIP boundary downsampling process is invoked for the left reference samples with the block size nTbH, the reference samples refL[ y ] with y = 0..nTbH − 1, and the boundary size boundarySize as inputs, and reduced boundary samples redL[ x ] with x = 0..boundarySize − 1 as outputs.

- The reduced top and left boundary samples redT and redL are assigned to the boundary sample array pTemp[ x ] with x = 0..2 * boundarySize − 1 as follows:

- If isTransposed is equal to 1, pTemp[ x ] is set equal to redL[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redT[ x ] with x = 0..boundarySize − 1.

- Otherwise, pTemp[ x ] is set equal to redT[ x ] with x = 0..boundarySize − 1 and pTemp[ x + boundarySize ] is set equal to redL[ x ] with x = 0..boundarySize − 1.

- The variable offset is set equal to 1 << (BitDepthY – 8)

- The input values p[ x ] with x = 0.. inSize − 1 are derived as follows:

- If MipSizeId[ xTbCmp ][ yTbCmp ] is equal to 2, the following applies:

p[ x ] = ( pTemp[ x + 1 ] − pTemp[ 0 ] + offset) >> (BitDepthY + 1 – 8)	(8-61)

- Otherwise (MipSizeId[ xTbCmp ][ yTbCmp ] is less than 2), the following applies:

p[ 0 ] = ( pTemp[ 0 ] − ( 1 << ( BitDepthY − 1 ) ) + offset) >> (BitDepthY + 1 – 8)
p[ x ] = ( pTemp[ x ] − pTemp[ 0 ] + offset) >> (BitDepthY + 1 – 8), for x = 1.. inSize − 1	(8-62)

For the intra sample prediction process according to predModeIntra, the following ordered steps apply:

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

- The variable modeId is derived as follows:

modeId = predModeIntra − ( isTransposed  ?  numModes / 2  :  0 )	(8-63)

- The weight matrix mWeight[ x ][ y ] with x = 0..2 * inSize − 1, y = 0..predC * predC − 1 is derived by invoking the MIP weight matrix derivation process as specified in clause 8.4.5.2.3 with MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as inputs.

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-5.

- The variable sW is further modified is as sW = sW – (BitDepthY + 1 – 8)

- The variable sO is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-6.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i )	(8-64)

incW = ( predC > mipW )  ?  2  :  1		(8-65)

incH = ( predC > mipH )  ?  2  :  1		(8-66)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> sW ) + pTemp[ 0 ]	(8-67)

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are clipped as follows:

predMip[ x ][ y ] = Clip1Y( predMip[ x ][ y ] )	(8-68)

- When isTransposed is equal to TRUE, the predH x predW array predMip[ x ][ y ] with x = 0..predH − 1, y = 0..predW − 1 is transposed as follows:

predTemp[ y ][ x ] = predMip[ x ][ y ]	 	(8-69)

predMip =predTemp			(8-70)

- The predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If nTbW is greater than predW or nTbH is greater than predH, the MIP prediction upsampling process is invoked with the input block width predW, the input block height predH, matrix-based intra prediction samples predMip[ x ][ y ] with x = 0..predW − 1, y = 0..predH − 1, the transform block width nTbW, the transform block height nTbH, the top reference samples refT[ x ] with x = 0..nTbW − 1, and the left reference samples refL[ y ] with y = 0..nTbH − 1 as inputs, and the output is the predicted sample array predSamples.

- Otherwise, predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 is set equal to predMip[ x ][ y ].

Conclusion

This contribution proposed that inputred[j] is right shifted to 8-bit value before the matrix multiplication process so that MIP matrix multiplication process can be implemented with 8-bit SIMD multiplications. It is reported that the proposed method has 0.02% and 0.01% BD-rate impact for AI and RA configurations, respectively. It is suggested to adopt the proposed method.

References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).