| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0510 |
| --- | --- |

| Title: | Non-CE3/AHG8: On picture size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, Byeongdoo Choi, Stephan Wenger, Shan Liu | Tel: Email: | {leolzhao, xinzzhao, xlxiangli, bdchoi, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VVC Draft 6, there are two sets of parameters related to picture width and height. The first set is (pic_width_max_in_luma_samples, pic_height_max_in_luma_samples), and the second set is (pic_width_in_luma_samples, pic_height_in_luma_samples). The parameters in the second set are restricted to be multiples of Max (8, MinCbSizeY) whereas the the parameters in the first set are restricted to be multiples of MinCbSizeY, which is not a consistent design. In this contribution, firstly, it is proposed to use the same restriction for the two sets. Secondly, it is proposed to consider chroma format in the restriction of Max (8, MinCbSizeY) for picture width and height.

Introduction

In VVC draft 6, picture width and height in luma samples are restricted to be a multiple of the maximum of 8 and MinCbSizeY to avoid 2x2, 2x4, and 4x4 chroma blocks at the corner of the picture [1].

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples.

In addition, there is another parameter set (pic_width_max_in_luma_samples, pic_height_max_in_luma_samples) to specify the maximum width and height, in units of luma samples, of each decoded picture referring to the SPS.

pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each decoded picture referring to the SPS. pic_width_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each decoded picture referring to the SPS. pic_height_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

Proposed method

Proposed alignment of related HLS

Firstly, it is proposed to use the same restriction for both picture width and height and maximum picture width and height. The specification text is modified accordingly as follows:

pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each decoded picture referring to the SPS. pic_width_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY).

pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each decoded picture referring to the SPS. pic_height_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY).

Proposed modifications considering chroma format

Secondly, the restriction of Max (8, MinCbSizeY) for picture width and height is to remove 2x2, 2x4, 4x2 chroma blocks. However, current picture width and height restriction only considers YUV 4:2:0 format and does not consider other YUV formats. For example, for YUV 4:4:4 format, when picture width or height is multiple of 4, there is no 2x2, 2x4/4x2 chroma blocks at all. It is proposed to restrict the picture width and height based on YUV format, which is listed in Table 2-1.

Table 2-1: Restrictions of picture width and height for different YUV format

| YUV format | Chroma_format_idc | Restrictions on width | Restrictions on height |
| --- | --- | --- | --- |
| YUV400 | 0 | multiple of MinCbSizeY | multiple of MinCbSizeY |
| YUV420 | 1 | Same as VTM-6 | Same as VTM-6 |
| YUV422 | 2 | Same as VTM-6 | multiple of MinCbSizeY |
| YUV444 | 3 | multiple of MinCbSizeY | multiple of MinCbSizeY |

The specification text is modified accordingly as follows:

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. When chroma_format_idc is equal to 1 or 2, pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. Otherwise, pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of MinCbSizeY, and shall be less than or equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. When chroma_format_idc is equal to 1, pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. Otherwise, pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY, and shall be less than or equal to pic_height_max_in_luma_samples.

Conclusion

In this contribution, firstly, an alignment between the SPS parameters (pic_width_max_in_luma_samples, pic_height_max_in_luma_samples) and (pic_width_in_luma_samples, pic_height_in_luma_samples) is proposed. Secondly, it is proposed to restrict the picture width and height based on YUV format. It is suggested to adopt the proposed method.

References

- L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, and M. Karczewicz, “CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-O0640, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).