| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0512 |
| --- | --- |

| Title: | Non-CE4: SIMD support for motion compensated prediction at high internal bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution provides software patch for VTM-6.0 to support single instruction multiple data (SIMD) implementation for motion compensated prediction when the internal bit-depth is larger than 10-bit. Simulation results reportedly show that for 12-bit coding encoding/decoding time reductions of RA and LD configurations over VTM-6.0 are 19%/14% and 19%/15%, respectively, with bit-exact BD-rate results.

# Introduction

Motion compensated prediction (MCP) is one of the most computationally intensive modules at both encoding and decoding stages. In VTM-6.0 [1], single instruction multiple data (SIMD) implementation is applied to speed up the MCP process when the internal bit-depth, i.e., InternalBitDepth, is equal to or smaller than 10-bit.

To reduce the encoding and decoding complexity, the following software changes are proposed in this document to enable the SIMD support for the MCP when InternalBitDepth > 10:

- In VTM-6.0, the existing AVX2/SIMD42 based MCP functions, namely, simdInterpolateHorM8_AVX2() /simdInterpolateHorM8(), simdInterpolateVerM8_AVX2()/simdInterpolateVerM8(), simdInterpolateHorM4(), simdInterpolateVerM4(), simdInterpolateN2_M8() and simdInterpolateN2_M4() use register length of 32-bit. They can support the MCP for internal bit-depths up to 16-bit. Currently, those functions are conditionally applied when InternalBitDepth ≤ 10. In this document, it is proposed to remove such condition, i.e., enabling those SIMD implementations for high internal bit-depth (> 10-bit).

- In VTM-6.0, the SIMD function of generating the bilinear interpolation samples for the DMVR, i.e., simdInterpolateN2_10BIT_M4(), uses the register length of 16-bit. This function may cause 16-bit overflow problem when InternalBitDepth > 10. To fix such issue, AVX2 and SIMD42 implementation of the function are provided for the DMVR when InternalBitDepth > 10.

# Simulation results

The proposed software changes are tested on top of VTM-6.0 [1]. In the simulation, both anchor and test are generated by following the common test condition (CTC) [2] except that InternalBitDepth is set to 12. It is shown that the proposed changes produce bit-exact BD-rate results and reduce the encoding and decoding time by 19% and 14% for RA, and 19% and 15% for LD, respectively.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 82% | 88% |
| Class A2 | 0.00% | 0.00% | 0.00% | 80% | 83% |
| Class B | 0.00% | 0.00% | 0.00% | 80% | 85% |
| Class C | 0.00% | 0.00% | 0.00% | 81% | 87% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 81% | 86% |
| Class D | 0.00% | 0.00% | 0.00% | 82% | 87% |
| Class F | 0.00% | 0.00% | 0.00% | 88% | 94% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 82% | 85% |
| Class C | 0.00% | 0.00% | 0.00% | 84% | 91% |
| Class E | 0.00% | 0.00% | 0.00% | 76% | 78% |
| Overall | 0.00% | 0.00% | 0.00% | 81% | 85% |
| Class D | 0.00% | 0.00% | 0.00% | 83% | 88% |
| Class F | 0.00% | 0.00% | 0.00% | 89% | 97% |

# Reference

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).