| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0513 |
| --- | --- |

| Title: | Non-CE4: Extending switchable interpolation filter to affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In the VVC working draft 6, different numbers of motion vector difference (MVD) resolutions are available for regular inter mode and affine mode. Specifically, a MVD resolution for regular inter mode can be chosen from {1/4-pel, 1/2-pel, 1-pel, 4-pel}, while a MVD resolution for affine mode can be selected from {1/4-pel, 1/16-pel, 1-pel}. Additionally, when the 1/2-pel MVD resolution is selected for regular inter mode, one alternative 6-tap interpolation filter is applied to generate luma prediction samples.

To achieve one more unified design, this contribution proposes to extend the alternative 6-tap interpolation filter to affine mode, with 1/2-pel resolution introduced for affine MVDs. For affine non-merge mode, the alternative filter is used only when the 1/2-pel MVD resolution is explicitly selected. For affine merge mode, the decision of using the alternative interpolation filter is inherited from the corresponding spatial neighbor. Simulation results reportedly show that compared to VTM-6.0 anchors the proposed method provides average {Y, U, V} BD-rate savings of {0.07%, 0.01%, 0.09%} for RA with encoding and decoding time of 102% a 100%. The corresponding average BD-rate savings for LD are {0.03%, -0.16%, 0.02%} with encoding and decoding time of 100% and 99%.

# Introduction

In the VVC working draft 6 [1], adaptive motion vector resolution (AMVR) is applied to reduce the signaling overhead of inter coding. Additionally, after the adoption of switchable interpolation filter [2] at Gothenburg meeting, different numbers of motion vector difference (MVD) resolutions are applied to regular inter mode (i.e., non-affine mode) and affine mode. Specifically, the MVDs of regular inter mode can be adaptively signaled in the precisions of {1/4-pel, 1/2-pel, 1-pel and 4-pel} while the allowed MVD precisions of affine mode include {1/4-pel, 1-pel and 4-pel}. Additionally, the switch to using alternative 6-tap interpolation filter [3, 9, 20, 20, 9, 3] is only allowed when the 1/2-pel MVD precision is selected for one regular inter coding unit (CU). For affine mode, the alternative interpolation filter is always not applied.

# Proposal

For one more unified design, this contribution proposes to enable the alternative 6-tap interpolation filter for affine mode. Similar to regular inter mode, the usage of the alternative interpolation filter is indicated by adding 1/2-pel MVD precision to affine mode. This is done by using the same MVD precision syntax (i.e., amvr_flag and amvr_precision_idx) and binarization methods of regular inter mode to indicate the MVD precisions of affine mode, as illustrated as

| amvr_flag | amvr_precision_idx | MVD precision |  |
| --- | --- | --- | --- |
|  |  | regular inter | affine |
| 0 | - | 1/4-pel | 1/4-pel |
| 1 | 0 | 1/2-pel | 1/2-pel |
| 1 | 1 | 1-pel | 1/16-pel |
| 1 | 2 | 4-pel | 1-pel |

Additionally, when the 1/2-pel MVD precision is selected, the alternative 6-tap interpolation filter is enabled for affine subblocks only when their MVs points to reference samples at half-pel positions; for affine subblocks whose reference samples are not located at half-pel positions, the default 6-tap affine interpolation filters are still applied.

Similar to inter merge mode, the information on whether to apply the alternative interpolation filter or not is inherited for affine merge mode. Specifically, for inherited affine merge candidates, the information is directly obtained by reusing the filter selection decision of the corresponding spatial neighboring block; for constructed affine merge candidates, the alternative interpolation filter can only be applied when the alternative interpolation filter is enabled for all the selected control-point neighboring blocks.

# Experimental results

The proposed method was implemented on top of VTM-6.0 [3]. Simulations were carried out by following the common test conditions (CTCs) as specified in [4].

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.06% | -0.07% | 101% | 101% |
| Class A2 | -0.09% | 0.01% | -0.06% | 103% | 98% |
| Class B | -0.07% | 0.05% | -0.03% | 102% | 102% |
| Class C | -0.07% | -0.06% | -0.21% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.01% | -0.09% | 102% | 100% |
| Class D | -0.06% | 0.09% | 0.03% | 103% | 99% |
| Class F | -0.04% | -0.06% | -0.05% | 102% | 96% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.03% | 0.04% | 96% | 100% |
| Class C | -0.02% | 0.20% | -0.13% | 104% | 99% |
| Class E | -0.05% | 0.32% | 0.05% | 101% | 98% |
| Overall | -0.03% | 0.16% | -0.02% | 100% | 99% |
| Class D | 0.04% | 0.10% | 0.19% | 100% | 101% |
| Class F | -0.02% | 0.34% | 0.40% | 98% | 100% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

A. Henkel, I. Zupancic, B. Bross, M. Winken, H. Schwarz, D. Marpe, T. Wiegand, “CE4: Switchable interpolation filter (CE4-1.1, CE4-1.2, CE4-1.3, CE4-1.4, CE4-1.5, CE4-1.6, CE4-1.7)”, JVET-O0057, July 2019, Gothenburg, Sweden.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).