| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P515 |
| --- | --- |

| Title: | Non-CE8: Quantization unification for palette escape and transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Xiaoyu Xiu Yi-Wen Chen Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com xiaoyuxiu@kwai.com yiwenchen@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In this contribution, two methods are proposed to align the quantization schemes used for transform skip and escape samples. In the first method, the transform shift design of the transform skip mode is used for escape quantization under palette mode. In the second method, the transform shift design of the transform skip mode is removed to align the quantization for escape samples under palette mode. The proposed methods report negligible BD-rate changes compared to VTM6.0.

Introduction

In current VVC [1], two different designs of quantization are applied to transform skip and palette mode. For transform skip mode, prediction residuals are quantized and coded directly without transform operations performed. More specifically, its quantization and de-quantization processes can be described as follows:

Quantization:

pLevel =QuantTSpResi= pResi∙2transformShiftencScaleQP%6214∙2QP/62transformShift		(1)

De-quantization:

pResi'=QuantTS-1pLevel= pLevel2transformShift∙decScaleQP%6∙2QP/62transformShift26			(2)

where  pResi and pResi’ are the original and reconstructed prediction residual sample values; pLevel is the quantized value, or quantization level; The variable transformShift represents the bit-shift that is used to compensate the dynamic range increase due to 2D transforms, which is equal to 15 – bitDepth- (log2(W)+log2(H))/2, where W and H are the width and height of the current transform unit, bitDepth is the coding bit-depth. The encScale[] and decScale[] are the quantization and dequantization look-up table which are in 14-bit and 6-bit precisions and defined as

| QP%6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| encScale[QP%6] | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| decScale[QP%6] | 40 | 45 | 51 | 57 | 64 | 72 |

For quantization/de-quantization of the escape value, the following equations describe the corresponding process that are applied at encoder and decoder, respectively.

Quantization:

pLevel =QuantEpEsca=pEsca∙encScaleQP%6214∙2QP/6					(3)

De-quantization:

pEsca'=QuantE-1pLevel=pLevel∙decScaleQP%62QP/626					(4)

where  pEsca and pEsca’ are the original and reconstructed escape values; pLevel is the quantized value, or quantization level; encScale[] and decScale[] are the quantization and dequantization look-up table which are in 14-bit and 6-bit precisions and defined as

| QP%6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| encScale[QP%6] | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| decScale[QP%6] | 40 | 45 | 51 | 57 | 64 | 72 |

To achieve one more uniform design, two methods are proposed in the following to harmonize the quantization methods for the transform skip and the palette.

Method One: Unify the quantization/de-quantization operations used under transform skip mode and palette mode, by applying the quantization/de-quantization operations used under transform skip mode to palette mode.

Method Two: Unify the quantization/de-quantization operations used under transform skip mode and palette mode, by applying the current quantization/de-quantization operations under palette mode to transform skip mode.

Experimental results

Method One on Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 1 and Table 2.

Table 1: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | -0.01% | 99% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Animation | 0.00% | -0.01% | 0.01% | 97% | 99% |
| Mixed content | 0.00% | -0.01% | 0.00% | 98% | 99% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.00% | 0.00% | 102% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Animation | 0.00% | -0.01% | -0.01% | 95% | 97% |
| Mixed content | 0.00% | 0.00% | -0.01% | 96% | 97% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | -0.03% | -0.01% | 102% | 101% |
| TGM 720p | -0.10% | -0.05% | -0.08% | 102% | 105% |
| Animation | 0.01% | -0.09% | -0.09% | 96% | 97% |
| Mixed content | 0.06% | -0.03% | 0.16% | 96% | 95% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 97% | 96% |
| Overall | 0.00% | -0.04% | -0.01% | 99% | 99% |

Table 2: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 105% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.00% | 0.00% | 97% | 98% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Animation | 0.01% | 0.01% | 0.01% | 97% | 99% |
| Mixed content | 0.01% | 0.00% | 0.00% | 99% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | -0.07% | -0.05% | 101% | 101% |
| TGM 720p | 0.00% | 0.01% | 0.12% | 99% | 100% |
| Animation | 0.01% | 0.00% | -0.04% | 92% | 91% |
| Mixed content | -0.04% | -0.13% | -0.20% | 96% | 96% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | -0.01% | -0.04% | -0.03% | 98% | 98% |

Method Two on Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, BDPCM, PLT are turned on for all Classes. The results with DualITree OFF and DualITree ON are shown in Table 3 and Table 4.

Table 3: Results of proposed modification, DualITree = 0 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.05% | 0.04% | 0.05% | 99% | 97% |
| TGM 720p | 0.17% | -0.15% | 0.06% | 103% | 98% |
| Animation | -0.01% | 0.03% | 0.07% | 97% | 88% |
| Mixed content | -0.09% | -0.07% | -0.11% | 100% | 92% |
| Camera-Captured content | -0.01% | -0.08% | 0.01% | 101% | 94% |
| Overall | 0.01% | -0.05% | 0.02% | 100% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.00% | 0.07% | 103% | 98% |
| TGM 720p | -0.03% | -0.02% | 0.13% | 103% | 99% |
| Animation | -0.02% | -0.09% | -0.09% | 100% | 96% |
| Mixed content | -0.12% | -0.06% | -0.13% | 101% | 98% |
| Camera-Captured content | -0.06% | 0.06% | 0.06% | 103% | 98% |
| Overall | -0.04% | -0.03% | 0.02% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT0, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.14% | 0.25% | 0.11% | 97% | 93% |
| TGM 720p | 0.06% | -0.27% | 0.64% | 95% | 95% |
| Animation | -0.08% | -0.05% | 0.06% | 96% | 98% |
| Mixed content | 0.21% | 0.72% | 0.22% | 94% | 92% |
| Camera-Captured content | -0.02% | -0.06% | 0.06% | 98% | 97% |
| Overall | 0.07% | 0.11% | 0.25% | 96% | 95% |

Table 4: Results of proposed modification, DualITree = 1 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | 0.09% | -0.04% | 104% | 99% |
| TGM 720p | 0.02% | -0.07% | -0.08% | 100% | 98% |
| Animation | -0.05% | 0.00% | 0.00% | 103% | 96% |
| Mixed content | 0.00% | 0.03% | 0.03% | 104% | 95% |
| Camera-Captured content | -0.02% | 0.06% | -0.01% | 102% | 97% |
| Overall | -0.02% | 0.02% | -0.02% | 103% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.02% | -0.03% | 102% | 99% |
| TGM 720p | 0.00% | -0.05% | -0.07% | 102% | 98% |
| Animation | 0.08% | 0.06% | 0.07% | 101% | 97% |
| Mixed content | 0.01% | -0.16% | -0.11% | 102% | 100% |
| Camera-Captured content | -0.06% | 0.01% | 0.03% | 104% | 101% |
| Overall | -0.01% | -0.04% | -0.03% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0_PLT1_IBC1_DT1, HEVC PLT |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | -0.23% | -0.10% | 98% | 97% |
| TGM 720p | 0.25% | -0.27% | 0.45% | 102% | 96% |
| Animation | 0.03% | 0.04% | 0.16% | 100% | 97% |
| Mixed content | -0.15% | 0.46% | 0.26% | 93% | 92% |
| Camera-Captured content | -0.06% | 0.02% | -0.19% | 99% | 95% |
| Overall | 0.01% | -0.03% | 0.14% | 99% | 96% |

Method Two on Test sequences 4:2:0

The proposed method was implemented based on the VTM6 reference software and tested following the common test conditions as specified in [3].

Table 5: Results of proposed modification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.04% | 0.02% | 102% | 96% |
| Class A2 | 0.01% | 0.01% | 0.01% | 102% | 97% |
| Class B | 0.00% | 0.06% | -0.03% | 103% | 92% |
| Class C | 0.00% | -0.07% | -0.01% | 100% | 99% |
| Class E | 0.00% | 0.02% | 0.06% | 100% | 96% |
| Overall | -0.01% | 0.01% | 0.00% | 102% | 96% |
| Class D | -0.02% | -0.01% | -0.12% | 100% | 95% |
| Class F | -0.09% | -0.09% | 0.03% | 102% | 96% |
| Class TGM | -0.09% | -0.07% | -0.08% | 102% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | -0.04% | 102% | 98% |
| Class A2 | 0.04% | 0.11% | -0.01% | 99% | 96% |
| Class B | -0.02% | 0.12% | 0.02% | 99% | 96% |
| Class C | -0.03% | -0.02% | 0.10% | 101% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.07% | 0.02% | 100% | 95% |
| Class D | -0.08% | -0.14% | 0.02% | 98% | 88% |
| Class F | -0.03% | -0.03% | 0.02% | 98% | 94% |
| Class TGM | -0.04% | -0.09% | -0.09% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6, CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | -0.06% | -0.32% | 100% | 93% |
| Class E | -0.03% | -0.17% | 0.89% | 96% | 91% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.44% | 0.01% | 98% | 91% |
| Class F | -0.08% | -0.28% | 0.35% | 101% | 94% |
| Class TGM | -0.04% | -0.08% | -0.03% | 96% | 97% |

Working Draft Text

Method One

The changes to align the scanning order of residual blocks in the VVC working draft [1] is highlight below.

8.7.2 Scaling and transformation process

…

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = transform_skip_flag ? 0 : Max( 20 − bitDepth, 0 )	(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )	(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift				(8-909)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift

…

8.7.3 Scaling process for transform coefficients

…

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = ( ( rectNonTsFlag  ?  1  :  0 ) + 		(8-956)
	( transform_skip_flag ? 0 : ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) + bitDepth - 15 ) − 20 + dep_quant_enabled_flag

bdOffset = ( 1 << bdShift ) >> 1			(8-909)

…

Method Two

8.4.5.3	Decoding process for palette mode

The changes to align the scanning order of residual blocks in the VVC working draft [1] is highlight below.

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0, Qp′Cr )		(8-274)

- The variables bitDepth and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 )  ?  BitDepthY  :  BitDepthC	(8-275)

tsShift = 15 − bitDepth − ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )

- The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

- The following applies:

tmpVal = ( PaletteEscapeVal[ cIdx ][ xCb + xL ][ yCb + yL ] *
			levelScale[ qP%6 ] )  <<  ( qP / 6 ) + 32 + tsShift )  >>  (6 + tsShift )	(8-276)

recSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, tmpVal )

…

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, the 15th JVET meeting, Gothenburg, SE, 3–12 July 2019

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, 15th meeting, Gothenburg, SE, July 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-O1010, the 14th JVET meeting, March 2019, Geneva, CH

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).