| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0518 |
| --- | --- |

| Title: | AHG16/Non-CE4: Addressing 16-bit multiplication overflow issue of the PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

At the 15th JVET meeting, prediction refinement with optical flow (PROF) was adopted into VVC draft 6. Later, experts commented on the JVET reflector that PROF refined samples may exceed the dynamic range of 16-bit signed integer. As a result, multiplier beyond 16-bit become necessary when use PROF together with weighted prediction (WP) and/or bi-prediction with CU-level weight (BCW). To fix such 16-bit multiplication overflow issue, two methods are proposed in this contribution: 1) clipping the PROF sample refinements within the range of 14-bit signed integer for input 8- to 12-bit video, i.e., [-8192, 8191], to ensure that the refined sample value is within 16-bit dynamic range; 2) directly clipping the refined prediction samples after PROF to be within 16-bit dynamic range for input 8- to 12-bit video. Simulation results reportedly show that compared with VTM-6.0 anchor, the average BD-rate impacts of Method 1) are {0.00%, 0.00%, 0.00%} for RA and {0.00%, 0.00%, 0.00%} for LD; the corresponding BD-rate impacts of Method 2) are {0.00%, 0.00%, 0.00%} for RA and {0.00%, 0.00%, 0.00%} for LD. The encoding and decoding time changes of two methods are negligible.

# Introduction

At the 15th JVET meeting, prediction refinement with optical flow (PROF) [1] was adopted into VVC draft 6 [2]. According to the current design, the PROF can be combined with weighted prediction (WP) and bi-prediction with CU-level weight (BCW). Specifically, assuming one affine coding unit (CU) is bi-predicted, the prediction samples of the CU are generated by

- Given one sample position (x, y), calculate L0 and L1 refined prediction samples as

| ΔI0x,y= gh0x,y∙Δvx0x,y+gv0x,y∙Δvy0x,y+1≫1 ΔI1x,y=gh1x,y∙Δvx1x,y+gv1x,y∙Δvy1x,y+1≫1 I0'x,y=I0x,y+ΔI0x,y I1'x,y=I1x,y+ΔI1x,y | (1) |
| --- | --- |

where gh0(x,y) and gv0(x,y), gh1(x,y) and gv1(x,y) are horizontal and vertical gradients in L0 and L1 directions; Δvx0(x,y) and Δvy0(x,y), Δvx1(x,y) and Δvy1(x,y) are horizontal and vertical motion refinements in L0 and L1 directions; I0'x,y and I1'x,y are the L0 and L1 refined samples after the PROF.

- Combine the refined L0 and L1 prediction signal, i.e.,

| Ibix,y=W0∙I0'x,y+W1∙I1'x,y+Offset≫shift | (2) |
| --- | --- |

where W0 and W1 are WP/BCW weights; shift and Offset are the offset and right shift that are applied to calculate the weighted average of the WP and BCW.

Based on the existing design of motion compensation (MC), for internal bit-depth from 8- to 12-bit, the prediction samples I0x,y and I1x,y are within the range of 16-bit signed integer. Therefore, after the PROF sample refinement, the dynamic of the refined samples I0'x,y and I1'x,y will be increased to 17-bit. Therefore, the current PROF design may cause 16-bit multiplication overflow when it is combined with the WP or the BCW.

# Proposal

To fix the above 16-bit multiplication overflow issue, two methods are proposed to the current PROF design in VVC draft 6:

- Method One: As calculated in annex, the range of affine prediction samples are [-23314, 23296] for 8- to 12-bit input video. Therefore, in the first method, the PROF sample refinements ΔI0x,y and ΔI1x,y are clipped to the range of [-2dI-1, 2dI-1-1] where dI = max (14, BD + 2). For input videos of 8- to 12-bit, dI is equal to 14 and the range is equal to14-bit signed integer, i.e., [-8192, 8191], such that the range of the refined prediction samples after the PROF is [-31506, 31487], i.e., within 16-bit signed range.

| ΔI0x,y=clip3-2max14, BD+2-1,2max14, BD+2-1-1, ΔI0x,y ΔI1x,y=clip3-2max14,BD+2-1,2max14, BD+2-1-1, ΔI1x,y | (3) |
| --- | --- |

- Method two: In the second method, instead of clipping PROF refinements, the refined prediction samples are directly clipped into the range of [-2dR-1, 2dR-1-1] where dR = max(16, BD + 4). For input videos of 8- to 12-bit, dR is equal to 16 and the range is equal to 16-bit signed range, i.e.,

| I0'x,y=clip3-2max16,BD+4-1,2max16, BD+4-1-1, I0'x,y I1'x,y=clip3-2max16,BD+4-1,2max16,BD+4-1-1, I1'x,y | (4) |
| --- | --- |

# Simulation results

Method one and Method two were implemented based on the VTM-6.0 [3] and tested using common test conditions (CTCs) as specified in [4].

Table 1 BD-rate performance of Method One

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class F | 0.01% | 0.01% | 0.01% | 98% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.07% | 0.20% | 0.00% | 100% | 101% |

Table 2 BD-rate performance of Method Two

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 105% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 105% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 102% |

# Proposed spec changes

## Method One

#### 8.5.6.4 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

Variable dLimit is set equal to 1<<(max (14, BitDepthY + 2) - 1).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3(-dLimit, dLimit, ( dI + 1 ) >> 1 )	(8-799)

## Method Two

#### 8.5.6.4 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) * (sbHeight + borderExtension) prediction sample array predSamples,

- one (sbWidth * sbHeight) motion vector difference array diffMv.

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variable shift1 is set equal to Max( 6, BitDepthY − 6 ).

Variable dLimit is set equal to 1<<(max(16, BitDepthY + 4)-1).

For x =0..sbWidth − 1, y =0..sbHeight − 1, the following ordered steps apply:

- The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-796)

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] >> shift1 ) − ( predSamples[ x ][ y ] >> shift1 )	(8-797)

- The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]	(8-798)

- Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = Clip3(-dLimit, dLimit, predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 ) )	(8-799)

# Reference

J. Luo, Y. He, “CE4-2.1: Prediction refinement with optical flow for affine mode”, JVET-O0070, July 2019, Gothenburg, Sweden.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Annex

# Dynamic range analysis for affine prediction samples

In the annex, the dynamic range of the prediction samples for affine mode are measured. In the following calculation, it is assumed that both horizontal and vertical motion vectors (MVs) point to fractional samples at half-pel sample positions (i.e., 6-tap filter [3, -11, 40, 40, -11, 3] is applied in both horizontal and vertical interpolations) which corresponds to the worst-case bit-width of the interpolated samples. The dynamic range of the intermediate data after the first interpolation (i.e., horizontal interpolation) is calculated by setting the values of the input reference samples associated with positive and negative filter coefficients to the maximum value (i.e., 2BD-1) and the minimum value (i.e., 0), respectively. Specifically, the minimum and maximum of output data from the horizontal interpolation, i.e., MinValh and MaxValh, are obtained by

| MinValh=SumNeg∙2BD-1+SumPos∙0+Oh≫Sh MaxValh=SumNeg∙0+SumPos∙2BD-1+Oh≫Sh | (6) |
| --- | --- |

where Oh and Sh are the offset and the number of right shifts that are applied at horizontal interpolation stage, which are equal to –(1<<13)<<min(4, BD-8) and min(4, BD-8), respectively. SumNeg and SumPos are the sums of negative and positive filter coefficients. BD is the internal coding bit-depth.

Then, the dynamic range of the second interpolation (i.e., vertical interpolation) is calculated by setting the values of the input data for the second interpolation to the minimum and maximum possible output from the first interpolation. Specifically, the minimum and maximum of output data from the vertical interpolation, i.e., MinValv and MaxValv, are calculated as

| MinValv=SumNeg∙MaxValh+SumPos∙MinValh+Ov≫Sv MaxValv=SumNeg∙MinValh+SumPos∙MaxValh+Ov≫Sv | (7) |
| --- | --- |

where Ov and Sv are the offset and the number of right shifts that are applied at vertical interpolation stage, which are equal to 0 and 6, respectively. Assuming the internal bit-depth BD is equal to 10, Table 3 shows the values of the intermediate parameters in the above calculation.

Table 3 Dynamic range derivation of affine prediction samples

| Parameter | SumNeg | SumPos | MinValh | MaxValh | MinValv | MaxValv |
| --- | --- | --- | --- | --- | --- | --- |
| Value | -22 | 86 | -13819 | 13802 | -23314 | 23296 |

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).