| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0519 |
| --- | --- |

| Title: | Non-CE4: On SAD threshold for BDOF early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is reported that there is one mismatch between VVC draft 6 and VTM-6.0 on the SAD calculation for early terminating the BDOF process. In the VVC draft 6, the SAD threshold used to bypass the BDOF process is dependent on the internal bit-depth of L0 and L1 prediction signals. In the VTM-6.0, the SAD threshold is independent of internal bit-depth and calculated based on the 10-bit intermediate prediction samples used for DMVR motion refinement. Since the SAD value dmvrSAD is always calculated as a 10-bit value, it is proposed to align the specification draft with the VTM-6.0 reference software. Otherwise, the SAD value dmvrSAD could be compared with a SAD threshold with different bit-depth based on the current VVC draft.

# Introduction

At the 15th JVET meeting, JVET-O0055 was adopted to simplify the early termination method of the BDOF [1]. With such adoption, when both DMVR and BDOF are enabled, the SAD value that is calculated at searching center for each 16x16 subblock, i.e., offset (0, 0), is used to early terminate the BDOF of the subblock. Specifically, if the SAD of the subblock is larger than or equal to one predefined threshold, the BDOF is applied after the DMVR of the subblock is done; otherwise, the BDOF of the subblock will be bypassed.

It was found that there is one mismatch between VVC draft 6 [2] and VTM-6.0 [3] on calculating the SAD threshold for the BDOF early termination. Specifically, in the VVC draft 6, the SAD threshold is calculated depending the internal coding bit-depth, as depicted as

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variable bdofBlkDiffThres is set equal to 1 << ( BitDepthY − 4 + shift ).

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables sbBdofFlag and the bi-directional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

- If dmvrFlag is equal to 1 and the variable dmvrSAD is less than ( ( ( sbHeight >> 2 ) * ( sbWidth >> 2 ) * bdofBlkDiffThres ) >> 5 ), the variable sbBdofFlag is set equal to FALSE and bdofUtilizationFlag[ xIdx ][ yIdx ] is set equal to FALSE

- .Otherwise, the variable sbBdofFlag is set equal to TRUE and bdofUtilizationFlag[ xIdx ][ yIdx ] is set equal to TRUE.

On the other hand, in the VTM-6.0, one fixed SAD threshold is calculated as

bdofBlkDiffThres = 2 * sbHeight * sbWidth

Given that the intermediate prediction samples (generated based on bilinear filters) used for the DMVR motion refinement is fixed to be 10-bit, the SAD threshold used for the BDOF early termination should be independent from internal coding bit-depth. Therefore, this contribution proposes to change the specification text to be aligned with the VTM-6.0 reference software. The proposed change has no BD-rate impact under common test condition (CTC).

# Proposed spec changes

### 8.5.6 Decoding process for inter blocks

#### 8.5.6.1 General

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variable bdofBlkDiffThres is set equal to 1 << ( BitDepthY − 4 + shift ) 2 * sbWidth * sbHeight.

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables sbBdofFlag and the bi-directional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

- If dmvrFlag is equal to 1 and the variable dmvrSAD is less than ( ( ( sbHeight >> 2 ) * ( sbWidth >> 2 ) * bdofBlkDiffThres ) >> 5 ), the variable sbBdofFlag is set equal to FALSE and bdofUtilizationFlag[ xIdx ][ yIdx ] is set equal to FALSE

- .Otherwise, the variable sbBdofFlag is set equal to TRUE and bdofUtilizationFlag[ xIdx ][ yIdx ] is set equal to TRUE.

# Reference

C.-Y. Lai, et al, “CE9-1.2 and CE9-1.4: BDOF early termination”, JVET-O0055, July 2019, Gothenburg, Sweden.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).