| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0524 |
| --- | --- |

| Title: | Non-CE4/AHG17: On high-level control flags of BDOF and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC draft 6, bidirectional optical flow (BDOF) and prediction refinement with optical flow (PROF) are two sample refinement methods that are built upon optical flow model. In the current VVC design, the BDOF can be enabled/disabled based on the control flags in sequence parameter set (SPS) and slice header while the PROF can only be controlled at SPS level. In this contribution, it is proposed to add slice level control to enable/disable the PROF.

# Proposal

In VVC draft 6 [1], two optical-flow-based sample refinement tools, i.e., BDOF and PROF, are applied to enhance inter prediction efficiency. Both two tools derive one offset for each prediction sample by multiplying the gradient and the local delta motion at the sample position. According to the current VVC design, the BDOF can be adaptively turned on/off at both sequence level (by the flag sps_bdof_enabled_flag in SPS) and slice level (by the flag slice_disable_bdof_dmvr_flag in slice header). However, the PROF is only controlled by the flag sps_affine_prof_enabled_flag in SPS.

Similar to the BDOF, it is not guaranteed that the PROF can always work very efficiently. In this contribution, it is proposed to introduce slice level on/off control for the PROF. Specifically, the following three methods are proposed in the contribution.

## Method One

In the first method, it is proposed to introduce one new control flag slice_disable_prof_flag at the slice header syntax table to enable and disable the PROF for one slice.

## Method Two

In the second method, it is proposed to reuse the existing slice header control flag slice_disable_bdof_dmvr_flag used for disabling the BDOF and DMVR to additionally control on/off of the PROF for one slice. Additionally, it is proposed to rename the flags sps_bdof_dmvr_slice_present_flag and slice_disable_bdof_dmvr_flag as sps_bdof_prof_dmvr_slice_present_flag slice_disable_bdof_prof_dmvr_flag.

## Method Three

The third method is based on the second method. In the second method, the BDOF and PROF are jointly enabled/disabled by the same control flag at slice level, but separately controlled at SPS level using two different flags, i.e. sps_bdof_enabled_flag and sps_affine_prof_enabled_flag. To achieve one more unified design, in the third method, it is proposed to introduce one new SPS flag sps_bdof_prof_enabled_flag to replace the existing flags sps_bdof_enabled_flag and sps_affine_prof_enabled_flag to jointly control the BDOF and the PROF in the SPS.

# Proposed spec changes

## Method one

| slice_header( ) { | Descriptor |
| --- | --- |
| if( sps_bdof_dmvr_slice_present_flag ) { |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| slice_disable_prof_flag | u(1) |
| } |  |
| } |  |

slice_disable_prof_flag equal to 1 specifies that prediction refinement with optical flow is disabled for affine coding blocks in the current slice. slice_disable_prof_flag equal to 0 specifies that prediction refinement with optical flow is enabled for affine coding blocks in the current slice. When not present, the value of slice_disable_prof_flag is inferred to 0.

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

The variable cbProfFlagLX is derived as follows:

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

  - sps_affine_prof_enabled_flag is equal to 0 or slice_disable_prof_flag is equal to 1.

  - fallbackModeTriggered is equal to 1.

  - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

  - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

- Otherwise, cbProfFlagLX set equal to TRUE

## Method Two

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| if( sps_bdof_enabled_flag ¦ ¦ sps_prof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flagsps_bodf_prof_dmvr_slice_present_flag | u(1) |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| if( sps_bdof_dmvr_slice_present_flagsps_bdof_prof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag | u(1) |
| } |  |

slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag equal to 1 specifies that neither of bi-directional optical flow inter prediction, prediction refinement with optical flow or decoder motion vector refinement based inter bi-prediction is enabled in the current slice. slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag equal to 0 specifies that bi-directional optical flow inter prediction, prediction refinement with optical flow or decoder motion vector refinement based inter bi-prediction may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is not present, the value of slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is inferred to be 0.

### 8.5.1 General decoding process for coding units coded in inter prediction mode

- When all of the following conditions are true, dmvrFlag is set equal to 1:

  - sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag is equal to 0

  - general_merge_flag[ xCb ][ yCb ] is equal to 1

  - both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

  - mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

  - ciip_flag[ xCb ][ yCb ] is equal to 0

  - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

  - BcwIdx[ xCb ][ yCb ] is equal to 0

  - Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

  - cbWidth is greater than or equal to 8

  - cbHeight is greater than or equal to 8

  - cbHeight*cbWidth is greater than or equal to 128

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

The variable cbProfFlagLX is derived as follows:

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

  - sps_affine_prof_enabled_flag is equal to 0 or slice_disable_bdof_prof_dmvr_flag is equal to 1.

  - fallbackModeTriggered is equal to 1.

  - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

  - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

- Otherwise, cbProfFlagLX set equal to TRUE

### 8.5.6.1 General

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

  - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is equal to 0.

  - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

  - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

  - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

  - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

  - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

  - ciip_flag[ xCb ][ yCb ] is equal to 0.

  - BcwIdx[ xCb ][ yCb ] is equal to 0.

  - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

  - cbWidth is greater than or equal to 8.

  - cbHeight is greater than or equal to 8.

  - cbHeight * cbWidth is greater than or equal to 128.

## Method Three

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_bdof_enabled_flagsps_bdof_prof_enabled_flag | u(1) |
| if( sps_bdof_enabled_flagsps_bdof_prof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flagsps_bdof_prof_dmvr_slice_present_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| } |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| if( sps_bdof_dmvr_slice_present_flagsps_optical_bdof_prof_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag | u(1) |
| } |  |

sps_bdof_enabled_flag equal to 0 specifies that the bi-directional optical flow inter prediction is disabled. sps_bdof_enabled_flag equal to 1 specifies that the bi-directional optical flow inter prediction is enabled.

sps_affine_prof_enabled_flag specifies whether the prediction refinement with optical flow can be used for affine motion compensation. If sps_affine_prof_enabled_flag is equal to 0, the affine motion compensation shall not be refined with optical flow. Otherwise (sps_affine_prof_enabled_flag is equal to 1), the affine motion compensation can be refined with optical flow. When not present, the value of sps_affine_prof_enabled_flag is inferred to be equal to 0.

sps_bodf_prof_enabled_flag equal to 1 specifies that bidirectional optical flow can be enabled for the bi-prediction of non-affine inter modes and the prediction refinement with optical flow can be enabled for the prediction of affine mode. sps_bdof_prof_enabled_flag equal to 0 specifies that neither bidirectional optical flow or prediction refinement with optical flow is enabled. When not present, the value of sps_bdof_prof_enabled_flag is inferred to 0.

slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag equal to 1 specifies that neither of bi-directional optical flow inter prediction, prediction refinement with optical flow and decoder motion vector refinement based inter bi-prediction is enabled in the current slice. slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag equal to 0 specifies that bi-directional optical flow inter prediction, prediction refinement with optical flow or decoder motion vector refinement based inter bi-prediction may or may not be enabled in the current slice. When slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is not present, the value of slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is inferred to be 0.

### 8.5.1 General decoding process for coding units coded in inter prediction mode

- When all of the following conditions are true, dmvrFlag is set equal to 1:

  - sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flagslice_disable_bdof_prof_dmvr_flag is equal to 0

  - general_merge_flag[ xCb ][ yCb ] is equal to 1

  - both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

  - mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

  - ciip_flag[ xCb ][ yCb ] is equal to 0

  - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

  - BcwIdx[ xCb ][ yCb ] is equal to 0

  - Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

  - cbWidth is greater than or equal to 8

  - cbHeight is greater than or equal to 8

  - cbHeight*cbWidth is greater than or equal to 128

#### 8.5.5.9 Derivation process for motion vector arrays from affine control point motion vectors

The variable cbProfFlagLX is derived as follows:

- If one or more of the following conditions are true, cbProfFlagLX is set equal to FALSE.

  - sps_affine_prof_enabled_flagsps_bdof_prof_enabled_flag is equal to 0 or slice_disable_bdof_prof_dmvr_flag is equal to 1.

  - fallbackModeTriggered is equal to 1.

  - numCpMv is equal to 2 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

  - numCpMv is equal to 3 and cpMvLX[ 1 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 1 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ] and cpMvLX[ 2 ][ 0 ] is equal to cpMvLX[ 0 ][ 0 ] and cpMvLX[ 2 ][ 1 ] is equal to cpMvLX[ 0 ][ 1 ].

- Otherwise, cbProfFlagLX set equal to TRUE

### 8.5.6.1 General

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

  - sps_bdof_enabled_flagsps_bdof_prof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag slice_disable_bdof_prof_dmvr_flag is equal to 0.

  - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

  - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

  - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

  - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

  - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

  - ciip_flag[ xCb ][ yCb ] is equal to 0.

  - BcwIdx[ xCb ][ yCb ] is equal to 0.

  - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

  - cbWidth is greater than or equal to 8.

  - cbHeight is greater than or equal to 8.

  - cbHeight * cbWidth is greater than or equal to 128.

### 8.5.6.3.1 General

The prediction block border extension size brdExtSize is derived as follows:

brdExtSize = ( bdofFlag | | ( inter_affine_flag[ xSb ][ ySb ] && sps_affine_prof_enabled_flagsps_bdof_prof_enabled_flag ) ) ? 2 : 0	(8-752)

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flagsps_bdof_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, Sweden.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).