| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0525 |
| --- | --- |

| Title: | AHG18: Residual coding selection signaling for lossless VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Kwai Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution is proposed to signal 1 bit syntax to select residual or transform coefficient coding scheme for lossless. Four tests are provided in this contribution.

The test1 is signaling 1 bit syntax in cu level to select either residual coding or transform coefficient coding for current CU. The result of test1 is XXX %, XXX %, and XXX % in All Intra, RA, and LDB, respectively.

The test2 is based on JVET-P0058 [1] and signaling 1 bit syntax by reusing the transform_skip_flag to indicate the selection between residual coding and transform coefficient coding scheme for non-BDPCM modes. The result of test1 is XXX %, XXX %, and XXX % in All Intra, RA, and LDB, respectively. The result of test2 is XXX%, XXX%, XXX% in All Intra, RA, and LDB, respectively.

The test 3 is based on JVET-P0059 [2], enabling BDPCM and Transform-skip coding for chroma. The selection of coding scheme is signaled by transform_skip_flag. The result of test3 is XXX%, XXX%, XXX% in All Intra, RA, and LDB, respectively.

The test 4 is an alternative of test 1, same encoder but different decoder by signaling 1 bit syntax in tb level to indicate the selection of coding scheme for non-BDPCM modes. The result of test4 is XXX%, XXX%, XXX% in All Intra, RA, and LDB, respectively.

# Introduction

In current VVC [3], both the residual coding used for transform skip mode and the transform coefficient coding scheme can be applied to code the residual blocks under lossless mode. However, the residual coding scheme is only applied if the coding mode is luma BDPCM or the transform mode is luma transform skip. If the coding mode is not luma BDPCM or the transform is not luma transform skip mode, the residual block is coded by transform coefficient coding scheme in current lossless VVC [3]. Since it is asserted that the selection of coding scheme based on luma BDPCM and luma transform skip mode may not be optimal for lossless coding, it is proposed to signal 1 bit syntax to select which coding scheme (i.e., residual coding or transform coefficient coding) in lossless coding. Two tests are implemented in this proposal, first is in CU level signaling and second is TB level signaling.

# Implementation

## CU level signaling

In test 1, the anchor is from [4]. The signaling conditioned by luma BDPCM and luma transform mode for lossy coding is not modified. One additional syntax, cu.residualCodingFlag, is signaling in CU level to indicate the coding scheme under lossless coding. In TB level, the coding scheme selection is defined as followed.

If one of following condition is true, the residual coding is applied for current CU.

- tu.mtsIdx is set equal to MTS_SKIP

- cIdx (component index) is set equal to COMPONENT_Y and intra_bdpcm_flag is set equal to 1

- cu.residualCodingFlag is set equal to 1

If cu.residualCodingFlag is set equal to 1, it indicates that the residual coding for transform skip mode is selected for current residual block under lossless mode. If cu.residualCodingFlag is not signaled, it is inferred to be 0.

Meanwhile, if the residual coding for transform skip is not selected, the transform coefficient coding is applied to code current residual block under lossless coding.

## TB level signaling with CE8-2.1

In test 2, the test is CE8-2.1 combined with AHG18 lossless VVC. The test 2 is enabling the transform skip mode for both luma and chroma components under lossless coding. It reuses the transform_skip_flag to indicate the between residual coding scheme and transform coefficient coding scheme. In this test, the selection of coding scheme for non-BDPCM mode is indicated by transform_skip_flag. For BDPCM mode, it is not modified.

## TB level signaling with CE8-4.1

In test 3, test is modified from CE8-4.1. The selection of coding scheme solely dependents on the transform_skip_flag, even for BDPCM mode. This means that, under lossless coding, BDPCM can be applied with transform coefficient coding for both luma and chroma components if this combination is optimal and selected by encoder. Since the BDPCM is applied, the test 3 is simulated with “--BDPCM=2” in configuration.

## TB level signaling with Test 1 Enc

The test 4 is modified from test 1. The selection of coding scheme is indicated by 1 bit syntax tu.residualCodingFlag.

If one of following codition is true, the residual coding is applied for current TB.

- tu.mtsIdx is set equal to MTS_SKIP

- cIdx (component index) is set equal to COMPONENT_Y and intra_bdpcm_flag is set equal to 1

- tu.residualCodingFlag is set equal to 1

If tu.residualCodingFlag is set equal to 1, it indicates that the residual coding for transform skip mode is selected for current residual block under lossless mode. If tu.residualCodingFlag is not signaled, it is inferred to be 0.

Meanwhile, if the residual coding for transform skip is not selected, the transform coefficient coding is applied to code current residual block under lossless coding.

# Experimental results

In test 1, the anchor is AHG18 lossless VVC [4] while the test is proposed CU level signaling to indicate which coding scheme is used in lossless mode.

In test 2, the anchor is CE8-2.1 [1] combined with [4] while the test is proposed TB level signaling to indicate which coding scheme is used in lossless mode.

|  | All Intra |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST1 |  | AHG18 | TEST1 |  | AHG18 | TEST1 |  |
| Class A1 | 2.2 | 2.2 | -0.16% | 2.3 | XX | XXX % |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.04% | 1.9 | 1.9 | -0.01% |  |  |  |
| Class B | 2.1 | 2.1 | 0.07% | 2.3 | 2.3 | 0.00% | XX | XX | XXX % |
| Class C | 2.1 | 2.1 | 0.01% | 2.6 | 2.6 | -0.05% | 2.6 | 2.6 | -0.05% |
| Class D | 1.9 | 2.0 | -0.17% | 2.8 | 2.6 | -0.10% | 2.7 | 2.7 | -0.13% |
| Class E | 2.9 | 2.9 | -0.03% |  |  |  | 3.3 | 3.3 | 0.02% |
| Class F | 5.5 | 5.6 | -1.18% | 39.4 | 41.1 | -2.63% | 78.8 | XX | XXX % |
| TGM | 12.3 | 12.5 | -2.17% | 112.6 | 129.4 | -10.50% | 186.1 | 166.6 | -12.04% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -0.02% | 2.3 | XX | XXX % | XX | XX | XXX % |
| Enc Time[%] | 186% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | 102% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |  |

Table 1: Simulation result of test 1 (to be updated)

|  |  | All Intra |  |  |  | Random Access |  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST2 |  | AHG18 | TEST2 |  | AHG18 | TEST2 |  |
| Class A1 | 2.2 | 2.2 | -0.18% | 2.3 | 2.3 | -0.10% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.00% | 1.9 | XX | XXX % |  |  |  |
| Class B | 2.1 | 2.1 | -0.05% | 2.3 | 2.3 | -0.03% | 2.3 | XX | XXX % |
| Class C | 2.1 | 2.1 | -0.14% | 2.6 | 2.6 | -0.09% | 2.6 | 2.6 | -0.09% |
| Class D | 1.9 | 2.0 | -0.18% | 2.8 | 2.8 | -0.07% | 2.7 | 2.7 | -0.10% |
| Class E | 2.9 | 2.9 | -0.33% |  |  |  | 3.3 | 3.3 | 0.00% |
| Class F | 5.5 | 5.5 | -1.09% | 39.4 | 40.7 | -1.84% | 78.8 | XX | XXX % |
| TGM | 12.3 | 12.8 | -3.61% | 112.6 | 115.5 | -2.62% | 142.4 | 146.3 | -2.70% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -0.13% | 2.3 | XX | XXX % | 0.6 | XX | XXX % |
| Enc Time[%] |  | 111% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |
| Dec Time[%] |  | 101% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |

Table 2: Simulation result of test 2 (to be updated)

|  | All Intra |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST3 |  | AHG18 | TEST3 |  | AHG18 | TEST3 |  |
| Class A1 | 2.2 | 2.2 | -1.40% | 2.3 | XX | XXX % |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.44% | 1.9 | XX | XXX % |  |  |  |
| Class B | 2.1 | 2.1 | -1.67% | 2.3 | 2.3 | -0.33% | 2.3 | XX | XXX % |
| Class C | 2.1 | 2.1 | -1.93% | 2.6 | 2.6 | -0.47% | 2.6 | 2.6 | -0.57% |
| Class D | 1.9 | 2.0 | -3.25% | 2.8 | 2.8 | -0.69% | 2.7 | 2.8 | -0.61% |
| Class E | 2.9 | 3.0 | -2.32% |  |  |  | 3.3 | XX | XXX % |
| Class F | 5.5 | 5.7 | -4.42% | 39.4 | 41.1 | -3.31% | 60.6 | XX | XXX % |
| TGM | 12.3 | 12.7 | -6.87% | 112.6 | 115.5 | -3.88% | 142.4 | XX | XXX % |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -1.30% | 2.7 | XX | XXX % | 1.7 | XX | XXX % |
| Enc Time[%] | 107% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |  |
| Dec Time[%] | 95% |  |  |  | #NUM! |  |  |  | #NUM! |  |  |  |

Table 3: Simulation result of test 3 (to be updated)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST2 |  | AHG18 | TEST2 |  | AHG18 | TEST2 |  |
| Class A1 | 2.2 | 2.2 | -0.18% | 2.3 | XX | XXX % |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.02% | 1.9 | XX | XXX % |  |  |  |
| Class B | 2.1 | 2.1 | -0.05% | 2.3 | 2.3 | -0.01% | 2.3 | XX | XXX % |
| Class C | 2.1 | 2.1 | -0.14% | 2.6 | 2.6 | -0.09% | 2.6 | 2.6 | -0.02% |
| Class D | 1.9 | 2.0 | -0.18% | 2.8 | 2.8 | -0.07% | 2.7 | 2.7 | -0.10% |
| Class E | 2.9 | 2.9 | -0.33% |  |  |  | 3.3 | XX | XXX % |
| Class F | 5.5 | 5.5 | -1.09% | 39.4 | 41.0 | -2.22% | 60.6 | XX | XXX % |
| TGM | 12.3 | 12.8 | -3.61% | 112.6 | 115.5 | -10.11% | 142.4 | 166.5 | -11.72% |
| Overall (A1, A2, B, C, E) | 2.2 | 2.2 | -0.05% | 9.1 | XX | XXX % | 0.6 | XX | XXX % |
| Enc Time[%] | 196% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 101% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 4: Simulation result of test 4 (to be updated)

| CTC | AI | RA | LDB | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Test 1 (1bit cu signal) | -0.02% | -0.03% | -0.01% | 186%/179%/XXX% | 102%/100%/XXX% |
| Test 2 (on top CE8-2.1) | -0.13% | -0.05% | -0.04% | 111%/113%/XXX% | 101%/101%/XXX% |
| Test 3 (on top CE8-4.1) | -1.75% | -0.23% | -0.59% | 117%/XXX%/XXX% | 90%/XXX%/XXX% |
| Test 4 (1bit TB level) | -0.05% | -0.02% | 0.01% | 196%/178%/XXX% | 101%/98%/XXX% |

| F/TGM | AI | RA | LDB | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Test 1 (1bit cu signal) | -1.18% -2.17% | -2.63% -10.50% | -4.15% -12.04% | 176%/161%/XXX% | 99%/100%/XXX% |
| Test 2 (on top CE8-2.1) | -1.09% -3.61% | -1.84% -2.62% | 2.96% -2.70% | 111%/116%/XXX% | 101%/102%/XXX% |
| Test 3 (on top CE8-4.1) | -4.42% -6.87% | -3.31% -3.88% | -4.12% -3.21% | 115%/115%/XXX% | 90%/96% /XXX% |
| Test 4 (1bit TB level) | -0.99% -2.22% | -2.22% -10.11% | -3.68% -11.72% | 182%/166%/XXX% | 98%/99%/XXX% |

# Runtime comparison

In test1, because every CU is encoded two times, one with residual coding scheme, the other with transform coding scheme, the runtime of test in CU level signaling is two times of AHG18 anchor. However, the runtime of proposed method in test1 is comparable with lossy coding mode if qp is set equal to 2. Table 3 is the result that compares the runtime of lossy coding when qp is set qual to 2 and the proposed method in test 1 for the first 100 frame in each sequence.

| Run time | Lossy if qp is 2 (hr) (All Intra/RA/LDB) | Proposed method in test 1 (hr) (All Intra/RA/LDB) | Ratio (%) (All Intra/RA/LDB) |
| --- | --- | --- | --- |
| A1 | 15.96/ 270.38 / N/A | 19.86 / 355.41 / N/A | 124% / 131% / N/A |
| A2 | 22.62/ XXX / N/A | 20.27 / 359.00 / N/A | 89% / XX / N/A |
| B | 4.69/ 67.41 / 53.43 | 5.17 / 88.94 / 61.13 | 110% / 131% / 114% |
| C | 1.09/ 16.96 / 12.81 | 1.04 / 18.35 / 11.74 | 95% / 108% / 92% |
| D | 0.27/ 3.98 / 2.80 | 0.24 / 4.03 / 2.94 | 90% / 101% / 105% |
| E | 2.50/ / 29.94 | 2.82 / / 27.02 | 112% / / 90% |
| F | 2.09/ 11.05 / 9.50 | 2.09 /12.37 / 9.22 | 100% / 115% / 97% |
| TGM | 3.07/ 8.25 / 7.35 | 2.70/7.95 / 5.71 | 88% / 91% / 77% |
| Overall | 3.07/ XXX / 12.60 | 3.09/ XXX / 12.00 | 101%/ XXX / 95% |

Table 3: Runtime comparison between lossy coding if qp is 2 and proposed Test1 (to be updated)

# Conclusion

This contribution presents several methods to improve the coding efficiency for lossless VVC by signaling 1 bit with extra or reused syntax. In the test 1 and test 4, signaling with extra 1 bit to indicate the coding scheme brings coding gain in all class and high gain in TGM class. In the test 2, signaling by reusing transform_skip_flag in TB level for non-BDPCM modes also provides significant coding gain in lossless VVC. In the test 3, for both BDPCM and non-BDPCM can be applied with either transform coefficient coding or residual coding for Transform-skip mode, it brings interesting coding gain in lossless VVC. It is recommended to adopt one of proposed method regarding the improvement of coding efficiency under lossless coding.

# References

[1] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, the 16th JVET meeting, Geneva, 1-11 October 2019

[2] G. Clare (bcom), F. Henry (Orange), T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony), “CE8-4.1: BDPCM and Transform skip for Chroma”,  the 16th JVET meeting, Geneva, 1-11 October 2019

[3] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[4] AHG18 reference software, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM, branch VTM-6.0-Lossless.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).