The change for test1 is highlight as following.

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| init_qp_minus26 | se(v) |
| pps_transquant_bypass_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( pps_transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| … |  |
| if( cu_cbf ) { |  |
| if(!cu_transquant_bypass_flag && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if(!cu_transquant_bypass_flag && Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( LfnstDcOnly = = 0 && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

#### Transform unit syntax

| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && cu_residual_coding_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |

#### 7.4.3.4 Picture parameter set RBSP semantics

…

pps_transquant_bypass_enabled_flag equal to 1 specifies that cu_transquant_bypass_flag is present. transquant_bypass_enabled_flag equal to 0 specifies that cu_transquant_bypass_flag is not present.

…

#### Coding unit semantics

The following assignments are made for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

CbPosX[ chType ][ x ][ y ] = x0		(7-134)

CbPosY[ chType ][ x ][ y ] = y0		(7-135)

CbWidth[ chType ][ x ][ y ] = cbWidth		(7-136)

CbHeight[ chType ][ x ][ y ] = cbHeight		(7-137)

CqtDepth[ chType ][ x ][ y ] = cqtDepth		(7-138)

The variable CclmEnabled is derived by invoking the cross-component chroma intra prediction mode checking process specified in clause 8.4.4 with the luma location ( xCb, yCb ) set equal to ( x0, y0 ) as input.

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in clause 8.6 and the in-loop filter process as specified in clause 8.7 are bypassed. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

cu_residual_coding_flag equal to 1 specifies where the residual coding for transform skip is selected for coding residual blocks. When cu_residual_coding_flag is not present, it is inferred to be equal to 0.

…

### Scaling and transformation process

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable cIdx specifying the colour component of the current block,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height.

Output of this process is the (nTbW)x(nTbH) array of residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables bitDepth, bdShift and tsShift are derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC		(8-942)

bdShift = Max( 20 − bitDepth, 0 )		(8-943)

tsShift = 5 + ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 )		(8-944)

The variable codedCIdx is derived as follows:

- If cIdx is equal to 0 or TuCResMode[ xTbY ][ yTbY ] is equal to 0, codedCIdx is set equal to cIdx.

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 1 or 2, codedCIdx is set equal to 1.

- Otherwise, codedCIdx is set equal to 2.

The variable cSign is set equal to ( 1 − 2 * slice_joint_cbcr_sign_flag ).

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

  - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

  - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  								 (8-xxx)

- Otherwise, if transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	(8-945)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

  - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-946)