| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0527 |
| --- | --- |

| Title: | Non-CE6: On context model for coding last NZ position |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma, Yi-wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang, 8910 University Center Dr., Suite 400 San Diego, CA-92122, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution is proposed to align the context model selection of last non-zero coefficient position coding with the size of zero-out TUs. The proposed modification reduces 10 context models.

The result of test shows that the coding loss is 0.02%, 0.03%, XXX% for All Intra, RA, and LDB, respectively.

# Introduction

In current VVC [1], the context model selection for the last non-zero coefficient position coding is determined by the original TU size. In this contribution, one test is implemented to show the performance impact if the selection of context model is selected by the size of zero-out TUs.

# Experimental results

Tests were carried out by using the VTM6.0 [1].

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.04% | 100% | 101% |
| Class A2 | 0.04% | -0.01% | -0.02% | 101% | 100% |
| Class B | 0.01% | 0.02% | 0.02% | 101% | 101% |
| Class C | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class E | 0.00% | -0.02% | -0.02% | 102% | 99% |
| Overall | 0.02% | 0.02% | 0.01% | 101% | 100% |
| Class D | 0.00% | 0.04% | 0.01% | 99% | 101% |
| Class F | -0.01% | -0.06% | -0.06% | 99% | 100% |
| Class TGM | 0.00% | 0.01% | 0.00% | 101% | 103% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.05% | 0.02% | 102% | 103% |
| Class A2 | 0.04% | -0.05% | 0.07% | 103% | 103% |
| Class B | 0.02% | 0.00% | 0.06% | 98% | 97% |
| Class C | 0.01% | 0.16% | 0.07% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.06% | 101% | 101% |
| Class D | -0.02% | -0.04% | 0.02% | 105% | 102% |
| Class F | 0.00% | -0.01% | 0.01% | 103% | 100% |
| Class TGM | -0.01% | -0.01% | -0.02% | 101% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.03% | -0.07% | 101% | 101% |
| Class E | 0.03% | 1.17% | 0.68% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | 0.23% | 0.53% | 100% | 94% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | 0.00% | -0.08% | -0.04% | 103% | 110% |

Table 1: Simulation result of test (To be updated)

# Conclusion

This contribution presents one test to select the context model of last non-zero coefficient position coding based on the size of zero-out TUs. It is recommended to adopt the second method regarding minor performance impact.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).