| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0528 |
| --- | --- |

| Title: | AHG18: On residual scanning order for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma, Yi-wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang, 8910 University Center Dr., Suite 400 San Diego, CA-92122, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution is proposed to align the scanning order for residual blocks if transform coefficient coding is applied for lossless coding. The result shows that the coding gain is -0.42%, XXX%, XXX% for All Intra, RA, and LDB, respectively, if the scanning order of transform coefficient coding is aligned with the residual coding for transform skip in lossless coding.

# Introduction

In current VVC [1], both the residual coding used for transform skip mode and the transform coefficient coding scheme can be applied to code the residual blocks under lossless mode. If the residual coding designed for transform skip mode is applied, it codes the residual block samples based on the scan order from top-left of the block to bottom-right of the block. If the transform coefficient coding scheme is applied, it codes the residual block samples based on the exactly reversed scan order from bottom-right of the block to top-left of the block. Meanwhile, it is asserted that the prediction potentially causes larger amplitude residues at the bottom right area of the block because of the distance between the reconstructed pixels that are used for prediction and the predicted pixels. Therefore, it is proposed to reverse the scanning order of transform coefficient coding scheme under lossless coding.

# Experimental results

Test is carried out by using the VTM6.0 reference software released by AHG18 for lossless coding as an anchor [2].

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST |  | AHG18 | TEST |  | AHG18 | TEST |  |
| Class A1 | 2.2 | 2.2 | -0.32% | 62.2 | XX | XXX% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.01% | 52.3 | XX | XXX % |  |  |  |
| Class B | 2.1 | 2.1 | -0.36% | 2.3 | XX | XXX % | 0.0 | XX | XXX % |
| Class C | 2.1 | 2.1 | -0.70% | 2.6 | 2.6 | -0.21% | 2.6 | 2.6 | -0.22% |
| Class D | 1.9 | 2.0 | -0.74% | 2.8 | 2.8 | -0.18% | 2.7 | 2.7 | -0.11% |
| Class E | 2.9 | 3.0 | -0.65% |  |  |  | 0.0 | XX | XXX % |
| Class F | 5.5 | 5.5 | -0.67% | 39.4 | 39.5 | -0.21% | 78.8 | XX | XXX % |
| TGM | 12.3 | 12.3 | -0.08% | 112.6 | 112.6 | 0.02% | 186.1 | XX | XXX % |
| Overall | 2.2 | 2.2 | -0.42% | 9.1 | XX | XXX % | 0.6 | XX | XXX % |
| Enc Time[%] | 99% |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | 99% |  |  | #NUM! |  |  | #NUM! |  |  |

Table 1: Simulation result

# Conclusion

This contribution presents to align the scanning order of residual blocks if transform coefficient is applied for lossless coding. Since the result shows coding gain in all category, it is recommended to adopt this modification in lossless VVC.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] AHG18 reference software in at https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM, branch VTM-6.0-Lossless.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).