| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0528 |
| --- | --- |

| Title: | AHG18: On residual scanning order for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-chuan Ma, Yi-wen Chen, Xiaoyu Xiu, Hong-jheng Jhu, Xianglin Wang, 8910 University Center Dr., Suite 400 San Diego, CA-92122, USA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution it is proposed to rotate the residual blocks if transform coefficient coding is applied for lossless coding. The result shows that compared with AHG18 lossless coding anchor, the coding gain is -0.42%, -0.14%, -0.12% for All Intra, RA, and LDB, respectively.

# Introduction

In current VVC [1], residual samples from intra prediction located at the bottom-right area of the block tend to have larger amplitude than those located at the top-left area, because of the distance between the reconstructed pixels that are used for prediction and the predicted pixels. When transform coefficient coding is applied for lossless residual coding, rotating the residual blocks (i.e. flipping the blocks along both horizontal and vertical direction) would place the statistically large amplitude residue at the top left area of the TU, which is more suitable for transform coefficient coding. Therefore, it is proposed to rotate the residual blocks if transform coefficient coding is applied for lossless residual.

# Experimental results

Test is carried out by using the AHG18 software for lossless coding as an anchor [2].

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST |  | AHG18 | TEST |  | AHG18 | TEST |  |
| Class A1 | 2.2 | 2.2 | -0.32% | 2.3 | 2.3 | -0.18% |  |  |  |
| Class A2 | 1.7 | 1.7 | 0.01% | 1.9 | 1.9 | -0.05% |  |  |  |
| Class B | 2.1 | 2.1 | -0.36% | 2.3 | 2.3 | -0.12% | 2.3 | 2.3 | -0.1% |
| Class C | 2.1 | 2.1 | -0.70% | 2.6 | 2.6 | -0.21% | 2.6 | 2.6 | -0.17% |
| Class D | 1.9 | 2.0 | -0.74% | 2.8 | 2.8 | -0.18% | 2.7 | 2.7 | -0.11% |
| Class E | 2.9 | 3.0 | -0.65% |  |  |  | 3.3 | 3.3 | -0.08% |
| Class F | 5.5 | 5.5 | -0.67% | 39.4 | 39.5 | -0.21% | 60.6 | 60.7 | -0.11% |
| TGM | 12.3 | 12.3 | -0.08% | 112.6 | 112.6 | 0.02% | 142.4 | 141.9 | 0.22% |
| Overall | 2.2 | 2.2 | -0.42% | 2.3 | 2.3 | -0.14% | 2.6 | 2.6 | -0.12% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 101% |  |  |

Table 1: Simulation result

# Conclusion

This contribution proposes to rotate the residual blocks if transform coefficient coding is applied for lossless coding. The results show coding gain in all three coding configurations. It is recommended to adopt this modification in VVC for lossless coding.

# Working Draft Text

The changes to align the scanning order of residual blocks in the VVC working draft [3] is highlight below.

8.7.2 Scaling and transformation process

The (nTbW)x(nTbH) array of residual samples resSamples is derived as follows:

- If all following conditions are true, the residual rotation (8-xxx) is applied,

    - cu_transquant_bypass_flag is set equal to 1,

    - cIdx is not equal to 0 or BdpcmFlag[ xCb ][ yCb ] is equal to 0

    - The residual sample array values resSamples [ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS – 1 are derived as follows:

resSamples [ x ][ y ] =

d[ xTbY ][ yTbY ][ cIdx ][ nTbS − x −1 ][ nTbS − y − 1 ]    			(8-xxx)

- Otherwise, the following ordered steps apply:

    - The scaling process for transform coefficients as specified in clause 8.7.3 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx being set equal to codedCIdx and the bit depth of the current colour component bitDepth as inputs, and the output is an (nTbW)x(nTbH) array of scaled transform coefficients d.

    - The (nTbW)x(nTbH) array of residual samples r is derived as follows:

      - If transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

r[ x ][ y ] = d[ x ][ y ]  <<  tsShift	 					(8-905)

      - Otherwise (transform_skip_flag[ xTbY ][ yTbY ] is equal to 0 or and cIdx is not equal to 0), the transformation process for scaled transform coefficients as specified in clause 8.7.4.1 is invoked with the transform block location ( xTbY, yTbY ), the transform block width nTbW and the transform block height nTbH, the colour component variable cIdx and the (nTbW)x(nTbH) array of scaled transform coefficients d as inputs, and the output is an (nTbW)x(nTbH) array of residual samples r.

    - The intermediate residual samples res [ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

res [ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift 	(8-906)

    - The residual samples resSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

      - If cIdx is equal to codedCIdx, the following applies:

resSamples[ x ][ y ] = res[ x ][ y ]						(8-907)

      - Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

resSamples[ x ][ y ] = cSign * res[ x ][ y ]					(8-908)

      - Otherwise, the following applies:

resSamples[ x ][ y ] = ( cSign * res[ x ][ y ] ) >> 1				(8-909)

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] AHG18 reference software in at https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM, branch VTM-6.0-Lossless.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).