| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0530 |
| --- | --- |

| Title: | Non-CE4: Alignment of luma and chroma weights calculation for TPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current TPM blending process, the weights assigned to a chroma sample and its collocated luma sample are different. This contribution proposes to align the chroma weights with that of the collocated luma samples used in TPM blending. In addition, the logic for weights derivation of luma and chroma components is unified as well. Therefore, there is no need to handle luma and chroma components individually. The proposed modification was tested for JVET YUV4:2:0, JCT-VC RExt YUV4:2:2, and JVET TGM YUV4:4:4. The overall BD-rate changes for {Y, Cb, Cr} are listed as follows.

| YUV4:2:0 | RA | 0.01% | -0.04% | -0.03% |
| --- | --- | --- | --- | --- |
|  | LDB | -0.02% | 0.10% | -0.11% |
| YUV4:2:2 | RA | -0.01% | -0.03% | -0.02% |
|  | LDB | -0.06% | 0.02% | -0.07% |
| YUV4:4:4 | RA | -0.01% | 0.04% | 0.00% |
|  | LDB | -0.01% | -0.08% | -0.03% |

# TPM weights calculation in VVC draft 6

In current VVC WD6 [1], the TPM blending weights for collocated luma and chroma samples are calculated individually without alignment. It turns out that the blending weights distribution of collocated luma and chroma samples in a triangle inter block are not aligned together, as shown in Figure 1(a). This doesn’t make sense especially for YUV4:4:4 or RGB4:4:4, where either channel of G, B, and R would probably be coded as the main channel, hence it unreasonable to assign different weights for different color channel especially for 4:4:4. Moreover, in WD6, additional logic would be necessary to calculate chroma weights in addition to luma weights, and only even weights are allowed for chroma component TPM blending, which may be not desirable.

- (b)

Figure 1. Illustration of TPM blending weights allocation of (a) WD6, and (b) proposed method.

# Proposed solution

In order to align chroma weights to luma weights in TPM blending, a unified luma and chroma weights calculation mechanism is proposed. Other than calculating the luma and chroma weights separately, in this contribution, a single equation is used so that the luma and chroma can share the same computing logic. Therefore, for collocated samples in different chroma formats such as 4:2:0, 4:2:2, and 4:4:4, the resulted TPM chroma weights always align with the collocated luma weights, as shown in Figure 1(b).

Regarding the benefit of the proposed method, firstly, making the chroma weights align with luma is theoretically more reasonable for an INTER coding tool (e.g., all other weighted prediction in VVC such as BCW and weighted prediction are designed with luma and chroma alignment), especially for 4:4:4 color format in which each color channel should be treated equally. Secondly, if the TPM weighting values are computed on the fly, a unified computing logic for both luma and chroma makes it more friendly for hardware implementation. Otherwise, if the TPM weighting values are not computed on the fly, the proposed method can reduce the storage size of weighting values since only luma weights need to be stored and chroma weights can be directly subsampled from luma.

# Simulation results

The proposed method was implemented on top of VTM-6.0 [2] and tested for JVET SDR CTC test set [3] and JCT-VC RExt 4:2:2 CTC test set [4] under JVET CTC conditions [3], and 4:4:4 TGM sequences under JVET CE8 test conditions [5].

Simulation results for JVET CTC test set, RExt 4:2:2 CTC test set and 4:4:4 TGM sequences reportedly are provided in Table 1, Table 2 and Table 3, respectively, for RA and LDB configurations.

Table 1: BD-Rate results for 4:2:0 in JVET SDR CTC test set

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.08% | 99% | 99% |
| Class A2 | 0.04% | -0.02% | 0.00% | 99% | 99% |
| Class B | 0.00% | -0.06% | -0.03% | 100% | 98% |
| Class C | 0.02% | -0.05% | -0.04% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | -0.03% | 100% | 99% |
| Class D | -0.03% | -0.08% | 0.06% | 101% | 99% |
| Class F | 0.03% | -0.03% | -0.05% | 100% | 101% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.07% | -0.35% | 101% | 99% |
| Class C | -0.04% | -0.03% | -0.16% | 99% | 96% |
| Class E | -0.03% | 0.32% | 0.36% | 100% | 96% |
| Overall | -0.02% | 0.10% | -0.11% | 100% | 97% |
| Class D | -0.02% | 0.26% | 0.05% | 100% | 98% |
| Class F | 0.03% | -0.18% | -0.05% | 100% | 97% |

Table 2: BD-Rate results for 4:2:2 in JCT-VC RExt CTC test set

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.04% | -0.03% | 0.04% |
| Kimono1_1920x1080_24_10bit_422.yuv | -0.02% | 0.01% | -0.01% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.00% | -0.01% | -0.06% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.04% | -0.09% | -0.06% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | -0.01% | 0.03% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.01% | 0.07% | -0.04% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.02% | -0.19% | -0.07% |
| Overall | -0.01% | -0.03% | -0.02% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.03% | 0.08% | -0.11% |
| Kimono1_1920x1080_24_10bit_422.yuv | -0.14% | 0.08% | 0.06% |
| EBUHorse_1920x1080_50_10bit_422.yuv | -0.10% | 0.08% | -0.32% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | -0.18% | -0.12% | -0.12% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | 0.07% | -0.01% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | -0.01% | -0.05% | 0.13% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.01% | -0.01% | -0.13% |
| Overall | -0.06% | 0.02% | -0.07% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

Table 3: BD-Rate results for 4:4:4 TGM sequences (--DualTree=1)

|  |  |  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| TGM 1080p | 0.00% |  | 0.10% |  | 0.08% | 100% | 100% |
| TGM 720p | -0.02% |  | -0.03% |  | -0.14% | 101% | 101% |
| Animation | 0.00% |  | 0.00% |  | 0.03% | 100% | 100% |
| Mixed content | 0.01% |  | 0.18% |  | 0.11% | 101% | 100% |
| Camera-Captured content | -0.06% |  | -0.06% |  | -0.08% | 100% | 99% |
| Overall | -0.01% |  | 0.04% |  | 0.00% | 100% | 100% |
|  |  |  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-6.0 |  |  |  |  |  |  |
|  | Y |  |  | U | V | EncT | DecT |
| TGM 1080p | 0.07% |  |  | 0.00% | 0.06% | 98% | 98% |
| TGM 720p | 0.01% |  |  | -0.24% | -0.09% | 98% | 98% |
| Animation | 0.02% |  |  | 0.06% | 0.03% | 97% | 98% |
| Mixed content | -0.16% |  |  | -0.17% | -0.09% | 101% | 99% |
| Camera-Captured content | -0.04% |  |  | -0.02% | -0.13% | 101% | 99% |
| Overall | -0.01% |  |  | -0.08% | -0.03% | 99% | 98% |

# Conclusions

In this contribution, a unification of luma and chroma weights calculation for triangle inter blocks is proposed. Same as other INTER coding tools such as BCW and weighted prediction, the proposed method makes the blending weight of chroma sample align to the weight of collocated luma sample, which is more reasonable especially for 4:4:4 chroma format. Moreover, by using the proposed method, luma and chroma can share the same computing logic for TPM weight calculation, which is more hardware friendly. It is recommended adopting the proposed method.

# Proposed spec changes

The proposed changes on top of VVC draft 6 [1] are described as below, with newly added parts and deleted parts highlighted.

### 8.5.7 Decoding process for triangle inter blocks

#### 8.5.7.1 General

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- ...

Outputs of this process are:

- ...

Let predSamplesLAL and predSamplesLBL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesLACb, predSamplesLBCb, predSamplesLACr and predSamplesLBCr be (cbWidth / SubWidthC)x(cbHeight / SubHeightC) arrays of predicted chroma sample values.

The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- ...

- The partition direction of merge triangle mode variable triangleDir is set equal to merge_triangle_split_dir[ xCb ][ yCb ].

- The prediction samples inside the current luma coding block, predSamplesL[ xL ][ yL ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 with the coding block width nCbW set equal to cbWidth, the coding block height nCbH set equal to cbHeight, the sample arrays predSamplesLAL and predSamplesLBL, and the variables triangleDir, and cIdx equal to 0 as inputs.

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause  8.5.7.2  with the luma coding block width nCbW set equal to cbWidth / SubWidthC, the luma coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACb and predSamplesLBCb, and the variables triangleDir, and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2  with the luma coding block width nCbW set equal to cbWidth / SubWidthC, the luma coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACr and predSamplesLBCr, and the variables triangleDir, and cIdx equal to 2 as inputs.

- The motion vector storing process for merge triangle mode specified in clause  8.5.7.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the partition direction triangleDir, the luma motion vectors mvA and mvB, the reference indices refIdxA and refIdxB, and the prediction list flags predListFlagA and predListFlagB as inputs.

#### 8.5.7.2	Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current luma coding block,

- two (nCbW/ SubWidthC)x(nCbH/ SubHeightC) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW/ SubWidthC)x(nCbH/ SubHeightC) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-841)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW/ SubWidthC − 1 and y = 0..nCbH/ SubHeightC − 1 are derived as follows:

–	The variables xIdx and yIdx are derived as follows:

xIdx = ( cIdx == 0 ) ? x : x * SubWidthC

yIdx = ( cIdx == 0 ) ? y : y * SubHeightC

–	The variable wIdx wValue specifying the weight of the prediction sample is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx wValue = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( xIdx / nCbR − yIdx  ) + 4 ) )	(8-842)
	: ( Clip3( 0, 8, ( xIdx  − yIdx  / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and (triangleDir is equal to 1), the following applies:

wIdx wValue = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − xIdx / nCbR − yIdx ) + 4 ) ) 	(8-843)
		( Clip3( 0, 8, ( nCbW − 1 − xIdx − yIdx / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-844)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-845)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-846)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-847)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

VTM-6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006-N1028, January 2014

X. Xu, Y.-H. Chao, Y.-C. Sun. J., Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, July 2019, Gothenburg, SE.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).