| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0539 |
| --- | --- |

| Title: | Non-CE5: A chroma deblocking clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu 8910 University Center Lane, San Diego, CA 92122, USA. | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

A flexible chroma QP table design was adopted at the 15th JVET meeting, which influences the chroma deblocking process. In the current draft, chroma QP table is applied after averaging luma QPs on both side of an edge. It is proposed to firstly apply the chroma QP table on each side and then perform averaging to avoid extreme cases. No performance changes are observed under current common test conditions.

# Problem statement

In VVC draft 6, chroma deblocking QP for an edge is derived by applying chroma QP table on the average of luma QPs on both sides. The corresponding text is as follows:

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable QpC is derived as follows:

qPi = Clip3( 0, 63, ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset )	(8-1132)

QpC = ChromaQpTable[ cIdx − 1 ][ qPi ]	(8-1133)

Before adoption of the flexible chroma table design, chroma QP mapping on the average of luma QP approximates the average of chroma QP mapping. However, because the chroma QP tables can be flexibly signalled, there might be large discrepancy between chroma QP mapping on the average of luma QP approximates the average of chroma QP mapping.

In some cases, the difference between the average of mapping and mapping of the average can be much large. For example, in the case shown in Figure 1, where a chroma QP table consists of two flat lines, the difference between the two methods can be as large as 37. In such a case, the current design can lead to a unstable situation: when QP on one side is decreased from 36 to 35, the QP for deblocking decision will be changed from 63 to -12, i.e. from a maximum deblocking to totally no deblocking.

Although in practice, maybe no one uses such an extreme chroma QP table by purpose, the above example demonstrates that the current design is not robust enough.

Figure 1. An examplar chroma QP table applied to chroma deblocking.

# Proposed clean-up

In principle, deblocking is based on the QP applied to the block. Thus, it is proposed to use the average of chroma mapped QPs, to align to the spirit of deblocking design. The other issue of the current design is that a PPS chroma QP offset should be added on the chroma QP, i.e. the output of a chroma QP table.

# Experimental results

The proposed change does not influence BD-rate performance under CTC because constant QPs are always used.

# Changes to the draft text

The changes, marked in grey, are based on JVET-O2001-vE.

8.8.3.6.3 	Decision process for chroma block edges

…

The variable QpC is derived as follows:

qPi = Clip3( 0, 63, ( ( QpQ + QpP + 1 )  >>  1 ) + cQpPicOffset )	(8-1132)

QpC = ChromaQpTable[ cIdx − 1 ][ qPi ]	(8-1133)

QpC = (( ChromaQpTable[ cIdx – 1 ][ QpQ ] + ChromaQpTable[ cIdx – 1 ][ QpP ] + 1 ) >> 1 ) +
     	                                                        cQpPicOffset	(8-1133)

The value of the variable β′ is determined as specified in Table 8-18 based on the quantization parameter Q derived as follows:

Q = Clip3( 0, 63, QpC + ( slice_beta_offset_div2  <<  1 ) )	(8-1134)

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).