| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0541 |
| --- | --- |

| Title: | Non-CE4: Context modeling for inter prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, hm.jang, naeri.park, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VVC, inter_pred_idc syntax which specifies inter prediction mode uses five context models. Especially, the 5th context model is being shared by the 4x16, 8x8, or 16x4 bi-predicted CUs and uni-predicted CUs. This contribution proposes two alternative methods to prevent from the 5th context model being shared between bi-prediction and uni-prediction cases. From experiment results, luma BD-rate changes of method 1 is reported as -0.01% and -0.02% for RA and LDB. luma BD-rate changes of method 2 is reported as -0.02% and -0.03% for RA and LDB.

Introduction

To specify inter prediction mode e.g. PRED_L0, PRED_L1, and PRED_BI, inter_pred_idc syntax is signaled up to 2 bits with five context models as shown in Table 1. The first bin determines whether the coding block is coded by bi-prediction or uni-prediction. If it is determined to be a uni-prediction by the first bin, the second bin is signaled to indicate whether the coding block is L0 uni-prediction or L1. For the second bin associated uni-prediction, a value of context increment is always equal to 4. Especially, if block size is equal to 4x8 or 8x4 which are only allowed uni-prediction, a value of context increment is also equal to 4.

Table 1. Context model for inter_pred_idc syntax

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 12 ? 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ) : 4 | 4 | na | na | na | na |

Context model of the first bin for other blocks except 4x8 or 8x4 blocks is determined by following formula.

ctxInc = 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 )                   (1)

Depending on block size, Table 2 shows values of context increment for the first and second bins. According to Table 2, a value of context increment of the first bin for 4x16, 8x8, or 16x4 blocks is equal to 4. It may be an inadequate because the value 4 is assigned to uni-prediction in terms of a design of context model.

Table 2. Values of context increment for the first and second bins depending on block size

|  |  | Width |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4 | 8 | 16 | 32 | 64 | 128 |
| Height | 4 | N/A | x, 4 | 4, 4 | 3, 4 | 3, 4 | N/A |
|  | 8 | x, 4 | 4, 4 | 3, 4 | 3, 4 | 2, 4 | N/A |
|  | 16 | 4, 4 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | N/A |
|  | 32 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | N/A |
|  | 64 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | 1, 4 | 0, 4 |
|  | 128 | N/A | N/A | N/A | N/A | 0, 4 | 0, 4 |

Proposed method

In this contribution, there are two options to modify an assign of context model for the inter_pred_idc syntax.

Method 1

For the first bin for 4x16, 8x8, or 16x4 blocks, a value of context increment replaces 4 with 3 by clipping operation as shown in Table 3.

Table 3. Method 1: context model for inter_pred_idc syntax

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 12 ? Clip3(0, 3, 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 )) : 4 | 4 | na | na | na | na |

This formula can be expressed by min operation as follows,

ctxInc = min(3, 7 − ( ( 1 + Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 ))         (2)

Depending on block size, Table 4 shows values of context increment for the first and second bins. According to Table 4, a value of context increment of the first bin for 4x16, 8x8, or 16x4 blocks is changed to 3.

Table 4. Method 1: values of context increment for the first and second bins depending on block size

|  |  | Width |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4 | 8 | 16 | 32 | 64 | 128 |
| Height | 4 | N/A | x, 4 | 3, 4 | 3, 4 | 3, 4 | N/A |
|  | 8 | x, 4 | 3, 4 | 3, 4 | 3, 4 | 2, 4 | N/A |
|  | 16 | 3, 4 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | N/A |
|  | 32 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | N/A |
|  | 64 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | 1, 4 | 0, 4 |
|  | 128 | N/A | N/A | N/A | N/A | 0, 4 | 0, 4 |

Method 2

In the method 2, values of context increment for inter_pred_idc syntax are rearranged depending on block sizes as shown in Table 5. For the first bin for 4x16, 8x8, or 16x4 blocks, a value of context increment replaces 4 with 3 same as method 1.

Table 5. Method 2: context model for inter_pred_idc syntax

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| inter_pred_idc[ x0 ][ y0 ] | ( cbWidth + cbHeight ) != 12 ? max(0, 6 − ( ( Log2( cbWidth ) + Log2( cbHeight ) ) >> 1 )) : 4 | 4 | na | na | na | na |

Table 6 shows values of context increment for the first and second bins. According to Table 6, a value of context increment of the first bin for 4x16, 8x8, or 16x4 blocks is changed to 3.

Table 6. Method 1: values of context increment for the first and second bins depending on block size

|  |  | Width |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 4 | 8 | 16 | 32 | 64 | 128 |
| Height | 4 | N/A | x, 4 | 3, 4 | 3, 4 | 2, 4 | N/A |
|  | 8 | x, 4 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | N/A |
|  | 16 | 3, 4 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | N/A |
|  | 32 | 3, 4 | 2, 4 | 2, 4 | 1, 4 | 1, 4 | N/A |
|  | 64 | 2, 4 | 2, 4 | 1, 4 | 1, 4 | 0, 4 | 0, 4 |
|  | 128 | N/A | N/A | N/A | N/A | 0, 4 | 0, 4 |

Experimental results

The proposed methods have been implemented on VTM-6.0 and tested under common test conditions. Table 7 and Table 8 shows coding performance of method 1 on RA and LDB configurations. Table 9 and Table 10 shows coding performance of method 2 on RA and LDB configurations.

Table 7. Performance of method 1 in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.10% | 0.00% | 102% | 102% |
| Class A2 | 0.02% | -0.01% | 0.00% | 103% | 102% |
| Class B | -0.01% | 0.07% | -0.07% | 100% | 101% |
| Class C | -0.04% | 0.06% | -0.03% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.02% | -0.03% | 100% | 100% |
| Class D | -0.07% | -0.07% | -0.10% | 96% | 93% |
| Class F | -0.02% | -0.04% | -0.01% | 98% | 99% |

Table 8. Performance of method 1 in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.07% | -0.44% | 99% | 99% |
| Class C | -0.07% | -0.03% | 0.01% | 99% | 101% |
| Class E | 0.02% | 0.37% | 0.35% | 99% | 101% |
| Overall | -0.02% | 0.05% | -0.09% | 99% | 100% |
| Class D | -0.10% | 0.64% | -0.03% | 99% | 98% |
| Class F | 0.01% | -0.15% | 1.15% | 99% | 101% |

Table 9. Performance of method 2 in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.04% | 0.03% | 101% | 102% |
| Class B | -0.01% | 0.07% | 0.01% | 100% | 101% |
| Class C | -0.04% | -0.03% | -0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.01% | 0.01% | 100% | 101% |
| Class D | -0.06% | -0.09% | 0.00% | 100% | 102% |
| Class F | -0.03% | -0.03% | -0.01% | 100% | 101% |

Table 10. Performance of method 2 in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.03% | -0.24% | 102% | 100% |
| Class C | -0.07% | -0.05% | -0.12% | 100% | 101% |
| Class E | -0.01% | -0.07% | -0.13% | 101% | 101% |
| Overall | -0.03% | -0.02% | -0.17% | 101% | 101% |
| Class D | -0.07% | -0.08% | 0.55% | 101% | 99% |
| Class F | -0.08% | 0.39% | 0.69% | 101% | 99% |

Conclusion

In this contribution, two context modeling methods are proposed to avoid using 5th context model for the first bin relevant to bi-prediction. In addition, slight coding gain is observed. It is recommended to adopt the proposed method in the next specification.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).