| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0549-v1 |
| --- | --- |

| Title: | Non-CE3 : Interpolation filter selection for chroma intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution suggests unified interpolation filter usage between luma and chroma intra prediction. Specifically the 4-tap DCT-IF interpolation filter, which has been used for luma intra prediction, is applied to directional chroma intra modes. The proposed method provides the coding performance 0.00% / -0.16% / -0.11% and 0.00% / -0.20% / -0/19 % BD-rate in AI and RA configuration, respectively. Encoding and decoding run-times are unaffected.

Proposed method

In VTM-6.0, the linear interpolation filter is applied for directional chroma intra modes. Whereas the decision between the 4-tap Gaussian and the 4-tap DCT-IF interpolation filter is determined based on the sub-partition size and the intra mode for directional luma intra modes. This contribution proposes to apply the 4-tap DCT-IF interpolation filter as used in the luma intra prediction for the chroma intra prediction. Since this contribution has not changed the interpolation filter selection process, the 4-tap DCT-IF interpolation filter is always used for prediction of directional chroma intra modes.

Experimental result

The proposed method has been implemented using VTM-6.0 and under the common test conditions [1].

Table 1. Coding performance of the proposed method (AI)

Table 2. Coding performance of the proposed method (RA)

Conclusion

In conclusion, the proposed method unifies the interpolation filter process in luma and chroma intra prediction. The proposed method provides the coding performance 0.00% / -0.16% / -0/11% and 0.00% / -0.20% / -0.19% BD-rate in AI and RA configuration, respectively without increasing the complexity of encoding and decoding. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).