| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0551 |
| --- | --- |

| Title: | AHG16/Non-CE5: On ALF boundary padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, there are two padding methods for boundary treatment in adaptive loop filter (ALF), i.e. repetitive padding and mirrored padding. It is desirable to further simplify and unify the padding methods for ALF boundaries. In this contribution, different unification methods are tested.

# Introduction

In VVC draft 6, there are two ways to handle unavailable pixels beyond boundaries for adaptive loop filter (ALF).

- Repetitive padding: for picture boundaries and 360 virtual boundaries that are not on CTU boundaries, pixels are padded towards outside of the boundaries. Then after padding, Laplacian calculation and filtering process are performed without changes.

- Mirrored padding: for the boundaries (virtual boundary for line buffer reduction, sub-picture boundaries and 360 virtual boundaries on CTU boundaries), the samples located beyond the line buffer boundary are excluded in ALF process, where Laplacian is re-scaled and truncated symmetrical filtering is applied.

In JVET-N0654, it was proposed to unify the padding methods, such that only one padding method is applied to all type of boundaries.

# Proposed method

Three possible unification and simplification by reusing existed methods are applied:

- apply mirrored padding to 360 virtual boundaries and picture boundaries

- apply repetitive padding for all boundaries excluding virtual boundaries for ALF line buffer reduction

- apply repetitive padding for all boundaries

# Experimental results

## Apply mirrored padding to picture boundaries

The experiment is performed in VTM-6.0 with common test conditions defined in JVET-N1010. ALF SIMD was turned off in the test.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.02% | 123% | 183% |
| Class A2 | 0.00% | 0.01% | 0.01% | 124% | 169% |
| Class B | 0.00% | 0.00% | -0.01% | 101% | 133% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 123% |
| Class E | 0.00% | 0.01% | 0.01% | 100% | 138% |
| Overall | 0.00% | 0.01% | 0.01% | 108% | 144% |
| Class D | 0.00% | 0.02% | 0.02% | 100% | 121% |
| Class F | 0.00% | 0.01% | 0.00% | 102% | 132% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | 0.03% | 128% | 186% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.08% | 0.05% | 101% | 141% |
| Class C | 0.00% | 0.06% | 0.00% | 100% | 133% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.09% | 0.03% | 98% | 126% |
| Class F | -0.03% | 0.01% | 0.02% | 101% | 126% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | 0.03% | -0.03% | 101% | 138% |
| Class E | 0.00% | 0.63% | 0.12% | 100% | 132% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.07% | 0.49% | 101% | 133% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Performance when disabling loop filters across slice and tile boundaries

The experiment is performed on top of VTM-6.0 with common test conditions defined in JVET-N1010. ALF SIMD is turned off in the test and additional slice configuration is set as below for both anchor and the test.

SliceMode                : 3

SliceArgument            : 1

TileUniformSpacing                  : 0                # 0: the column boundaries are indicated by TileColumnWidth array, the row boundaries are indicated by TileRowHeight array

# 1: the column and row boundaries are distributed uniformly

NumTileColumnsMinus1                : 1                # Number of tile columns in a picture minus 1

TileColumnWidthArray                : 2                # Array containing tile column width values in units of CTU (from left to right in picture)

NumTileRowsMinus1                   : 1                # Number of tile rows in a picture minus 1

TileRowHeightArray                  : 1                # Array containing tile row height values in units of CTU (from top to bottom in picture)

RectSliceFlag                       : 1

NumRectSlicesInPicMinus1            : 3

RectSlicesBoundaryArray             : 0 0 1 1 2 2 3 3

Test: VTM-6.0 + https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/920

It uses the same configuration as the anchor with the disabling loop filtering across slice and tile boundaries.

LFCrossSliceBoundaryFlag            : 0

LFCrossTileBoundaryFlag             : 0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.08% | 0.08% | 101% | 143% |
| Class A2 | 0.04% | 0.04% | 0.04% | 101% | 137% |
| Class B | 0.03% | 0.07% | 0.07% | 101% | 135% |
| Class C | 0.05% | 0.08% | 0.11% | 100% | 124% |
| Class E | 0.05% | 0.13% | 0.14% | 100% | 143% |
| Overall | 0.04% | 0.08% | 0.09% | 101% | 135% |
| Class D | 0.06% | 0.17% | 0.23% | 100% | 120% |
| Class F | 0.03% | 0.07% | 0.08% | 101% | 126% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.03% | 101% | 148% |
| Class A2 | 0.05% | 0.07% | 0.09% | 101% | 143% |
| Class B | 0.04% | 0.01% | 0.17% | 100% | 137% |
| Class C | 0.07% | 0.14% | 0.05% | 100% | 131% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.07% | 0.10% | 100% | 138% |
| Class D | 0.05% | 0.03% | 0.06% | 100% | 124% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.02% | 0.01% | 0.09% | 101% | 136% |
| Class E | 0.00% | 0.21% | 0.05% | 102% | 134% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.27% | -0.01% | 101% | 131% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Apply repetitive padding to all boundaries except virtual boundaries for ALF line buffer reduction

The experiment is performed on top of VTM-6.0 with common test conditions defined in JVET-N1010. ALF SIMD is turned off for anchor. Additional slice configuration is set as below for the anchor and the test, which is the same as for the test in the section 3.2.

SliceMode                : 3

SliceArgument            : 1

TileUniformSpacing                  : 0                # 0: the column boundaries are indicated by TileColumnWidth array, the row boundaries are indicated by TileRowHeight array

# 1: the column and row boundaries are distributed uniformly

NumTileColumnsMinus1                : 1                # Number of tile columns in a picture minus 1

TileColumnWidthArray                : 2                # Array containing tile column width values in units of CTU (from left to right in picture)

NumTileRowsMinus1                   : 1                # Number of tile rows in a picture minus 1

TileRowHeightArray                  : 1                # Array containing tile row height values in units of CTU (from top to bottom in picture)

RectSliceFlag                       : 1

NumRectSlicesInPicMinus1            : 3

RectSlicesBoundaryArray             : 0 0 1 1 2 2 3 3

LFCrossSliceBoundaryFlag            : 0

LFCrossTileBoundaryFlag             : 0

Anchor: VTM-6.0 + https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/920

Test: VTM-6.0 + https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/920 + applying repetitive padding to all boundaries except virtual boundaries for ALF line buffer reduction

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | 0.03% | 0.04% | 100% | 76% |
| Class C | 0.02% | 0.07% | 0.09% | 101% | 81% |
| Class E | 0.04% | 0.04% | 0.03% | 101% | 71% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.09% | 0.12% | 101% | 88% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.04% | 0.00% | 100% | 73% |
| Class C | -0.01% | -0.02% | -0.06% | 100% | 77% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.14% | 0.05% | 99% | 78% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | 0.07% | -0.16% | -0.03% | 99% | 75% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | -0.48% | 0.37% | 100% | 78% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).