| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0557 |
| --- | --- |

| Title: | CE5-related: Multiplication removal for cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with an exponential-Golomb code. To reduce the complexity of multiplication, in this proposal, multiplications are replaced by bit shifting. Under common test conditions, compared to CE5-2.1, similar overall coding efficiency and running time are observed.

# Introduction

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with a third order exponential-Golomb code. However, this adds the complexity, especially in decoder, by introducing additional 12 8-bit multiplications, 1 9-bit multiplication and 1 10-bit multiplication per chroma pixel.

# Proposed method

To simplify the computation overhead in JVET-P0080, in this proposal, the multiplications are replaced by bit shifting.

## Method 1

CC-ALF filter coefficients are restricted to have only the following values: {-128, -64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64, 128}, which are signalled as {-8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8} respectively before binarization in order to reduce the signalling overhead. Third order exponential-Golomb codes are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

## Method 2

Method 1, where EG3 is replaced with EG1.

## Method 3

Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

## Method 4

Filter coefficients are restricted to have values: {-4, -2, -1, 0, 1, 2, 4}, which are signalled as {-3, -2, -1, 0, 1, 2, 3} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 2 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

# Experimental results

The proposed methods are implemented on top of the in-loop version of CC-ALF in CE5-2.1. The simulations were conducted under JVET common test conditions [1].

## Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.14% | 0.19% | 100% | 100% |
| Class A2 | -0.02% | 0.37% | 0.26% | 99% | 99% |
| Class B | 0.00% | 0.22% | 0.15% | 99% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 101% | 99% |
| Class E | 0.00% | 0.18% | 0.10% | 99% | 98% |
| Overall | 0.00% | 0.19% | 0.14% | 100% | 99% |
| Class D | 0.00% | 0.04% | 0.01% | 100% | 99% |
| Class F | 0.00% | 0.44% | 0.18% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.40% | 0.04% | 99% | 99% |
| Class A2 | 0.01% | 0.56% | 0.46% | 101% | 100% |
| Class B | 0.00% | 0.24% | 0.18% | 100% | 100% |
| Class C | 0.03% | 0.13% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.31% | 0.18% | 100% | 100% |
| Class D | 0.00% | 0.10% | 0.03% | 100% | 99% |
| Class F | 0.02% | 0.23% | 0.09% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.30% | 0.22% | 100% | 99% |
| Class C | 0.02% | 0.21% | -0.20% | 100% | 101% |
| Class E | -0.13% | -0.85% | 0.29% | 99% | 100% |
| Overall | -0.02% | -0.02% | 0.10% | 100% | 100% |
| Class D | -0.01% | 0.15% | 0.26% | 100% | 101% |
| Class F | 0.15% | 0.65% | -0.22% | 100% | 99% |

## Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.13% | 0.19% | 101% | 100% |
| Class A2 | -0.02% | 0.37% | 0.26% | 99% | 99% |
| Class B | -0.01% | 0.22% | 0.15% | 99% | 100% |
| Class C | -0.02% | 0.02% | -0.04% | 100% | 100% |
| Class E | -0.02% | 0.16% | 0.02% | 99% | 98% |
| Overall | -0.01% | 0.18% | 0.11% | 100% | 99% |
| Class D | 0.00% | -0.29% | -0.28% | 99% | 100% |
| Class F | -0.01% | 0.41% | 0.16% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.35% | 0.08% | 100% | 101% |
| Class A2 | 0.00% | 0.55% | 0.45% | 101% | 101% |
| Class B | 0.01% | 0.06% | -0.02% | 100% | 99% |
| Class C | 0.03% | -0.10% | -0.13% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.17% | 0.06% | 99% | 100% |
| Class D | 0.00% | -0.62% | -0.37% | 101% | 101% |
| Class F | 0.02% | 0.14% | 0.03% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.27% | -0.02% | 100% | 100% |
| Class C | 0.03% | -0.30% | -0.35% | 100% | 100% |
| Class E | -0.16% | -0.72% | 0.01% | 99% | 101% |
| Overall | -0.02% | -0.17% | -0.13% | 100% | 100% |
| Class D | 0.06% | -0.49% | 0.38% | 99% | 101% |
| Class F | 0.16% | 0.30% | -0.49% | 100% | 100% |

## Method 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.12% | 0.18% | 101% | 99% |
| Class A2 | -0.02% | 0.37% | 0.26% | 98% | 100% |
| Class B | -0.01% | 0.22% | 0.14% | 100% | 100% |
| Class C | -0.01% | 0.02% | -0.01% | 99% | 99% |
| Class E | -0.02% | 0.16% | -0.01% | 99% | 99% |
| Overall | -0.01% | 0.17% | 0.11% | 99% | 100% |
| Class D | 0.00% | -0.20% | -0.19% | 101% | 100% |
| Class F | -0.01% | 0.41% | 0.16% | 99% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.34% | -0.03% | 101% | 100% |
| Class A2 | 0.01% | 0.52% | 0.44% | 101% | 101% |
| Class B | 0.01% | 0.04% | -0.01% | 101% | 100% |
| Class C | 0.04% | 0.04% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.20% | 0.06% | 101% | 100% |
| Class D | -0.02% | -0.42% | -0.19% | 100% | 100% |
| Class F | 0.02% | 0.14% | 0.05% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | 0.46% | 100% | 98% |
| Class C | 0.02% | -0.13% | -0.16% | 101% | 100% |
| Class E | -0.14% | -0.68% | -0.30% | 101% | 101% |
| Overall | -0.03% | -0.19% | 0.07% | 101% | 100% |
| Class D | 0.03% | -0.22% | -0.26% | 100% | 100% |
| Class F | 0.13% | -0.01% | -0.08% | 100% | 100% |

## Method 4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.65% | 0.67% | 101% | 100% |
| Class A2 | -0.05% | 2.10% | 3.06% | 101% | 100% |
| Class B | -0.01% | 0.63% | 0.39% | 100% | 101% |
| Class C | -0.03% | 0.00% | -0.06% | 100% | 100% |
| Class E | -0.03% | 0.13% | -0.06% | 101% | 99% |
| Overall | -0.03% | 0.66% | 0.71% | 100% | 100% |
| Class D | 0.01% | -0.45% | -0.37% | 100% | 99% |
| Class F | -0.02% | 0.45% | 0.17% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 1.48% | 0.92% | 100% | 100% |
| Class A2 | -0.02% | 2.47% | 2.99% | 100% | 101% |
| Class B | 0.01% | 0.59% | 0.09% | 100% | 99% |
| Class C | 0.03% | -0.17% | -0.19% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.94% | 0.76% | 100% | 100% |
| Class D | 0.02% | -0.80% | -0.42% | 101% | 100% |
| Class F | 0.00% | 0.38% | 0.20% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 1.23% | 0.35% | 101% | 101% |
| Class C | 0.03% | -0.31% | -0.30% | 100% | 100% |
| Class E | -0.24% | -0.70% | -0.39% | 100% | 101% |
| Overall | -0.03% | 0.23% | -0.05% | 100% | 101% |
| Class D | 0.05% | -0.36% | -0.55% | 99% | 100% |
| Class F | 0.09% | 0.19% | -0.26% | 99% | 101% |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).