| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0558 |
| --- | --- |

| Title: | CE5-related: Reduced filter shape for cross component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. Each filter has 14 filter coefficients and 18 taps. Every coefficient has an 8-bit dynamic range and is signalled with a third order exponential-Golomb code. To reduce the complexity of multiplication, in this proposal, a reduced filter shape with 13 coefficients and 13 taps is proposed. Under common test conditions, compared to CE5-2.1, similar overall coding efficiency and running time are observed.

# Introduction

In JVET-P0080, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma sample values. Each filter has 14 filter coefficients and 18 taps as the figure below. However, this increases the complexity, especially in decoder, by introducing additional 12 8-bit multipliers, 1 9-bit multiplier and 1 10-bit multiplier per chroma pixel.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f4 |
| f4 | f8 | f9 | f10 | f4 |
|  | f11 | f12 | f13 |  |
|  |  | f0 |  |  |

# Proposed method

To simplify the computation overhead in JVET-P0080, a reduced symmetrical filter shape with 13 coefficients and 13 taps is proposed.

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

# Experimental results

The proposed methods are implemented on top of the in-loop version of CC-ALF in CE5-2.1. The simulations were conducted under JVET common test conditions [1].

## Proposed filter shape

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.11% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.17% | -0.11% | 100% | 100% |
| Class B | 0.00% | -0.44% | -0.35% | 100% | 100% |
| Class C | -0.01% | -0.14% | -0.11% | 100% | 100% |
| Class E | 0.00% | -0.92% | -0.79% | 100% | 100% |
| Overall | 0.00% | -0.26% | -0.27% | 100% | 100% |
| Class D | 0.00% | -0.63% | -0.03% | 100% | 100% |
| Class F | -0.01% | -0.57% | -0.54% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.26% | 0.08% | 100% | 99% |
| Class A2 | 0.00% | 0.33% | -0.18% | 99% | 100% |
| Class B | -0.02% | -0.16% | -0.35% | 100% | 100% |
| Class C | 0.00% | 0.24% | 0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.13% | -0.10% | 100% | 100% |
| Class D | -0.03% | -0.99% | -0.49% | 100% | 100% |
| Class F | 0.01% | -0.56% | -0.46% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.79% | 0.40% | 100% | 100% |
| Class C | -0.01% | 0.61% | 0.64% | 100% | 100% |
| Class E | -0.08% | -1.17% | -0.62% | 100% | 100% |
| Overall | -0.02% | 0.24% | 0.23% | 100% | 100% |
| Class D | -0.02% | 0.01% | 0.85% | 99% | 100% |
| Class F | 0.11% | 0.36% | -0.15% | 101% | 99% |

## Proposed filter shape + 6-bit dynamic range + fixed length coding

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.13% | 0.01% | 99% | 99% |
| Class A2 | -0.01% | 0.50% | -0.04% | 99% | 100% |
| Class B | 0.00% | -0.44% | -0.33% | 99% | 100% |
| Class C | 0.01% | -0.10% | -0.05% | 100% | 100% |
| Class E | 0.02% | -0.90% | -0.75% | 101% | 99% |
| Overall | 0.00% | -0.19% | -0.23% | 100% | 100% |
| Class D | -0.01% | -0.41% | 0.19% | 101% | 99% |
| Class F | 0.00% | -0.54% | -0.52% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.34% | 0.14% | 100% | 99% |
| Class A2 | -0.02% | 0.63% | -0.03% | 100% | 99% |
| Class B | -0.03% | 0.01% | -0.18% | 101% | 101% |
| Class C | 0.00% | 0.47% | 0.22% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.32% | 0.02% | 100% | 100% |
| Class D | -0.05% | -0.29% | -0.26% | 99% | 99% |
| Class F | 0.01% | -0.50% | -0.38% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.03% | 0.69% | 0.50% | 101% | 101% |
| Class E | -0.13% | -1.11% | -0.68% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | 0.84% | 0.66% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | 100% | 101% |

## Proposed filter shape + 5-bit dynamic range + fixed length coding

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.14% | 0.01% | 99% | 101% |
| Class A2 | -0.02% | 1.01% | 0.30% | 101% | 99% |
| Class B | 0.00% | -0.43% | -0.35% | 101% | 100% |
| Class C | 0.00% | -0.13% | -0.10% | 100% | 100% |
| Class E | 0.00% | -0.92% | -0.79% | 98% | 101% |
| Overall | -0.01% | -0.11% | -0.20% | 100% | 100% |
| Class D | 0.00% | -0.57% | 0.02% | 101% | 100% |
| Class F | 0.00% | -0.57% | -0.54% | 99% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.30% | 0.10% | 101% | 100% |
| Class A2 | -0.01% | 1.07% | 0.49% | 100% | 99% |
| Class B | -0.02% | -0.15% | -0.36% | 100% | 101% |
| Class C | -0.01% | 0.32% | 0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.31% | 0.03% | 100% | 100% |
| Class D | -0.03% | -0.88% | -0.35% | 98% | 102% |
| Class F | 0.00% | -0.53% | -0.42% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.83% | 0.30% | 99% | 98% |
| Class C | 0.03% | 0.52% | 0.45% | 100% | 101% |
| Class E | -0.14% | -1.78% | -0.80% | 100% | 98% |
| Overall | -0.02% | 0.08% | 0.07% | 100% | 99% |
| Class D | 0.02% | 0.49% | 0.41% | 99% | 100% |
| Class F | 0.10% | -0.16% | -0.27% | 99% | 100% |

## Proposed filter shape + Method 2 in JVET-P0557

Filter coefficients can assume only the values: {-128, -64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64, 128}. The allowed values are mapped to {-8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8} respectively before binarization in order to reduce the signalling overhead. First order exponential-Golomb codes are used to signal the absolute values of mapped coefficients. Another sign bit is signalled for non-zero coefficients.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | 0.32% | 99% | 100% |
| Class A2 | -0.01% | 0.60% | 0.24% | 99% | 99% |
| Class B | 0.00% | -0.19% | -0.04% | 100% | 100% |
| Class C | -0.02% | -0.07% | -0.07% | 100% | 101% |
| Class E | -0.02% | -0.57% | -0.66% | 99% | 100% |
| Overall | -0.01% | -0.03% | -0.04% | 100% | 100% |
| Class D | -0.01% | -0.82% | -0.22% | 100% | 101% |
| Class F | -0.02% | -0.32% | -0.31% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.50% | 0.17% | 101% | 99% |
| Class A2 | 0.01% | 0.77% | 0.34% | 99% | 100% |
| Class B | 0.02% | 0.18% | -0.13% | 101% | 101% |
| Class C | 0.03% | 0.17% | 0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.36% | 0.08% | 100% | 100% |
| Class D | 0.01% | -1.41% | -0.56% | 99% | 101% |
| Class F | 0.00% | -0.48% | -0.40% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.04% | 0.20% | 99% | 101% |
| Class C | 0.04% | 0.76% | 0.49% | 101% | 99% |
| Class E | -0.08% | -1.54% | -0.62% | 100% | 100% |
| Overall | 0.01% | 0.30% | 0.09% | 100% | 100% |
| Class D | 0.04% | -0.09% | 0.10% | 100% | 100% |
| Class F | 0.08% | 0.54% | 0.42% | 101% | 100% |

## Proposed filter shape + Method 3 in JVET-P0557

Filter coefficients can assume only the values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. The allowed values are mapped to {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes are used to signal the absolute values of mapped coefficients. Another sign bit is signalled for non-zero coefficients.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.22% | 0.30% | 100% | 101% |
| Class A2 | -0.02% | 0.60% | 0.23% | 102% | 100% |
| Class B | 0.00% | -0.19% | -0.05% | 100% | 101% |
| Class C | -0.02% | -0.08% | -0.06% | 99% | 101% |
| Class E | -0.02% | -0.58% | -0.67% | 100% | 100% |
| Overall | -0.01% | -0.03% | -0.05% | 100% | 101% |
| Class D | 0.00% | -0.76% | -0.18% | 99% | 101% |
| Class F | -0.01% | -0.30% | -0.32% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.48% | 0.14% | 100% | 100% |
| Class A2 | 0.01% | 0.79% | 0.27% | 101% | 100% |
| Class B | 0.02% | 0.14% | -0.12% | 100% | 101% |
| Class C | 0.00% | 0.20% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.35% | 0.05% | 100% | 100% |
| Class D | -0.01% | -1.17% | -0.58% | 100% | 99% |
| Class F | 0.00% | -0.46% | -0.38% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0-CE5.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.90% | 0.70% | 101% | 99% |
| Class C | 0.03% | 0.67% | 0.60% | 99% | 99% |
| Class E | -0.07% | -2.12% | -0.92% | 100% | 100% |
| Overall | 0.00% | 0.07% | 0.26% | 100% | 99% |
| Class D | -0.03% | -0.03% | 0.00% | 101% | 100% |
| Class F | 0.12% | 0.43% | 0.12% | 101% | 99% |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).