| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0559 |
| --- | --- |

| Title: | AHG18: Rice parameter derivation for coefficient level coding in lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Yung-Hsuan Chao, Muhammed Coban, | Tel: Email: | martak@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a rice parameter derivation method for coding abs_remainder in lossless coding. The method is implemented on top of method 3 in JVET-P0082, where transform residual coding is used for lossless coding (transQuanBypass = 1) and BDPCM is enabled for lossless coding. The results compared to AHG18 lossless coding show:

xxx% AI, xxx% RA, xxx% LB

# Proposed Method

In this proposal, for coding the non-binary syntax element abs_remainder [k] in transform residual coding, the rice parameter is derived based on the base level and sum of absolute levels of neighboring five transform coefficients in local template as follows:

Define noPos as the number of coefficients available in the local template and sum as sum of absolute levels of neighboring five transform coefficients in local template, the variable sumAbs is derived as follows:

sumAbs = (noPos = = 0 || noPos = = 5)? sum: (5*sum / noPosNorm[noPos-1])

where noPosNorm is given by noPosNorm[4] = {1, 2, 2, 4}

The rice parameter RicePara is derived as:

RicePara = (Idx = = 33)? 5: ( (Idx = = 32)? 4: RiceParTable[ Idx ])

If baseLevel = 0:

Idx = (sumAbs > 256)? 33: ( (sumAbs > 128)? 32: max(min( 31, sumAbs ), 0)

Otherwise (baseLevel = 4),

Idx = (sumAbs > 256)? 32: max(min( 31, sumAbs – 5 * baseLevel), 0)

, where the rice table RiceParTable[] is the same as defined in VTM6.0.

The method is implemented on top of method 3 in JVET-P0082, where transform residual coding is used for lossless coding (transQuanBypass = 1) and BDPCM is enabled for lossless coding.

# Simulation Results

In the simulation, the anchor is AHG18 VTM-6.0 Lossless branch. The following settings are used to run both the anchor and test: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. QP value was set as 0. Additionally, BDPCM is set to be 1 for the test. The results are shown in Table 1:

Table 1: Results of the proposed rice parameter derivation for lossless coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | Proposed |  | VTM6 | Proposed |  | VTM6 | Proposed |  |
| Class A1 | 2.2 | 0.0 |  | 2.3 | 0.0 |  |  |  |  |
| Class A2 | 1.7 | 0.0 |  | 1.9 | 0.0 |  |  |  |  |
| Class B | 2.1 | 2.2 |  | 2.3 | 0.0 |  | 2.3 | 0.0 |  |
| Class C | 2.1 | 0.0 |  | 2.6 | 0.0 |  | 2.6 | 0.0 |  |
| Class D | 1.9 | 2.0 | -3.02% | 2.8 | 0.0 |  | 2.7 | 0.0 |  |
| Class E | 2.9 | 3.0 | -1.70% |  |  |  | 3.3 | 0.0 |  |
| Class F | 5.5 | 11.5 |  | 39.4 | 0.0 |  | 60.7 | 0.0 |  |
| TGM | 12.3 | 12.7 | -3.19% | 112.6 | 148.9 |  | 142.4 | 188.9 |  |
| Overall | 2.2 | 1.0 |  | 2.3 | 0.0 |  | 2.6 | 0.0 |  |
| Enc Time[%] | #NUM! |  |  | 19% |  |  | 28% |  |  |
| Dec Time[%] | #NUM! |  |  | 36% |  |  | 41% |  |  |

# Reference

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)