| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0559 |
| --- | --- |

| Title: | AHG18: Rice parameter derivation for coefficient level coding in lossless |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Yung-Hsuan Chao, Muhammed Coban, | Tel: Email: | martak@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes new rice parameter derivation methods for coding abs_remainder in transform residual coding and transform skip residual coding in lossless coding. In the first method, the method is implemented on top of method 3 in JVET-P0082, where transform residual coding and BDPCM are  used for lossless coding (transQuanBypass = 1) . In the second method, the methods are implemented on top of method 1 in JVET-P0485, where both transform skip and transform residual coding are used for lossless coding and the new Rice-Golomb coding for transform skip proposed in CE7-1.2d-alternative is included. The results compared to AHG18 lossless coding show:

Method 1:

-1.79% AI, xxx% RA, xxx% LB

Method 2:

xxx% AI, xxx% RA, xxx% LB

# Proposed Method

## Method 1

In this method, for coding the non-binary syntax element abs_remainder [k] in transform residual coding, the rice parameter is derived based on the base level and sum of absolute levels of neighboring five transform coefficients in local template as follows:

Define noPos as the number of coefficients available in the local template and sum as sum of absolute levels of neighboring five transform coefficients in local template, the variable sumAbs is derived as follows:

sumAbs = (noPos = = 0 || noPos = = 5)? sum: (5*sum / noPosNorm[noPos-1])

where noPosNorm is given by noPosNorm[4] = {1, 2, 2, 4}

The rice parameter RicePara is derived as:

RicePara = (Idx = = 33)? 5: ( (Idx = = 32)? 4: RiceParTable[ Idx ])

If baseLevel = 0:

Idx = (sumAbs > 256)? 33: ( (sumAbs > 128)? 32: max(min( 31, sumAbs ), 0)

Otherwise (baseLevel = 4),

Idx = (sumAbs > 256)? 32: max(min( 31, sumAbs – 5 * baseLevel), 0)

, where the rice table RiceParTable[] is the same as defined in VTM6.0.

The method is implemented on top of method 3 in JVET-P0082, where transform residual coding is used for lossless coding (transQuanBypass = 1) and BDPCM is enabled for lossless coding.

## Method 2

In this method, the proposed rice golomb derivations are implemented on top of [2]. In [2], both transform residual coding and transform skip residual coding are enabled for lossless coding. BDPCM is also enabled for lossless. In transform skip residual coding, the Rice Golumb coding proposed in CE7-1.2d-alternative [3] is included. The rice paramter in CE7-1.2d-alternative is given by

auiGoRicePars[std::min(max(sum + offset,0), 31)]

where sum is the summation of the absolute levels of top and left neighbors, and offset is derived based on the base level: offset = 0 when base level is equal to 0, offset = 4 when base level is equal to 1, and offset = -8 when base level is equal to 10.

For transform residual coding, the rice parameter derivation described in method 1 is applied. For transform skip residual coding, the rice parameter is derived based on the offset defined in [3] and sum of absolute levels of top and left neighboring coefficients as follows:

Define noPos as the number of neighboring (top and left) residuals available and sum as sum of absolute levels of top and left neighboring residuals, the variable sumAbs is derived as follows:

sumAbs = (noPos = = 1)? 2*sum: sum

The rice parameter RicePara is derived as:

RicePara = (Idx = = 33)? 5: ( (Idx = = 32)? 4: RiceParTable[ Idx ])

If offset >= 0:

Idx = (sumAbs > 128)? 33: ( (sumAbs > 64)? 32: min(max(sum + offset, 0), 31)

Otherwise (offset = -8),

Idx = (sumAbs > 128)? 32: min(max(sum + offset, 0), 31)

, where the rice table RiceParTable[] is the same as defined in [3, CE7-1.2d]

# Simulation Results

In the simulation, the anchor is AHG18 VTM-6.0 Lossless branch. The following settings are used to run both the anchor and test: TransquantBypassEnable : 1, CUTransquantBypassFlagForce : 1, CostMode : lossless, InternalBitDepth : 0. QP value was set as 0. Additionally, BDPCM is set to be 1 for the test. The results for method 1 and method 2 are shown in Table 1 and Table 2.

Table 1: Results of the proposed rice parameter derivation in method 1 for lossless coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | Proposed |  | VTM6 | Proposed |  | VTM6 | Proposed |  |
| Class A1 | 2.2 | 2.3 | -1.32% | 2.3 | 0.0 |  |  |  |  |
| Class A2 | 1.7 | 1.8 | -2.85% | 1.9 | 0.0 |  |  |  |  |
| Class B | 2.1 | 2.1 | -1.65% | 2.3 | 2.3 | -0.26% | 2.3 | 0.0 |  |
| Class C | 2.1 | 2.1 | -1.57% | 2.6 | 2.6 | -0.23% | 2.6 | 0.0 |  |
| Class D | 1.9 | 2.0 | -3.02% | 2.8 | 2.8 | -0.50% | 2.7 | 2.8 | -0.41% |
| Class E | 2.9 | 3.0 | -1.70% |  |  |  | 3.3 | 0.0 |  |
| Class F | 5.5 | 11.5 |  | 39.4 | 40.4 | -1.35% | 60.7 | 0.0 |  |
| TGM | 12.3 | 12.7 | -3.19% | 112.6 | 114.5 | -1.50% | 142.4 | 188.9 |  |
| Overall | 2.2 | 2.2 | -1.79% | 2.3 | 0.0 |  | 2.6 | 0.0 |  |
| Enc Time[%] | 101% |  |  | 19% |  |  | 28% |  |  |
| Dec Time[%] | 98% |  |  | 36% |  |  | 41% |  |  |

Table 2: Results of the proposed rice parameter derivation in method 2 for lossless coding

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM6 | 0 |  | VTM6 | 0 |  | VTM6 | 0 |  |
| Class A1 | 2.2 | 0.0 | #DIV/0! | 2.3 | 0.0 | #DIV/0! |  |  |  |
| Class A2 | 1.7 | 1.8 | #DIV/0! | 1.9 | 0.0 | #DIV/0! |  |  |  |
| Class B | 2.1 | 2.1 | -1.94% | 2.3 | 2.3 | #DIV/0! | 2.3 | 0.0 | #DIV/0! |
| Class C | 2.1 | 2.1 | -2.04% | 2.6 | 2.6 | -0.32% | 2.6 | 0.0 | #DIV/0! |
| Class D | 1.9 | 2.0 | -3.72% | 2.8 | 2.8 | -0.62% | 2.7 | 2.8 | -0.50% |
| Class E | 2.9 | 3.0 | -2.80% |  |  |  | 3.3 | 0.0 | #DIV/0! |
| Class F | 5.5 | 5.8 | -4.83% | 39.4 | 53.6 | #DIV/0! | 60.7 | 125.2 | #DIV/0! |
| TGM | 12.3 | 13.1 | -6.08% | 112.6 | 118.0 | -4.37% | 142.4 | 195.3 | #DIV/0! |
| Overall | 2.2 | 1.7 | #DIV/0! | 2.3 | 1.0 | #DIV/0! | 2.6 | 0.0 | #DIV/0! |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

# Reference

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz, “AHG18: BDPCM for Lossless”, JVET-P0082, 15th meeting, Gothenburg, SE, July 2019.

- Y.-H. Chao, A. Nalci, H. Wang, M. Coban, M. Karczewicz, “AHG18: Performance of CE7-1.2d and CE7-2.2 on lossless coding”, JVET-P0485, 15th meeting, Gothenburg, SE, July 2019.

- M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban, “CE7: Modifications to transform-skip residual coding (CE7-1.2)”, JVET-P0079, 15th meeting, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)