| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0561-v2 |
| --- | --- |

| Title: | AHG16: Context restriction on CTU boundary for line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu Yi-Wen Chen Xiaoyu Xiu Tsung-Chuan Ma Xianglin Wang | Email: | jhuhong-jheng@kwai.com yiwenchen@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to reduces the line buffer of Matrix Intra Prediction (MIP) and CU Depth information storage. For MIP mode flag storage, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. Compared to VTM6, the BD-rate results are 0.00%, 0.00%, and -0.04 % for AI, RA and LD, respectively, without increasing encoding and decoding time.

For intra mode flag storage, the context selection for the CABAC coding of prediction mode flag is not depending on the intra mode flag of above CU when the above CU is not located in the same CTU as current CU. Compared to VTM6, the BD-rate results are 0.01% and 0.00% for RA and LD, respectively, without increasing encoding and decoding time.

For depth information storage, the context selection for the CABAC coding of quadtree (or called QT) split  flag is not depending on the depth information of above CU when the above CU is not located in the same CTU as current CU. Compared to VTM6, the BD-rate results are -0.01%, 0.01%, and 0.01% for AI, RA and LD, respectively, without increasing encoding and decoding time.

Introduction

In current VVC [1], the MIP flag can be 0 and 1 to indicate if current coded block is MIP mode or not. In order to calculate the context index (ctxIdx) of MIP flag, the MIP flags of both the left and above neighboring blocks are used. As a result, the information of MIP flag needs to be stored in line buffer.

There is the same issue for intra mode flag. The intra mode flag can be 0 and 1 to indicate if current coded block is intra mode or not. The mode constraint flag equal to 0 specifies that coding units inside the current coding tree node can only use inter prediction coding modes and equal to 1 specifies that coding units inside the current coding tree node cannot use inter prediction coding modes. In order to calculate the context index (ctxIdx) of intra mode flag and mode constraint flag for a current block, the intra mode flag of both the left and above neighboring blocks are used. As a result, the information of intra mode flag needs to be stored in line buffer.

There is the same issue for depth information. The quadtree (or called QT) split flag can be 0 and 1 to indicate if current coded block is split based on quadtree partition or based on binary/ternary tree partition. In order to calculate the context index (ctxIdx) of QT split flag, the QT partition depth of above and left neighboring blocks are used. As a result, the information of such QT depth needs to be stored in line buffer.

Proposed method

In this contribution, it is proposed to reduce the line buffer of MIP mode storage. For MIP mode, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. In this case, the value of MIP flag of the above CU can be assumed as 0. As a result, there is no need to store MIP flag information in line buffer. The memory reduction of 4K sequences are 1K bits if one mode flag is 1 bit and stored per 4x4 unit.

For intra mode flag, the context selection for the CABAC coding of intra mode flag and mode constraint flag is not depending on the intra mode flag of above CU when the above CU is not located in the same CTU as current CU. In this case, the value of intra mode flag of the above CU can be assumed as 0. As a result, there is no need to store intra mode flag information in line buffer. The memory reduction of 4K sequences are 1K bits if one mode flag is 1 bit and stored per 4x4 unit.

For QT partition depth information, the context selection for the CABAC coding of QT split flag is not depending on the QT partition depth of above CU when the above CU is not located in the same CTU as current CU. In this case, the above CU can be assumed as unavailable CU. As a result, there is no need to store blocks’ QT partition depth information in line buffer. The memory reduction of 4K sequences are 4K bits if one QT partition depth information is 4 bit and stored per 4x4 unit.

Experimental results

Test1 : Remove MIP mode storage from line buffer

The proposed modification is integrated into VTM-6.0 [2] and its results and its results against VTM-6.0 under CTC [3] are shown in Table 1.

Table 1. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 98% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class B | 0.01% | 0.05% | 0.00% | 102% | 104% |
| Class C | -0.01% | 0.00% | 0.01% | 93% | 93% |
| Class E | 0.00% | 0.05% | 0.04% | 98% | 98% |
| Overall | 0.00% | 0.02% | 0.01% | 98% | 100% |
| Class D | -0.01% | -0.04% | 0.05% | 97% | 91% |
| Class F | 0.00% | -0.07% | 0.03% | 97% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | -0.02% | 103% | 103% |
| Class A2 | -0.01% | -0.01% | -0.07% | 101% | 99% |
| Class B | -0.01% | 0.07% | 0.09% | 99% | 101% |
| Class C | 0.00% | 0.08% | 0.03% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.02% | 99% | 100% |
| Class D | -0.05% | -0.20% | -0.12% | 93% | 94% |
| Class F | -0.03% | -0.10% | -0.17% | 96% | 97% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.06% | 0.03% | 98% | 96% |
| Class C | -0.03% | 0.05% | -0.06% | 103% | 104% |
| Class E | -0.11% | 0.48% | 0.37% | 101% | 105% |
| Overall | -0.04% | 0.16% | 0.09% | 100% | 101% |
| Class D | -0.01% | 0.56% | 0.51% | 100% | 108% |
| Class F | 0.00% | 0.12% | 0.25% | 98% | 98% |

Test2 : Remove intra mode storage from line buffer

The proposed modification is integrated into VTM-6.0 [2] and its results and its results against VTM-6.0 under CTC [3] are shown in Table 2.

Table 2. Performance of the proposed modification

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.01% | 102% | 101% |
| Class A2 | 0.03% | 0.03% | 0.00% | 103% | 101% |
| Class B | -0.01% | 0.07% | 0.06% | 102% | 101% |
| Class C | 0.00% | 0.06% | -0.05% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.01% | 102% | 101% |
| Class D | -0.01% | 0.00% | -0.04% | 97% | 95% |
| Class F | -0.01% | 0.00% | -0.01% | 101% | 99% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | 0.00% | 99% | 98% |
| Class C | -0.04% | 0.03% | -0.08% | 103% | 101% |
| Class E | 0.03% | 0.07% | 0.03% | 99% | 104% |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.04% | 0.00% | 0.53% | 101% | 103% |
| Class F | -0.02% | -0.17% | 0.53% | 101% | 99% |

Test3 : Remove QT partition depth storage from line buffer

The proposed modification is integrated into VTM-6.0 [2] and its results and its results against VTM-6.0 under CTC [3] are shown in Table 3.

Table 3. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.19% | 0.14% | 99% | 101% |
| Class A2 | -0.03% | 0.19% | 0.17% | 103% | 101% |
| Class B | 0.00% | 0.20% | -0.03% | 105% | 104% |
| Class C | -0.01% | 0.00% | 0.09% | 94% | 86% |
| Class E | 0.01% | 0.06% | 0.03% | 100% | 99% |
| Overall | -0.01% | 0.13% | 0.07% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.04% | 103% | 99% |
| Class F | 0.01% | 0.14% | 0.10% | 100% | 100% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.03% | 99% | 98% |
| Class A2 | 0.04% | 0.18% | 0.16% | 98% | 97% |
| Class B | 0.01% | 0.09% | 0.03% | 100% | 96% |
| Class C | -0.01% | 0.08% | 0.00% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.05% | 100% | 97% |
| Class D | -0.03% | -0.08% | -0.07% | 98% | 93% |
| Class F | -0.01% | -0.09% | 0.00% | 101% | 96% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.02% | -0.07% | 100% | 100% |
| Class C | -0.07% | -0.16% | 0.02% | 99% | 92% |
| Class E | 0.05% | -0.06% | 1.11% | 99% | 99% |
| Overall | 0.01% | -0.06% | 0.25% | 100% | 97% |
| Class D | -0.01% | 0.19% | -0.02% | 97% | 95% |
| Class F | -0.12% | -0.51% | 0.19% | 97% | 93% |

Test1+ Test2+ Test3

The proposed modification is integrated into VTM-6.0 [2] and its results and its results against VTM-6.0 under CTC [3] are shown in Table 4.

Table 4. Performance of the proposed modification

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.15% | 0.13% | 98% | 100% |
| Class A2 | -0.03% | 0.17% | 0.19% | 96% | 102% |
| Class B | 0.01% | 0.18% | -0.02% | 103% | 100% |
| Class C | -0.01% | 0.01% | 0.18% | 100% | 93% |
| Class E | 0.02% | 0.11% | 0.09% | 101% | 103% |
| Overall | 0.00% | 0.12% | 0.10% | 100% | 99% |
| Class D | 0.00% | -0.02% | 0.02% | 101% | 105% |
| Class F | 0.01% | 0.11% | 0.06% | 100% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.05% | 98% | 96% |
| Class A2 | 0.06% | 0.17% | 0.16% | 98% | 96% |
| Class B | 0.04% | 0.19% | 0.00% | 102% | 95% |
| Class C | 0.02% | -0.06% | -0.09% | 111% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.08% | 0.02% | 102% | 97% |
| Class D | -0.07% | -0.09% | -0.20% | 103% | 97% |
| Class F | -0.02% | -0.02% | 0.11% | 104% | 98% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.18% | 0.15% | 100% | 104% |
| Class C | 0.00% | 0.11% | -0.08% | 97% | 101% |
| Class E | -0.07% | 0.75% | -0.03% | 96% | 101% |
| Overall | 0.00% | 0.30% | 0.03% | 98% | 102% |
| Class D | 0.01% | 0.62% | 0.42% | 103% | 100% |
| Class F | -0.06% | 0.21% | 0.26% | 101% | 98% |

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, the 15th JVET meeting, Gothenburg, SE, 3–12 July 2019

- VTM-6.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-O1010, the 14th JVET meeting, March 2019, Geneva, CH

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).