| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0567-v1 |
| --- | --- |

| Title: | CE6-related: Optimized LFNST matrices for CE6-2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Amir Said, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a complete set of LFNST matrices (kernels) optimized for the simplification method studied in CE6-2.1. The experimental results show that the proposed set of matrices recovers the coding loss introduced in CE6-2.1. Over VTM-6.0, -0.02% AI and -0.02% RA BD-rates are achieved.

# Introduction

In CE6-2.1, the transform set mapping in VVC shown in Table 1 is replaced with the mapping rules in Table 2, removing one of the LFNST sets (lfnstTrSetIdx=2), so that the 8KB memory needed for LFNST matrices is reduced to 6KB.

Table 1: Mapping rules for four LFNST sets in VVC

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |

Table 2: Mapping rules for three LFNST sets in CE6-2.1

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 18 | 1 |
| 19 <= predModeIntra <= 49 | 3 |
| 50 <= predModeIntra <= 80 | 1 |

# Proposed Method

The proposed method introduces a new set of LFNST transform matrices (kernels) optimized for the mapping in Table 2. The proposed matrices are provided in the attached “new_lfnst_matrices_for_CE621.h” file with this submission.

# Experimental Results

The proposed matrices are replaced with the ones tested in CE6-2.1, and their coding performance is evaluated under common test conditions (CTC) over VTM-6.0. Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

In the following, Table 1 presents the results of the proposed combination of CE6-2.1 and optimized LFNST matrices. Table 2 presents the results of CE6.2.1 over VTM-6.0 as the benchmark.

Table 1.  Results of proposed method over VTM-6.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.38% | 0.44% | 100% | 101% |
| Class A2 | 0.00% | 0.26% | 0.23% | 101% | 102% |
| Class B | -0.02% | 0.04% | 0.14% | 100% | 102% |
| Class C | -0.03% | -0.03% | 0.11% | 101% | 104% |
| Class E | -0.05% | 0.43% | 0.35% | 102% | 103% |
| Overall | -0.02% | 0.18% | 0.24% | 101% | 102% |
| Class D | -0.03% | 0.08% | 0.06% | 101% | 105% |
| Class F | 0.00% | 0.15% | -0.02% | 100% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.15% | 0.14% | 101% | 101% |
| Class A2 | 0.01% | 0.07% | 0.08% | 102% | 102% |
| Class B | -0.02% | 0.07% | 0.01% | 101% | 101% |
| Class C | -0.05% | 0.06% | 0.07% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.09% | 0.07% | 101% | 102% |
| Class D | -0.02% | -0.18% | -0.19% | 101% | 102% |
| Class F | 0.02% | -0.11% | -0.06% | 101% | 101% |

Table 2.  Results CE6-2.1 over VTM-6.0 under CTC, where AI results are reproduced, and RA results are copied from JVET-P0044 (CE6-2.1).

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.11% | 0.16% | 101% | 100% |
| Class A2 | 0.04% | 0.15% | 0.19% | 102% | 103% |
| Class B | 0.03% | -0.05% | -0.03% | 101% | 100% |
| Class C | 0.04% | -0.07% | 0.10% | 102% | 104% |
| Class E | 0.00% | 0.25% | 0.22% | 103% | 102% |
| Overall | 0.03% | 0.06% | 0.11% | 102% | 102% |
| Class D | 0.01% | 0.13% | 0.02% | 102% | 101% |
| Class F | 0.07% | 0.08% | 0.06% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.02% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.16% | 0.01% | 100% | 100% |
| Class B | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class C | -0.01% | 0.11% | 0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.03% | 100% | 100% |
| Class D | -0.07% | -0.16% | 0.03% | 100% | 101% |
| Class F | 0.06% | 0.00% | 0.22% | 100% | 101% |

# Conclusions

The current set of LFNST matrices in VVC are optimized specifically for the mapping in Table 1. This document demonstrates that the coding loss in CE6-2.1 can be compensated by optimizing the LFNST matrices. Specifically, the 0.03%AI loss is recovered, and 0.02% AI gain is achieved. It is recommended to (conditionally) adopt the proposed method in VVC if the mapping in CE6-2.1 is adopted.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- Y Zhao, H. Yang, “CE6-2.1: LFNST with 3 transform sets”, JVET-P0044, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th meeting, Macao, CN, Oct. 2018.

- A. Said, X. Zhao, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-M0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th meeting, Marrakech, MA, 19-27 March 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-N0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th meeting, Geneva, CH, 19-27 March 2019.

- X. Zhao, H. E. Egilmez, “CE6: Summary Report on Transforms and Transform Signaling”, JVET-O0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 15th meeting, Gothenburg, SE, 3-12 July 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).