| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0571 |
| --- | --- |

| Title: | CE5/CE8: Deblocking Filter for BDPCM coded block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to handle deblocking filter for BDPCM block as follow:

- Fix mismatch between SW and specification for boundary strength derivation

- Bypass filtering process for BDPCM coded block

The deblocking filter process for BDPCM block is handled by boundary strength set equal to 0 when the across blocks on edge is coded by BDPCM. But there is one mismatch between specification and SW. The first suggestion in this contribution is to fix this mismatch.

The second proposed method is to align BDPCM deblocking filter process with the other screen content prediction method; palette coded block. If one of the block across the edge is coded and the other is coded intra coded, the boundary strength is set equal to 2 even though the prediction error and quantization error of BDPCM is different from conventional intra/inter block. In this contribution, it is suggested that the pixel modification by filtering on the BDPCM coded block is bypassed. The experimental results for SCC and TGM shows respectively as below:

- 0.00%, 0.00% for AI, 0.00%, 0.XX% for RA and 0.04%, 0.09% for LDB

- 0.01%, -0.03% for AI, 0.01%, -0.03% for RA and 0.00%, -0.07% for LDB

Introduction

VTM6 supports BDPCM for screen content coding. When BDPCM is enabled a flag is transmitted at the CU level for the luma block. For the chroma component, the BDPCM is not allowed. If a CU is BDPCM coded, the transform skip flag is set equal to 1 and the predicted quantized residual values are sent to the decoder using the transform skip mode residual coding. For deblocking in VTM-6.0, if both blocks on the edge are coded using BDPCM then that particular block boundary is not filtered. The edge for chroma component is also not filtered even though the chroma is not coded as BDPCM. But the deblocking filter for chroma component is described that boundary strength is independently derived with luma component in specification. Hence, there is mismatch between specification and SW. In this contribution, it is suggested to SW to text specification. Additionally, it is also suggest to align filtering process with palette mode to prevent filtering inside coded region which has screen content property.

Proposal

- Fix mismatch : Align software with specification

if( ( MODE_INTRA == cuP.predMode && cuP.bdpcmMode ) && ( MODE_INTRA == cuQ.predMode && cuQ.bdpcmMode ) ) { return (BsSet(0, COMPONENT_Y) + BsSet(2, COMPONENT_Cb) + BsSet(2, COMPONENT_Cr)); }

- Align filtering process for BDPCM with Palette.

The proposed decoding process is dictated as below:

8.8.3.6.6 Filtering process for a luma sample using short filters

Inputs to this process are:

- the sample values pi and qi with i = 0..3,

- the locations of pi and qi, ( xPi, yPi ) and ( xQi, yQi ) with i = 0..2,

- a variable dE,

- the variables dEp and dEq containing decisions to filter samples p1 and q1, respectively,

- a variable tC.

Outputs of this process are:

- the number of filtered samples nDp and nDq,

- the filtered sample values pi′ and qj′ with i = 0..nDp − 1, j = 0..nDq − 1.

Depending on the value of dE, the following applies:

- If the variable dE is equal to 2, nDp and nDq are both set equal to 3 and the following strong filtering applies:

p0′ = Clip3( p0 − 3 * tC, p0 + 3 * tC, ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3 )	(8-1162)

p1′ = Clip3( p1 − 2 * tC, p1 + 2 * tC, ( p2 + p1 + p0 + q0 + 2 )  >>  2 )	(8-1163)

p2′ = Clip3( p2 − 1 * tC, p2 + 1*tC, ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3 )	(8-1164)

q0′ = Clip3( q0 − 3 * tC, q0 + 3 * tC, ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3 )	(8-1165)

q1′ = Clip3( q1 − 2 * tC, q1 + 2 * tC, ( p0 + q0 + q1 + q2 + 2 )  >>  2 )	(8-1166)

q2′= Clip3( q2 − 1 * tC, q2 + 1 * tC, ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3 )	(8-1167)

- Otherwise, nDp and nDq are set both equal to 0 and the following weak filtering applies:

  - The following applies:

= ( 9 * ( q0 −  p0 ) − 3 * ( q1 − p1 ) + 8 )  >>  4	(8-1168)

  - When Abs() is less than tC * 10, the following ordered steps apply:

    - The filtered sample values p0′ and q0′ are specified as follows:

= Clip3( −tC, tC,  )		(8-1169)

p0′ = Clip1Y( p0 +  )		(8-1170)

q0′ = Clip1Y( q0 −  )		(8-1171)

    - When dEp is equal to 1, the filtered sample value p1′ is specified as follows:

p = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( p2 + p0 + 1 )  >>  1 ) − p1 +  )  >>  1 )	(8-1172)

p1′ = Clip1Y( p1 + p )		(8-1173)

    - When dEq is equal to 1, the filtered sample value q1′ is specified as follows:

q = Clip3( −( tC  >>  1 ), tC  >>  1, ( ( ( q2 + q0 + 1 )  >>  1 ) − q1 −  )  >>  1 )	(8-1174)

q1′ = Clip1Y( q1 + q )		(8-1175)

    - nDp is set equal to dEp + 1 and nDq is set equal to dEq + 1.

When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

- intra_bdpcm_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q	0 is equal to 1.

- intra_bdpcm_flag of the coding unit that includes the coding block containing the sample q	0 is equal to 1.

8.8.3.6.7 Filtering process for a luma sample using long filters

Inputs to this process are:

- the variables maxFilterLengthP and maxFilterLengthQ,

- the sample values pi and qj with i = 0..maxFilterLengthP and j = 0..maxFilterLengthQ,

- the locations of pi and qj, ( xPi, yPi ) and ( xQj, yQj ) with i = 0..maxFilterLengthP − 1 and j = 0..maxFilterLengthQ − 1,

- a variable tC.

Outputs of this process are:

- the filtered sample values pi′ and qj′ with i = 0..maxFilterLengthP − 1, j = 0..maxFilterLenghtQ − 1.

The variable refMiddle is derived as follows:

- If maxFilterLengthP is equal to maxFilterLengthQ and maxFilterLengthP is equal to 5, the following applies:

refMiddle = ( p4 + p3 + 2* ( p2 + p1 + p0 + q0 + q1 + q2 ) + q3 + q4 + 8)  >>  4	(8-1176)

- Otherwise, if maxFilterLengthP is equal to maxFilterLengthQ and maxFilterLengthP is not equal to 5, the following applies:

refMiddle = ( p6 + p5 + p4 + p3 + p2 + p1 + 2* ( p0 + q0 ) + q1 + q2 + q3 + q4 + q5 + q6 + 8 )  >>  4	(8-1177)

- Otherwise, if one of the following conditions are true,

- maxFilterLengthQ is equal to 7 and maxFilterLengthP is equal to 5,

- maxFilterLengthQ is equal to 5 and maxFilterLengthP is equal to 7,

the following applies:

refMiddle = ( p4 + p3 + 2* ( p2 + p1 + p0 + q0 + q1 + q2 ) + q3 + q4 + 8 )  >>  4	(8-1178)

- Otherwise, if one of the following conditions are true,

- maxFilterLengthQ is equal to 5 and maxFilterLengthP is equal to 3,

- maxFilterLengthQ is equal to 3 and maxFilterLengthP is equal to 5,

the following applies:

refMiddle = ( p3 + p2 + p1 + p0 + q0 + q1 + q2 + q3 + 4)  >>  3	(8-1179)

- Otherwise, if maxFilterLengthQ is equal to 7 and maxFilterLengthP is equal to 3, the following applies:

refMiddle = ( 2 * ( p2 + p1 + p0 + q0 ) + p0 + p1 + q1 + q2 + q3 + q4 + q5 + q6 + 8 )  >>  4	(8-1180)

- Otherwise, the following applies:

refMiddle = ( p6 + p5 + p4 + p3 + p2 + p1 + 2*( q2 + q1 + q0 + p0)  + q0 + q1 + 8 )  >>  4	(8-1181)

The variables refP and refQ are derived as follows:

refP = ( pmaxFilterLengtP + pmaxFilterLengthP-1 + 1 )  >>  1		(8-1182)

refQ = ( qmaxFilterLengtQ + qmaxFilterLengthQ-1 + 1 )  >>  1		(8-1183)

The variables fi and tCPDi are defined as follows:

- If maxFilterLengthP is equal to 7, the following applies:

f0..6 = { 59, 50, 41, 32, 23, 14, 5 }		(8-1184)

tCPD0..6 = { 6,  5,  4,  3,  2,  1,  1 }		(8-1185)

- Otherwise, if maxFilterLengthP is equal to 5, the following applies:

f0..4 = { 58, 45, 32, 19, 6 }		(8-1186)

tCPD0..4 = { 6, 5, 4, 3, 2 }		(8-1187)

- Otherwise, the following applies:

f0..2 = { 53, 32, 11 }		(8-1188)

tCPD0..2 = { 6, 4, 2}		(8-1189)

The variables gj and tCQDj are defined as follows:

- If maxFilterLengthQ is equal to 7, the following applies:

g0..6 = { 59, 50, 41, 32, 23, 14, 5 }		 (8-1190)

tCQD0..6 = { 6, 5, 4, 3, 2, 1, 1 }		 (8-1191)

- Otherwise, if maxFilterLengthQ is equal to 5, the following applies:

g0..4 = { 58, 45, 32, 19, 6 }		 (8-1192)

tCQD0..4 = { 6, 5, 4, 3, 2 }		 (8-1193)

- Otherwise, the following applies:

g0..2 = { 53, 32, 11 }		(8-1194)

tCQD0..2 = { 6, 4, 2 }		(8-1195)

The filtered sample values pi′ and qj′ with i = 0..maxFilterLengthP − 1 and j = 0..maxFilterLengthQ − 1 are derived as follows:

pi′ = Clip3( pi − ( tC*tCPDi )  >> 1, pi + ( tC*tCPDi ) >> 1, ( refMiddle*fi + refP*( 64 − fi )  +  32)  >>  6 )	(8-1196)

qj′ = Clip3( qj − ( tC*tCQDj ) >> 1, qj + ( tC*tCQDj ) >> 1, ( refMiddle*gj + refQ*(64 − gj ) +  32)  >>  6 )	 (8-1197)

When one or more of the following conditions are true, the filtered sample value, p i′ is substituted by the corresponding input sample value p i with i = 0..maxFilterLengthP − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- intra_bdpcm_flag of the coding unit that includes the coding block containing the sample pi is equal to 1

When one or more of the following conditions are true, the filtered sample value, q i′ is substituted by the corresponding input sample value q j with j = 0..maxFilterLengthQ − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- intra_bdpcm_flag of the coding unit that includes the coding block containing the sample qi is equal to 1

8.8.3.6.9 Filtering process for a chroma sample

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

- the variable maxFilterLength,

- the chroma sample values pi and qi with i = 0..maxFilterLengthCbCr,

- the chroma locations of pi and qi, ( xPi, yPi ) and ( xQi, yQi ) with i = 0..maxFilterLengthCbCr − 1,

- a variable tC.

Outputs of this process are the filtered sample values pi′ and qi′ with i = 0..maxFilterLengthCbCr − 1.

The filtered sample values pi′ and qi′ with i = 0..maxFilterLengthCbCr − 1 are derived as follows:

- If maxFilterLengthCbCr is equal to 3, the following strong filtering applies:

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + p1 + 2 * p0 + q0 + q1 + q2  + 4 )  >>  3 )	(8-1198)

p1′ = Clip3( p1 − tC, p1 + tC, ( 2 * p3 + p2 + 2 * p1 + p0 + q0 + q1  + 4 )  >>  3 )	(8-1199)

p2′ = Clip3( p2 − tC, p2 + tC, ( 3 * p3 + 2 * p2 + p1 + p0 + q0 + 4 )  >>  3 )	(8-1200)

q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + p1 + p0 + 2 * q0 + q1 + q2 + q3  + 4 )  >>  3 )	(8-1201)

q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3  + 4 )  >>  3 )	(8-1202)

q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 )  >>  3 )	(8-1203)

- Otherwise, the following weak filtering applies:

= Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )	(8-1204)

p0′ = Clip1C( p0 +  )		(8-1205)

q0′ = Clip1C( q0 −  )		(8-1206)

When one or more of the following conditions are true, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthCbCr − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

- intra_bdpcm_chroma_flag of the coding unit that includes the coding block containing the sample pi is equal to 1.

When one or more of the following conditions are true, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthCbCr − 1:

- cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

- intra_bdpcm_chroma_flag of the coding unit that includes the coding block containing the sample qi is equal to 1.

Experimental result

The experimental result of the proposed method shows as following:

- Method 1. Fix mismatch to align software with specification

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 96% | 93% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class SCC | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 95% | 93% |
| Class F | 0.00% | 0.01% | -0.01% | 96% | 98% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.04% | 0.08% | 0.70% | 99% | 102% |
| Class SCC | 0.09% | 0.09% | 0.03% | 99% | 101% |

- Method 2. Bypass pixel modification by filtering to align with other screen coding tool

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class F | 0.01% | 0.01% | 0.00% | 98% | 100% |
| Class SCC | -0.03% | 0.00% | -0.01% | 100% | 103% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 94% | 93% |
| Class F | 0.01% | -0.06% | -0.05% | 97% | 99% |
| Class SCC | -0.03% | -0.05% | -0.03% | 100% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 104% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.29% | 0.35% | 100% | 102% |
| Class SCC | -0.07% | -0.04% | -0.09% | 99% | 102% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).