| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0572 |
| --- | --- |

| Title: | AHG17/Non CE-4: Restrict integer-pel fetch for BDOF and RPOF for subpicture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seethal Paluri Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes disabling fetch reference sample from different subpicture(s) during BDOF and PROF process to support treating a subpicture as a picture in the decoding process. When a subpicture is treated as a picture, restrictions are placed on the derivations processs for (advanced) temporal luma motion vector prediction, luma sample bilinear interpolation process, luma sample 8-tap interpolation filtering process and chroma sample interpolation process. However, it is asserted that the current Working Draft (WD 6) of the Versatile Video Coding (VVC) specification text, references samples outside the subpicture boundary that could be fetched as part of the integer reference sample process for the gradient computation in BDOF and PROF. This proposal therefore, presents a restriction on the Luma integer sample fetching. Additionally It is assert that Fractional sample interpolation processes, by disallowing the samples to span outside the subpicture boundaries.

Background

The predication sample is modified by bdofOffset for BDOF and dI for PROF. The dbofOffset and dI values are obtained from a gradient which is derived as the value of the difference of prediction samples in the reference picture. To achieve the gradient, the prediction sample is derived by invoking the luma integer sample fetching process and an 8-tap interpolation filter process. Current Working Draft (WD) describes that the luma integer sample could be fetched from the different subpicture unlike prediction sample derivation process based on 8-tap interpolation filter process [1]. As input in the 8-tap interpolation filter process, the reference integer sample position is clipped to the same subpicture. Figure 1 below shows the case which fetch area is across subpicture boundaries. The interpolation is applied based on integer reference sample within the same subpicture. However, Figure 2 demonstrates that fetched integer samples can also come from a different subpicture.

Figure 1 shows the case fetched area for PROF and BDOF is across subpicture boundaries.

Figure 2 shows the fetched integer reference sample at a different subpicture

Proposal

Additionally, this contribution suggests to consider subpicture for luma locations pointed to by a motion vector. The proposed specification text changes are provided as below:

1) Proposed 1: Restricted integer sample fetch process.

8.5.6.3.3 Luma integer sample fetching process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL  )	(8-771)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL  )	(8-772)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(8-782)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(8-783)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3		(8-784)

2) Proposed 2: Restricted collocated position for SbTMVP

The current SbTMVP can refer across vertical subpicture boundary an also can refer left subpicture boundary when the subpicture boundary is not aligned at CTU boundary. It is also asserted that the collocated subblock should be within same subpicture.

8.5.5.3 Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- the availability flag availableFlagA1 of the neighbouring coding unit,

- the reference index refIdxLXA1 of the neighbouring coding unit with X being 0 or 1,

- the prediction list utilization flag predFlagLXA1 of the neighbouring coding unit with X being 0 or 1,

- the motion vector in 1/16 fractional-sample accuracy mvLXA1 of the neighbouring coding unit with X being 0 or 1.

Outputs of this process are:

- the availability flag availableFlagSbCol,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the reference indices refIdxL0SbCol and refIdxL1SbCol,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0SbCol[ xSbIdx ][ ySbIdx ] and mvL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the prediction list utilization flags predFlagL0SbCol[ xSbIdx ][ ySbIdx ] and predFlagL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The availability flag availableFlagSbCol is derived as follows.

- If one or more of the following conditions is true, availableFlagSbCol is set equal to 0.

- slice_temporal_mvp_enabled_flag is equal to 0.

- sps_sbtmvp_enabled_flag is equal to 0.

- cbWidth is less than 8.

- cbHeight is less than 8.

- Otherwise, the following ordered steps apply:

- The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows:

xCtb = ( xCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-542)

yCtb = ( yCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-543)

xCtr = xCb + ( cbWidth / 2 )		(8-544)

yCtr = yCb + ( cbHeight / 2 )		(8-545)

- The luma location ( xColCtrCb, yColCtrCb ) is set equal to the top-left sample of the collocated luma coding block covering the location given by ( xCtr, yCtr ) inside ColPic relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for subblock-based temporal merging base motion data as specified in clause 8.5.5.4 is invoked with the location ( xCtb, yCtb ), the location ( xColCtrCb, yColCtrCb ), the availability flag availableFlagA1, and the prediction list utilization flag predFlagLXA1, and the reference index refIdxLXA1, and the motion vector mvLXA1, with X being 0 and 1 as inputs and the motion vectors ctrMvLX, and the prediction list utilization flags ctrPredFlagLX of the collocated block, with X being 0 and 1, and the temporal motion vector tempMv as outputs.

- The variable availableFlagSbCol is derived as follows:

- If both ctrPredFlagL0 and ctrPredFlagL1 are equal to 0, availableFlagSbCol is set equal to 0.

- Otherwise, availableFlagSbCol is set equal to 1.

When availableFlagSbCol is equal to 1, the following applies:

- The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows:

numSbX  =  cbWidth >> 3		(8-546)

numSbY  =  cbHeight >> 3		(8-547)

sbWidth  =  cbWidth / numSbX		(8-548)

sbHeight  =  cbHeight / numSbY		(8-549)

refIdxLXSbCol  =  0			(8-550)

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth + sbWidth / 2	(8-551)

ySb  =  yCb + ySbIdx * sbHeight + sbHeight / 2	(8-552)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

    - The following applies:

      - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies

yColSb = Clip3( Max(SubPicTopBoundaryPos, yCtb),
Min( SubPicBotBoundaryPos, yCtb + ( 1  <<  CtbLog2SizeY ) – 1. ),	(8-553)
ySb + ( tempMv[1]  >>  4 ) )

      - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

yColSb = Clip3( yCtb,
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ),	(8-553)
	ySb + ( tempMv[1]  >>  4 ) )

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xColSb = Clip3( Max(SubPicLeftBoundaryPos, xCtb), 
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(8-554)
	xSb + ( tempMv[0]  >>  4 ) )

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xColSb = Clip3( xCtb,
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(8-555)
	xSb + ( tempMv[0]  >>  4 ) )

2) Proposed 3: Clarify incorrect right and bottom subpicture boundary position.

Additionally, current subpicture boundary is derived incorrectly. It is asserted that the right and bottom boundary of subpicture could be clarified as below in order to restrict motion vector refer to outside subpicture for interpolation filter and SbTMVP processing

The variables SubPicIdx, SubPicLeftBoundaryPos, SubPicTopBoundaryPos, SubPicRightBoundaryPos, and SubPicBotBoundaryPos are derived as follows:

SubPicIdx = CtbToSubPicIdx[ CtbAddrBsToRs[ FirstCtbAddrBs[ SliceBrickIdx[ 0 ] ] ] ]
if( subpic_treated_as_pic_flag[ SubPicIdx ] ) {
	SubPicLeftBoundaryPos = SubPicLeft[ SubPicIdx ] * ( subpic_grid_col_width_minus1 + 1 ) * 4
	SubPicRightBoundaryPos = ( SubPicLeft[ SubPicIdx ] + SubPicWidth[ SubPicIdx ] ) *
				( subpic_grid_col_width_minus1 + 1 ) * 4 − 1	(7-93)
	SubPicTopBoundaryPos = SubPicTop[ SubPicIdx ] * ( subpic_grid_row_height_minus1 + 1 )* 4
	SubPicBotBoundaryPos = ( SubPicTop[ SubPicIdx ] + SubPicHeight[ SubPicIdx ] ) *
				( subpic_grid_row_height_minus1 + 1 ) * 4 − 1
}

Reference

- Y.Wang, Hendry, J. Chien, S. Hong, J. Boyce, L. Xu, R. Skupin, Y. Sanchez, K. Suehring and M. Hannuksela , “AHG12: Text for subpicture agreements integration,” document JVET-O1143, 15th JVET meeting: Gothernburg, SE, 3–12 July 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).