| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0575 |
| --- | --- |

| Title: | NonCE: to support SCIPU with local dual tree for various color format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to support smallest chroma intra prediction unit(SCIPU) restriction based on actual number of chroma samples for 4:2:2. In VVC draft 6, the small chroma block sizes (2x2/2x4/4x2) are restricted in dual tree as a partitioning restriction. The SCIPU is defined as coding tree node whose chroma block size is larger than or equal to 16 chroma samples and the local dual tree is introduced and is adopted to disallow the small chroma block size(2x2/2x4/4x2) in single tree. The local dual tree originally supports 4:2:0 only. After local dual tree adoption, it is disallowed for 4:4:4 color format to apply SCIPU by bug ticket #456. However, current the local dual tree decision mechanism further restricts the chroma partition for 4:2:2 color format and separate color plane coding. The proposed method shows identical results with VTM-6.0 anchor CTC.

Introduction

In VVC Draft6, chroma intra CBs smaller than 16 chroma samples is disallowed by constraining the partitioning of chroma intra CBs in order to improve worst case throughput. In single coding tree, a smallest chroma intra prediction unit (SCIPU) is defined as a coding tree node whose chroma block size is larger than or equal to 16 chroma samples and has at least one child luma block smaller than 64 luma samples. To restrict chroma partitioning in single tree, current it is defined that modeTypeCondition is derived from number of luma pixels as below:

The variable modeTypeCondition is derived as follows: - If one of the following conditions is true, modeTypeCondition is set equal to 0 - slice_type = = I and qtbtt_dual_tree_intra_flag is equal to 1 - modeTypeCurr is not equal to MODE_TYPE_ALL - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 - cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1 - cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER > 16 - cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type != I ? 1 : 0) - cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER - Otherwise, modeTypeCondition is set equal to 0

This derivation mechanism further restrict chroma block size for 4:2:2 as below:

Case 1: The number of luma pixel is 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

As depicted in Figure 1 and Figure 2, the chroma block is partitioned to 2x4/4x2 for 4:2:0 format. Hence, modeTypeCondition is derived by transmitted mode_constraint_flag to indicate whether intra coded without further split. But if the same logic is applied for 4:2:2 format, the chroma partitioning could be restricted for 4:2:2 color format even though the chroma block is not 2x2/2x4/4x2 after splitting.

Figure 1

Figure 2

Case 2: The number of luma pixel is 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

As depicted in Figure 3 and Figure 4, the chroma partitioning will be 2x4/4x2 after splitting for 4:2:0 format. Therefore the mode_constraint_flag is signaled to indicate whether intra coded without further split. For 4:2:2, it could be restricted chroma block partition even though the chroma block is not 2x2/2x4/4x2 after splitting.

Figure 3

Figure 4

Case 3: The number of luma pixel is 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

As the example described in Figure 5, the chroma partitioning will be 2x2 after splitting for 4:2:0 format. Therefore the modeTypeCondition is inferred to 1 to restricted the smallest chroma CBs without mode_constraint_flag signaling. But for 4:2:2 format, it could be restricted chroma block partition even though the chroma block is not 2x2 after splitting.

Proposal

The variable modeTypeCondition is derived as follows: - If one of the following conditions is true, modeTypeCondition is set equal to 0 - slice_type = = I and qtbtt_dual_tree_intra_flag is equal to 1 - modeTypeCurr is not equal to MODE_TYPE_ALL - separate_colour_plane_flag is equal to 1 - chroma_format_idc is equal to 0 - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 - (cbWidth/subWidthC) * (cbHeight/subHeightC) is equal to 16 and split_qt_flag is equal to 1 - (cbWidth/subWidthC) * (cbHeight/subHeightC) is equal to 16 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER - Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type != I ? 1 : 0) - (cbWidth/subWidthC) * (cbHeight/subHeightC) is equal to 16 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER - (cbWidth/subWidthC) * (cbHeight/subHeightC) is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER - Otherwise, modeTypeCondition is set equal to 0

Reference

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).