| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0576 |
| --- | --- |

| Title: | AHG18: on low level coding tool for lossless coding. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Jungah Choi Junghak Nam Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution suggests to support lossless coding for VVC. In this proposal, in order to adaptively control whether lossless coding or lossy coding, a flag is transmitted to bypass transform and quantization in CU level as HEVC, and the low level coding tools are disabled for lossless block. The mandatory low level coding tool restriction is described as below:

- Disabling transform related coding tools and syntax signaling; MTS, LFNST, SBT and transform skip for lossless coded block.

- Disabling JointCbCr and the syntax signaling for lossless coded block

- Disabling LMCS in slice level and the syntax signaling when lossless is enabled.

- Disabling sign data hiding for transform/quantization bypass block.

- Disabling RDO process for LFNST/MTS in encoder side.

In order to support co-existence between the lossless coding and lossy coding at the CU-level, the following additional restrictions are suggested..

- Bypass ALF processing for lossless coded block.

- Support residual coding for Size MxN(M= 64 or N = 64) when MaxTbSize is signaled 64 in SPS.

  - Implicit TU tiling to 32 for lossless block

  - Extend coefficient scanning

Introduction

At the 15th JVET meeting in Gothenburg, the JVET-O1061 was proposed by three companies to support lossless functionality for VVC[1]. In JVET-O1061 include several aspect as below:

- Transform/quantization bypass enable flag at PPS

- Transform/quantization bypass flag for CU level

- Disabling syntax signaling related transform; MTS, LFNST, SBT and transform skip.

- Disabling Sign data hiding for transform/quantization bypass block.

- Disabling LMCS for lossless is enabled in slice level

- Disabling JointCbCr for transform/quantization bypass block.

- Bypass ALF processing for transform/quantization bypass block.

- Disable Coeffient Zero-out for transform/quantization bypass block.

- Disabling syntax signaling related QP

Proposal

In order to support lossless coding functionality for VVC and also support flexibility for lossless and lossy coding at CU level, the handling method is suggested regarding ALF and residual coding for large tranaform unit as following:

- Bypass ALF processing for lossless coded block.

The proposed method is that ALF is bypassed when cu_transquant_bypass_flag of current sample is equal to 1. Current ALF process allows filtering even though cu_transquant_bypass_flag is equal to 1. Current DBF and SAO processing already considers transform/quantization bypassed block.

Subcluase 8.9.5.2 Coding tree block filtering process for luma samples - The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows: - If pcm_loop_filter_disabled_flag and pcm_flag[ xCtb+ x ][ yCtb + y ] are both equal to 1, the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = recPictureL[ hx, vy ] (8-1207) - Othersiwe if cu_transquant_bypass_flag[ xCtb+ x ][ yCtb + y ] is equal to 1, the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = recPictureL[ hx, vy ] (8-1207) - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies: alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum ) (8-1208) Subclause 8.8.5.4 Coding tree block filtering process for chroma samples - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows: - If pcm_loop_filter_disabled_flag and pcm_flag[ ( xCtbC + x ) * SubWidthC ][ ( yCtbC + y ) * SubHeightC ] are both equal to 1, the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = recPicturec[ hx, vy ] (8-1259) - Otherwise, if cu_transquant_bypass_flag[ ( xCtbC + x ) * SubWidthC ][ ( yCtbC + y ) * SubHeightC ] is equal to 1, the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = recPicturec[ hx, vy ] (8-1259) - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies: alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum ) (8-1260)

- Support residual coding for MxN(M= 64 or N = 64) when MaxTbSize is signaled 64 in SPS.

Current VTM-6.0 support maximum transform unit size up to 64x64 and the coefficient zeroing out is applied except top-left 32x32. The 64x64 lossless coded block is not allowed by the coefficient zeroing-out. It is asserted that in order to support 64x64 lossless block, implicit TU tiling or disabling zero-out process for lossless coded block respectively.

  - Implicit TU tiling to 32 for lossless block

The proposed method is that TU is split to the 32 implicitly when cu_transquant_bypass_flag is equal to 1.

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= implicitTuTileSize && cbHeight <= implicitTuTileSize) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| } |  |

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, chType ) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > implicitTuTileSize¦ ¦ tbHeight > implicitTuTileSize) { |  |
| verSplitFirst = ( tbWidth > implicitTuTileSize&& tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = verSplitFirst ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = !verSplitFirst ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight, chType ) |  |
| if( verSplitFirst ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| else |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, chType ) |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0, chType ) |  |
| } |  |
| } else if( cu_sbt_flag ) { |  |
| if( !cu_sbt_horizontal_flag ) { |  |
| trafoWidth = tbWidth * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, trafoWidth, tbHeight, treeType , 0, 0 ) |  |
| transform_unit( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType, 1, 0 ) |  |
| } else { |  |
| trafoHeight = tbHeight * SbtNumFourthsTb0 / 4 |  |
| transform_unit( x0, y0, tbWidth, trafoHeight, treeType , 0, 0 ) |  |
| transform_unit( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType, 1, 0 ) |  |
| } |  |
| } else if( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT ) { |  |
| trafoHeight = tbHeight / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0, y0 + trafoHeight * partIdx, tbWidth, trafoHeight, treeType, partIdx, 0 ) |  |
| } else if( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) { |  |
| trafoWidth = tbWidth / NumIntraSubPartitions |  |
| for( partIdx = 0; partIdx < NumIntraSubPartitions; partIdx++ ) |  |
| transform_unit( x0 + trafoWidth * partIdx, y0, trafoWidth, tbHeight, treeType, partIdx, 0 ) |  |
| } |  |
| } |  |

The variable of implicitTuTileSize is derive as follows:

mplicitTuTileSize = cu_transquant_bypass_flag[ x0 ][ y0 ] ? 32 : MaxTbSizeY

  - Extend coefficient scanning

The alternative method to support 64x64 TU. The coefficient zero-out is disabled when cu_transquant_bypass_flag is equal to 1.

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |
| log2ZoTbWidth = 4 |  |
| else |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 + cu_transquant_bypass_flag[x0][y0]) |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |
| log2ZoTbHeight = 4 |  |
| else |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 + cu_transquant_bypass_flag[x0][y0]) |  |

Reference

- “Lossless coding for VVC”, Tsung-Chuan Ma, Yi-Wen Chen, Xiaoyu Xiu, Xianglin Wang (Kwai Inc), Tangi Poirier, Fabrice Le Léannec, Karam NaserK. Naser, Edouard FrancoisFranck Galpin (InterDigital), Hyeongmun Jang, Junghak Nam, Naeri Park, Jungah Choi, Seunghwan Kim, Jaehyun Lim (LGE), Document JVET-O1061, Gothenburg, SE, July 2019

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).