| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0581-v1 |
| --- | --- |

| Title: | On Subpicture Cropping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

It is proposed that cropping information be optionally signalled on subpicture basis. Example are described for 360 and non-360 use cases where a subpicture can be extracted and the signalled cropping information can be applied for output.

# Introduction

VVC WD [1] includes signalling of cropping information for pictures.

It is proposed that cropping information be optionally signalled on subpicture basis. Example are described for 360 and non-360 use cases where a subpicture can be extracted and can be applied the signalled cropping information for output.

We describe two examples in detail below which illustrate that subpicture-based cropping information is useful.

## Example A: 360 Cubemap: 6 cube faces, using CtbSizeY=32

This example shows 6 subpictures each of which have a true resolution of 1200x1200, encoded with CtbSizeY equal to 32. Each subpicture represents a cubemap face for a 360 cubemap projection. This encoding may be suitable for a 2160x1200 resolution HMDs (e.g. HTC VIVE, Oculus Rift, etc.). In this case it is proposed to code each subpicture cubemap face as 1216x1216 and to specify cropping information for each subpicture. This will allow easy extraction of each subpicture from the bitstream. In this case the overall picture which is composed of 6 subpictures is coded with a picture size of 3648x2432. The cropping information for the overall picture is derived from the cropping information for each subpicture.

Fig. A: Cubemap with subpictures for each face coded with CtbSizeY=32. Each subpicture has cropping information.

## Example B: Non-360: 8K resolution: 4x4 subpictures each with 1080p resolution, using CtbSizeY=32

This example shows 16 subpictures each of which have a true resolution of Full HD (1920x1080), encoded with CtbSizeY equal to 32 in Fig. 1A. Such a layout could for example be used in a surveillance application where videos feeds from 16 different cameras are composed into one large video feed. Another example is a sports event or a concert where multiple videos are put together in a video mosaic to be edited further and/or consumed differently by different devices. In this case it is proposed to code each subpicture as 1920x1088 and to specify cropping information for each subpicture. This will allow easy extraction of each subpicture from the bitstream and then allow cropping each subpicture to a Full HD resolution. In this case the overall picture which is composed of 16 subpictures is coded with a picture size of 7680x4352. The cropping information for the overall picture is derived from the cropping information for each subpicture.

Fig 3: Sixteen Full HD cropped sub-pictures coded with CtbSizeY=32. Each subpicture has a cropping information.

Additionally following examples are asserted to benefit from subpicture based cropping information signaling.

- Example C: Non-360: 4K resolution:  3x3 subpictures each with 720p resolution, using CtbSizeY=128

- Example D: Non-360: 5K resolution: 4x4 subpictures each with 720p resolution, using CtbSizeY=128

# Proposals

It is proposed that cropping information be optionally signaled on subpicture basis.

A syntax modification is proposed for this.

The proposed changes are shown highlighted compared with VVC WD.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| subpics_cropping_info_present_flag | u(1) |
| if( subpics_cropping_info_present_flag ) { |  |
| for( i = 0; i < NumSubPics; i++ ) { |  |
| subpic_conformance_window_flag[ i ] | u(1) |
| if( subpic_conformance_window_flag ) { |  |
| subpic_conf_win_left_offset[ i ] | ue(v) |
| subpic_conf_win_right_offset[ i ] | ue(v) |
| subpic_conf_win_top_offset[ i ] | ue(v) |
| subpic_conf_win_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

subpics_cropping_info_present_flag equal to 1 indicates that the conformance cropping window offset parameters are present for subpictures and follow next in the SPS. subpics_cropping_info_present_flag equal to 0 indicates that the explicit conformance cropping window offset parameters are not present for subpictures.

subpic_conformance_window_flag[ i ] equal to 1 indicates that the conformance cropping window offset parameters for i-th subpicture for each coded picture in the CVS follow next in the SPS. subpic_conformance_window_flag[ i ] equal to 0 indicates that the conformance cropping window offset parameters are not present for i-th subpicture for each coded picture in the CVS and cropping parameters if present for the overall picture may apply to the subpicture.

subpic_conf_win_left_offset[ i ], subpic_conf_win_right_offset[ i ], subpic_conf_win_top_offset[ i ], and subpic_conf_win_bottom_offset[ i ]  specify the samples of the i-th subpicture for each coded picture in the CVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When subpic_conformance_window_flag[ i ]  is equal to 0, the values of subpic_conf_win_left_offset[ i ]  , subpic_conf_win_right_offset[ i ]  , subpic_conf_win_top_offset[ i ]  , and subpic_conf_win_bottom_offset[ i ]  are inferred to be equal to 0.

The conformance cropping window for i-th subpicture for each coded picture in the CVS contains the luma samples with horizontal picture coordinates from SubWidthC * subpic_conf_win_left_offset[ i ]  to SubPicWidth[ i ] − ( SubWidthC * subpic_conf_win_right_offset[ i ] + 1 ) and vertical picture coordinates from SubHeightC * subpic_conf_win_top_offset[ i ]  to SubPicHeight[ i ] − ( SubHeightC * subpic_conf_win_bottom_offset[ i ] + 1 ), inclusive.

The value of SubWidthC * ( subpic_conf_win_left_offset[ i ] + subpic_conf_win_right_offset[ i ] ) shall be less than SubPicWidth[ i ] , and the value of SubHeightC * ( subpic_conf_win_top_offset[ i ] + subpic_conf_win_bottom_offset[ i ] ) shall be less than SubPicHeight[ i ] .

The variables SubPicOutputWidthL[ i ] and SubPicOutputHeightL[ i ] are derived as follows:

SubPicOutputWidthL[ i ] = SubPicWidth[ i ]  − 	
			 SubWidthC * ( subpic_conf_win_right_offset[ i ]  + subpic_conf_win_left_offset[ i ]  )

SubPicOutputHeightL[ i ] = SubPicHeight[ i ] −				SubHeightC * ( subpic_conf_win_bottom_offset[ i ]  + subpic_conf_win_top_offset[ i ]  )

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE – The subpicture conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

Alternatively the above parameters can be signalled in PPS.

C.3.3 Picture output

…

The output of the current picture is specified as follows:

–	If PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to CpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PictureOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause C.3.4.

–	Otherwise (PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than CpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes DpbOutputTime[ n ].

When output, the picture is cropped, using the conformance cropping window information specified in the SPS or PPS for the picture and/ or for the subpictures.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextPicInOutputOrder ] − DpbOutputTime[ n ]	(C-16)

where nextPicInOutputOrder is the picture that follows picture n in output order and has PictureOutputFlag equal to 1.

C.5.2.4 "Bumping" process

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped, using the conformance cropping window information specified in the SPS or PPS for the picture and/ or for the subpictures, the cropped picture is output, and the picture is marked as "not needed for output".

- When the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

NOTE – For any two pictures picA and picB that belong to the same CVS and are output by the "bumping process", when picA is output earlier than picB, the value of PicOrderCntVal of picA is less than the value of PicOrderCntVal of picB.

Regarding picture cropping information:

conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the SPS. conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present. It is requirement of bitstream conformance that if subpics_cropping_info_present_flag is equal to 1, conformance_window_flag shall be equal to 0.

# Conclusion

This document proposes optionally signaling subpicture cropping information. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019