| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0585 |
| --- | --- |

| Title: | CE7-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to align the number of context coded bins per coefficient for transform skip residual coding and transform coefficient coding by assigning 1.75 bin/coefficient at TB-level.  The simulations results show Class F:0.11% AI, 0.07% RA, 0.05% LB BD-Rate, and Class TGM: 0.26% AI, -0.20% RA, -0.02% LB BD-Rate, versus VTM-6.0 anchor under common test conditions.

# Proposal

VVC (Draft 6) limits the number of context coded bins per coefficient for luma and chroma coefficients to be 1.75 bin/coefficient at TB-level [1]. Current transform skip (TS) residual coding sets this limit at TB-level to 2 bin/coefficient. It is proposed to align the number of context coded bins per coefficient at TB level for transform coefficient and TS residual coding schemes by setting the limit to 1.75 bin/coefficient as proposed in JVET-O1169[2].

The proposed VVC specification[1] change is highlighted below.

#### 7.3.8.11 Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 (7 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight )) >> 2 |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |

# Results

The proposed scheme is implemented on top of VTM-6.0 software [3]. The simulations were carried out under the JVET common test conditions [3].

The test results for proposed scheme is shown in Table 1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 99% | 99% |
| Class B | 0.00% | 0.02% | 0.00% | 101% | 101% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 95% |
| Class E | 0.00% | 0.10% | -0.01% | 100% | 103% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.01% | 0.00% | -0.01% | 102% | 107% |
| Class F | 0.11% | 0.10% | 0.15% | 101% | 105% |
| Class TGM | 0.26% | 0.22% | 0.23% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 100% | 100% |
| Class A2 | 0.02% | 0.14% | 0.08% | 101% | 101% |
| Class B | 0.01% | 0.10% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 100% | 100% |
| Class D | -0.03% | -0.48% | 0.02% | 101% | 102% |
| Class F | 0.07% | 0.10% | 0.17% | 100% | 101% |
| Class TGM | 0.20% | 0.14% | 0.13% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.04% | 100% | 101% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 100% |
| Class E | -0.09% | 0.46% | 0.57% | 101% | 99% |
| Overall | -0.03% | 0.15% | 0.10% | 100% | 100% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 101% |
| Class F | 0.05% | 0.29% | 0.62% | 100% | 100% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100% | 103% |

Table 1: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-6.0 anchor for CTC.

Low QP results, QP={2,7,12,17} are tabulated below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 99% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 101% |
| Class E | 0.05% | 0.00% | 0.02% | 100% | 100% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class F | 0.30% | 0.14% | 0.14% | 100% | 100% |
| Class TGM | 0.46% | 0.42% | 0.41% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.03% | 0.04% | 100% | 100% |
| Class F | 0.38% | 0.00% | 0.12% | 101% | 102% |
| Class TGM | 0.49% | 0.40% | 0.40% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.02% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.02% | 0.03% | -0.02% | 102% | 103% |
| Class F | 0.03% | 0.09% | 0.05% | 101% | 100% |
| Class TGM | 0.41% | 0.32% | 0.42% | 101% | 100% |

Table 2: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-6.0 anchor for low QP conditions.

Bin to bit ratio analysis is conducted according to CE7 testing conditions [5]. For CTC conditions, the bin to bit ratio analysis is provided in Table 3.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | -0.02% | 1.19 |  |  | 1.19 |  | 0.20% | 1.15 | 1.16 | 0.26% | 1.16 | 1.16 | -0.01% | 1.20 | 1.20 | -0.01% | 99% | 99% |
| Class A2 | 0.00% |  | -0.02% |  | 0.03% | 1.18 |  |  | 1.18 |  | -0.02% | 1.16 | 1.16 | -0.01% | 1.16 | 1.16 | -0.01% | 1.18 | 1.18 | -0.01% | 99% | 99% |
| Class B | 0.00% |  | 0.02% |  | 0.00% | 1.15 |  |  | 1.15 |  | 0.00% | 1.13 | 1.13 | -0.04% | 1.13 | 1.13 | -0.01% | 1.16 | 1.16 | 0.00% | 101% | 101% |
| Class C | 0.02% |  | -0.03% |  | 0.04% | 1.08 |  |  | 1.08 |  | -0.34% | 1.09 | 1.08 | -0.42% | 1.09 | 1.09 | -0.15% | 1.08 | 1.08 | -0.07% | 100% | 95% |
| Class E | 0.00% |  | 0.10% |  | -0.01% | 1.11 |  |  | 1.11 |  | 0.11% | 1.10 | 1.10 | 0.03% | 1.10 | 1.10 | -0.04% | 1.11 | 1.11 | -0.02% | 100% | 103% |
| Overall | 0.00% |  | 0.01% |  | 0.01% | 1.14 |  |  | 1.14 |  | -0.04% | 1.13 | 1.12 | -0.07% | 1.13 | 1.13 | -0.05% | 1.14 | 1.14 | -0.02% | 100% | 99% |
| Class D | 0.01% |  | 0.00% |  | -0.01% | 1.06 |  |  | 1.05 |  | -0.21% | 1.07 | 1.07 | -0.32% | 1.08 | 1.08 | -0.18% | 1.06 | 1.06 | -0.11% | 102% | 107% |
| Class F | 0.11% |  | 0.10% |  | 0.15% | 1.16 |  |  | 1.16 |  | -0.51% | 1.17 | 1.16 | -0.92% | 1.18 | 1.17 | -0.91% | 1.17 | 1.17 | -0.59% | 101% | 105% |
| Class TGM | 0.26% |  | 0.22% |  | 0.23% | 1.20 |  |  | 1.19 |  | -0.69% | 1.33 | 1.31 | -1.43% | 1.33 | 1.31 | -1.49% | 1.20 | 1.19 | -1.07% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | 0.03% | 1.18 |  | 1.18 |  | -0.03% |  | 1.14 | 1.14 | -0.08% | 1.18 | 1.18 | -0.01% | 1.23 | 1.23 | -0.01% | 100% | 100% |
| Class A2 | 0.02% |  | 0.14% |  | 0.08% | 1.17 |  | 1.17 |  | 0.04% |  | 1.14 | 1.14 | 0.04% | 1.21 | 1.21 | 0.00% | 1.23 | 1.23 | -0.01% | 101% | 101% |
| Class B | 0.01% |  | 0.10% |  | 0.01% | 1.14 |  | 1.14 |  | -0.06% |  | 1.10 | 1.10 | -0.07% | 1.17 | 1.17 | -0.01% | 1.20 | 1.20 | 0.00% | 100% | 100% |
| Class C | 0.00% |  | 0.06% |  | 0.03% | 1.05 |  | 1.05 |  | -0.03% |  | 1.03 | 1.03 | -0.17% | 1.13 | 1.13 | -0.06% | 1.12 | 1.11 | -0.04% | 101% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% |  | 0.08% |  | 0.03% | 1.13 |  | 1.13 |  | -0.03% |  | 1.10 | 1.10 | -0.08% | 1.17 | 1.17 | -0.02% | 1.19 | 1.19 | -0.01% | 100% | 100% |
| Class D | -0.03% |  | -0.48% |  | 0.02% | 1.02 |  | 1.02 |  | -0.16% |  | 1.01 | 1.00 | -0.17% | 1.14 | 1.14 | -0.20% | 1.09 | 1.09 | -0.12% | 101% | 102% |
| Class F | 0.07% |  | 0.10% |  | 0.17% | 1.11 |  | 1.11 |  | -0.66% |  | 1.08 | 1.07 | -0.92% | 1.16 | 1.15 | -0.83% | 1.18 | 1.17 | -0.53% | 100% | 101% |
| Class TGM | 0.20% |  | 0.14% |  | 0.13% | 1.15 |  | 1.13 |  | -1.28% |  | 1.22 | 1.20 | -1.79% | 1.36 | 1.34 | -1.21% | 1.24 | 1.23 | -0.71% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.11% |  | 0.04% | 1.18 | 1.18 |  |  | -0.16% |  | 1.16 | 1.16 | -0.20% | 1.29 | 1.29 | 0.02% | 1.26 | 1.26 | 0.01% | 100% | 101% |
| Class C | -0.03% |  | -0.04% |  | -0.20% | 1.09 | 1.09 |  |  | 0.16% |  | 1.11 | 1.11 | -0.08% | 1.27 | 1.27 | -0.04% | 1.17 | 1.17 | -0.02% | 100% | 100% |
| Class E | -0.09% |  | 0.46% |  | 0.57% | 1.11 | 1.11 |  |  | 0.05% |  | 1.11 | 1.11 | -0.01% | 1.36 | 1.36 | 0.02% | 1.26 | 1.26 | -0.01% | 101% | 99% |
| Overall | -0.03% |  | 0.15% |  | 0.10% | 1.13 | 1.13 |  |  | -0.02% |  | 1.13 | 1.13 | -0.14% | 1.30 | 1.30 | 0.01% | 1.23 | 1.23 | 0.00% | 100% | 100% |
| Class D | 0.03% |  | 0.31% |  | 0.80% | 1.05 | 1.05 |  |  | -0.17% |  | 1.08 | 1.08 | -0.08% | 1.31 | 1.31 | 0.09% | 1.16 | 1.16 | 0.02% | 100% | 101% |
| Class F | 0.05% |  | 0.29% |  | 0.62% | 1.19 | 1.18 |  |  | -0.69% |  | 1.21 | 1.20 | -0.92% | 1.30 | 1.29 | -0.41% | 1.27 | 1.27 | -0.23% | 100% | 100% |
| Class TGM | -0.02% |  | -0.03% |  | -0.03% | 1.19 | 1.18 |  |  | -0.87% |  | 1.29 | 1.27 | -1.40% | 1.49 | 1.48 | -0.60% | 1.33 | 1.32 | -0.37% | 100% | 103% |

Table 3.  Summary of CABAC bin to bit results for under CTCs.

For low QP conditions, the bin to bit ratio analysis is provided in Table 4.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.02% |  | 0.04% |  | 0.05% | 1.02 |  |  | 1.02 |  | -0.12% | 0.95 | 0.95 | -0.14% | 0.96 | 0.96 | -0.18% | 1.02 | 1.02 | -0.14% | 101% | 100% |
| Class A2 | 0.00% |  | 0.00% |  | 0.00% | 0.90 |  |  | 0.90 |  | 0.00% | 0.84 | 0.84 | -0.01% | 0.84 | 0.84 | 0.00% | 0.91 | 0.91 | 0.00% | 100% | 98% |
| Class B | 0.01% |  | -0.01% |  | 0.00% | 0.93 |  |  | 0.93 |  | -0.05% | 0.86 | 0.86 | -0.08% | 0.87 | 0.87 | -0.07% | 0.94 | 0.94 | -0.06% | 100% | 99% |
| Class C | 0.01% |  | 0.00% |  | 0.01% | 0.83 |  |  | 0.82 |  | -0.06% | 0.74 | 0.74 | -0.09% | 0.75 | 0.75 | -0.13% | 0.83 | 0.83 | -0.08% | 100% | 101% |
| Class E | 0.05% |  | 0.00% |  | 0.02% | 1.01 |  |  | 1.00 |  | -0.33% | 0.94 | 0.94 | -0.44% | 0.94 | 0.94 | -0.45% | 1.01 | 1.01 | -0.35% | 100% | 100% |
| Overall | 0.02% |  | 0.01% |  | 0.01% | 0.93 |  |  | 0.93 |  | -0.10% | 0.86 | 0.86 | -0.14% | 0.87 | 0.87 | -0.15% | 0.94 | 0.94 | -0.11% | 100% | 99% |
| Class D | 0.00% |  | -0.01% |  | 0.01% | 0.80 |  |  | 0.80 |  | -0.13% | 0.71 | 0.71 | -0.12% | 0.73 | 0.72 | -0.09% | 0.81 | 0.81 | -0.05% | 100% | 100% |
| Class F | 0.30% |  | 0.14% |  | 0.14% | 0.96 |  |  | 0.94 |  | -1.24% | 0.87 | 0.86 | -1.66% | 0.85 | 0.84 | -1.57% | 0.94 | 0.93 | -1.17% | 100% | 100% |
| Class TGM | 0.46% |  | 0.42% |  | 0.41% | 0.87 |  |  | 0.85 |  | -1.62% | 0.80 | 0.79 | -2.32% | 0.80 | 0.79 | -2.24% | 0.87 | 0.85 | -1.62% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.02% |  | 0.00% |  | 0.01% | 0.96 |  | 0.96 |  | -0.16% |  | 0.89 | 0.89 | -0.19% | 1.10 | 1.10 | -0.11% | 1.15 | 1.15 | -0.09% | 100% | 101% |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.00% |  | 0.00% | 0.87 |  | 0.87 |  | -0.11% |  | 0.80 | 0.80 | -0.14% | 1.04 | 1.04 | -0.04% | 1.12 | 1.12 | -0.03% | 101% | 101% |
| Class C | 0.00% |  | -0.01% |  | 0.01% | 0.77 |  | 0.77 |  | -0.08% |  | 0.68 | 0.68 | -0.12% | 0.97 | 0.97 | -0.03% | 1.04 | 1.04 | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! | 0.86 |  | #DIV/0! |  | #DIV/0! |  | 0.78 | #DIV/0! | #DIV/0! | 1.04 | #DIV/0! | #DIV/0! | 1.10 | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
| Class D | 0.00% |  | -0.03% |  | 0.04% | 0.75 |  | 0.75 |  | -0.08% |  | 0.66 | 0.66 | -0.11% | 0.95 | 0.94 | -0.15% | 1.02 | 1.02 | -0.10% | 100% | 100% |
| Class F | 0.38% |  | 0.00% |  | 0.12% | 0.90 |  | 0.89 |  | -1.44% |  | 0.81 | 0.80 | -1.89% | 0.95 | 0.94 | -1.21% | 1.02 | 1.02 | -0.87% | 101% | 102% |
| Class TGM | 0.49% |  | 0.40% |  | 0.40% | 0.84 |  | 0.82 |  | -1.70% |  | 0.77 | 0.75 | -2.17% | 0.87 | 0.85 | -2.36% | 0.95 | 0.93 | -1.62% | 102% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.00% |  | 0.01% | 0.93 | 0.93 |  |  | -0.07% |  | 0.86 | 0.86 | -0.06% | 1.03 | 1.03 | -0.03% | 1.10 | 1.10 | -0.03% | 101% | 101% |
| Class C | 0.00% |  | 0.00% |  | -0.02% | 0.82 | 0.82 |  |  | -0.13% |  | 0.73 | 0.73 | -0.18% | 0.98 | 0.98 | -0.05% | 1.05 | 1.05 | -0.04% | 100% | 99% |
| Class E | 0.03% |  | -0.02% |  | 0.02% | 1.01 | 1.01 |  |  | -0.47% |  | 0.94 | 0.94 | -0.64% | 1.18 | 1.18 | -0.17% | 1.24 | 1.24 | -0.12% | 100% | 101% |
| Overall | 0.00% |  | 0.00% |  | 0.00% | 0.92 | 0.92 |  |  | -0.16% |  | 0.84 | 0.84 | -0.21% | 1.06 | 1.06 | -0.07% | 1.12 | 1.12 | -0.05% | 100% | 100% |
| Class D | -0.02% |  | 0.03% |  | -0.02% | 0.79 | 0.79 |  |  | -0.03% |  | 0.71 | 0.71 | -0.13% | 0.97 | 0.97 | -0.10% | 1.04 | 1.04 | -0.07% | 102% | 103% |
| Class F | 0.03% |  | 0.09% |  | 0.05% | 0.92 | 0.91 |  |  | -1.21% |  | 0.84 | 0.83 | -1.67% | 1.03 | 1.02 | -1.00% | 1.10 | 1.09 | -0.72% | 101% | 100% |
| Class TGM | 0.41% |  | 0.32% |  | 0.42% | 0.85 | 0.84 |  |  | -1.57% |  | 0.79 | 0.77 | -2.23% | 0.91 | 0.89 | -2.26% | 1.00 | 0.98 | -1.50% | 101% | 100% |

Table 3.  Summary of CABAC bin to bit results for under low QP conditions.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

- M. Coban, M. Karczewicz, “CE8(7)-related: Alignment of number of context coded bins per coefficients for TS residual coding and transform coefficient coding”, JVET-O1169, Gothenburg, SE, July 2019.

- https://vcgit.hhi.fraunhofer.de/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, March 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).