| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0590 |
| --- | --- |

| Title: | AHG8: Scaling window for scaling ratio derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Adarsh K. Ramasubramonian Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, reference picture resampling and scalability support were adopted, where scaling ratio is derived based on the conformance cropping window and picture size. In this document, it is proposed to decouple scaling ratio derivation from the conformance cropping window and to signal a scaling window in PPS to derive scaling ratio.

# Introduction and proposal

In VVC draft 6, scaling ratio is derived as follows

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

where the variables PicOutputWidthL and PicOutputHeightL are derived based on the picture size and confromance cropping window.

PicOutputWidthL = pic_width_in_luma_samples − 	(7-43)
			 SubWidthC * ( conf_win_right_offset + conf_win_left_offset )

PicOutputHeightL = pic_height_in_pic_size_units −			SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )	(7-44)

Conformance window may be used to display a region of interest or to crop the extended picture boundaries coming from the restriction of the input picture size to be multiple of 8 samples. Coupling picture output and scaling ratio may lead to undesirable scaling ratio or undesirable picture output. Also, the same scaling ratio method is used for scalability scaling ratio derivation, where multiple layers are involved, coupling scaling ratio derivation with picture output in that layers may lead to the similar problems.

In this contribution, it is proposed to decouple conformance cropping window used in the picture output process from the scaling ratio derivation and to signal a scaling window in PPS to be used for scaling ratio derivation.

PicOutputWidthL and PicOutputHeightL are calculated based on picutre width and picture height adjusted by sclaing window offsets. This claculation is perfromed for a current and a reference pictures.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| scaling_window_flag | u(1) |
| if( scaling_window_flag ) { |  |
| scaling_win_left_offset | ue(v) |
| scaling_win_right_offset | ue(v) |
| scaling_win_top_offset | ue(v) |
| scaling_win_bottom_offset | ue(v) |
| } |  |
| … |  |

scaling_window_flag equal to 1 indicates that the scaling window offset parameters follow next in the PPS. scaling_window_flag equal to 0 indicates that the scaling window offset parameters are not present.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets in luma samples applied to the picture size for scaling ratio calculation. When sclaing_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −			 SubWidthC * ( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −			SubHeightC * ( scaling_win_bottom_offset + scaling_win_top_offset )

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).