| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0590-v3 |
| --- | --- |

| Title: | AHG8: Scaling window for scaling ratio derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Adarsh K. Ramasubramonian Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, reference picture resampling and scalability support were adopted, where scaling ratio is derived based on the conformance cropping window and picture size. In this document, it is proposed to decouple scaling ratio derivation from the conformance cropping window and to signal a scaling window in PPS to derive scaling ratio.

In the third revision of the document, the specification draft is attached reflecting the adopted scaling window (highlighted in yellow in the draft) with the additional changes discussed and agreed during the proposal review:

- Add an SPS-level reference_picture_resampling_enabled_flag (which was actually part of the prior proposal JVET-O0133), and constrain the picture width and height to be equal to the max values and constrain the presence flag for the new parameters to be 0 when that flag is 0.

- Replace the picture size check with the check of the scaling ratio to constrain for BDOF and DMVR

- Add a scaling ratio check for TMVP constraint

# Introduction and proposal

In VVC draft 6, scaling ratio is derived as follows

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

where the variables PicOutputWidthL and PicOutputHeightL are derived based on the picture size and confromance cropping window.

PicOutputWidthL = pic_width_in_luma_samples − 	(7-43)
			 SubWidthC * ( conf_win_right_offset + conf_win_left_offset )

PicOutputHeightL = pic_height_in_pic_size_units −			SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )	(7-44)

Conformance window may be used to display a region of interest or to crop the extended picture boundaries coming from the restriction of the input picture size to be multiple of 8 samples. Coupling picture output and scaling ratio may lead to undesirable scaling ratio or undesirable picture output. Also, the same scaling ratio method is used for scalability scaling ratio derivation, where multiple layers are involved, coupling scaling ratio derivation with picture output in that layers may lead to the similar problems.

In this contribution, it is proposed to decouple conformance cropping window used in the picture output process from the scaling ratio derivation and to signal a scaling window in PPS to be used for scaling ratio derivation.

PicOutputWidthL and PicOutputHeightL are calculated based on picutre width and picture height adjusted by sclaing window offsets. This claculation is perfromed for a current and a reference pictures.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| scaling_window_flag | u(1) |
| if( scaling_window_flag ) { |  |
| scaling_win_left_offset | ue(v) |
| scaling_win_right_offset | ue(v) |
| scaling_win_top_offset | ue(v) |
| scaling_win_bottom_offset | ue(v) |
| } |  |
| … |  |

scaling_window_flag equal to 1 indicates that the scaling window offset parameters follow next in the PPS. scaling_window_flag equal to 0 indicates that the scaling window offset parameters are not present.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets in luma samples applied to the picture size for scaling ratio calculation. When sclaing_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −			 SubWidthC * ( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −			SubHeightC * ( scaling_win_bottom_offset + scaling_win_top_offset )

The left and top scaling window offsets can be used to specify the offsets within the current and a reference picture, similar method is proposed in JVET-P0381 using conformance cropping window.

##### 8.5.6.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the half sample interpolation filter index hpelIfIdx,

- the bi-directional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth + brdExtSize)x(sbHeight + brdExtSize) array predSamplesLX of prediction sample values.

The prediction block border extension size brdExtSize is derived as follows:

brdExtSize = ( bdofFlag | | ( inter_affine_flag[ xSb ][ ySb ] && sps_affine_prof_enabled_flag ) ) ? 2 : 0	(8-752)

The variable fRefWidth is set equal to the PicOutputWidthL of the reference picture in luma samples.

The variable fRefHeight is set equal to PicOutputHeightL of the reference picture in luma samples.

The variable fRefLeftOffset is set equal to the scaling_win_left_offset << 10 of the reference picture in luma samples.

The variable fRefTopOffset is set equal to the scaling_win_top_offset << 10 of the reference picture in luma samples.

The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

- If cIdx is equal to 0, the following applies:

  - The scaling factors and their fixed-point representations are defined as

hori_scale_fp = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL >>  1 ) ) / PicOutputWidthL	(8-753)

vert_scale_fp = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL >>  1 ) ) / PicOutputHeightL	(8-754)

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntL, ySbIntL ) is set equal to ( xSb + ( mvLX[ 0 ]  >>  4 ), ySb + ( mvLX[ 1 ]  >>  4 ) ).

  - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

refxSbL = ( ( ( xSb – scaling_win_left_offset )  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + fRefLeftOffset + 32 )  >>  6	(8-756)

refySbL = ( ( ( ySb – scaling_win_top_offset )  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) + fRefTopOffset + 32 )  >>  6	(8-758)

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = refxL  >>  4		(8-759)

yIntL = refyL  >>  4		(8-760)

xFracL = refxL & 15		(8-761)

yFracL = refyL & 15		(8-762)

  - If bdofFlag is equal to TRUE or ( sps_affine_prof_enabled_flag is equal to TRUE and inter_affine_flag[ xSb ][ ySb ] is equal to TRUE ), and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL + ( xFracL >> 3) − 1), yIntL + ( yFracL >> 3 ) − 1 ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the  prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − ( brdExtSize > 0 ? 1 : 0 ), yIntL − ( brdExtSize > 0 ? 1 : 0 ) ), ( xFracL, yFracL ), ( xSbIntL, ySbIntL ), refPicLX, hpelIfIdx, sbWidth, sbHeight and ( xSb, ySb ) as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - The top-left coordinate of the bounding block for reference sample padding ( xSbIntC, ySbIntC ) is set equal to ( (xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5), ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) ).

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- Let (refxSbC, refySbC) and ( refxC, refyC ) be chroma locations pointed to by a motion vector (mvLX[ 0 ], mvLX[ 1 ]) given in 1/32-sample units. The variables refxSbC, refySbC, refxC and refyC are derived as follows:

refxSbC =  ( ( ( xSb – scaling_win_left_offset ) / SubWidthC  <<  5 ) + mvLX[ 0 ] ) * hori_scale_fp	 (8-763)

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 )
		+ xC * ( ( hori_scale_fp + 8 )  >>  4 ) ) + fRefLeftOffset / SubWidthC  + 16 )  >>  5	(8-764)

refySbC =  ( ( ( ySb – scaling_win_top_offset ) / SubHeightC  <<  5 )  + mvLX[ 1 ] ) * vert_scale_fp	 (8-765)

refyC = ( ( Sign( refySbC ) * ( ( Abs( refySbC ) + 256 )  >>  9 )
		+ yC* ( ( vert_scale_fp + 8 )  >>  4 ) ) + fRefTopOffset / SubHeightC + 16 )  >>  5	(8-766)

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows:

xIntC = refxC  >>  5		(8-767)

yIntC = refyC  >>  5		(8-768)

xFracC = refyC & 31		(8-769)

yFracC = refyC & 31		(8-770)

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), ( xSbIntC, ySbIntC ), sbWidth, sbHeight and refPicLX as inputs.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).