| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0593 |
| --- | --- |

| Title: | AHG8/CE1-related: Signalling filter coefficients for resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC draft 6, reference picture resampling process uses existed motion compensation filters. In CE1, depending on the scaling ratio alternative fixed filter coefficients for reference picture downsampling are tested.

In this contribution, it is proposed to signal filter coefficients in APS for luma component that are used for downsampling. In slice header, an APS index is signalled to indicate which filter coefficients set is applied for downsampling. This allows to specify the desired coefficients per picture, which can be scaling ratio dependent.

# Introduction and proposal

In VVC draft 6, reference picture resampling process uses existed motion compensation filters. In CE1, multiple tests are performed with fixed alternative filter coefficients introduced for reference picture downsampling depending on the scaling ratio.

In this contribution, instead of using multiple sets of the fixed filters switched based on the scaling ratio, it is proposed to signal filter coefficients in APS for luma component that are used for downsampling. In slice header, an APS index is signalled to indicate which filter coefficient set is applied for downsampling. This allows to specify the desired coefficients per picture, which can be scaling ratio dependent.

A new APS type FILTER_APS is introduced, and it follows the same process that already exists for ALF, LMCS, and scaling matrices APS types.

To estimate the filter overhead, the proposed filter coefficients signaling was applied to VTM luma motion compensation filters. It takes 320 bits to signal one APS filter set.

A flag is signaled in SPS to indicate that the signalled filters are used.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alternative_filters_enabled_flag | u(1) |
| … |  |

sps_alternative_filters_enabled_flag equal to 1 specifies that alternative filters are signaled in the CVS. sps_alternative_fitlers_enabled_flag equal to 0 specifies that alternative filters are not signaled and are not used in the CVS.

A flag is signalled to specify whether the signalled filters indicated by an APS index are applied for downsampling and whether the signalled filters are used for regular motion compensation.

| slice_header( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if( sps_alternative_filters_enabled_flag ) { |  |
| slice_alternative_filters_enabled_flag | u(1) |
| if(slice_alternative_filters_enabled_flag) { |  |
| slice_alternative_filters_used_for_motion_compensation_flag | u(1) |
| slice_filter_aps_id | u(3) |
| } |  |
| } |  |
| … |  |

slice_alternative_filters_enabled_flag equal to 1 specifies that alternative filters are used for reference picture downsampling for the current slice. slice_alternative_filters_enabled_flag equal to 0 specifies that the alternative filters are not used for the current slice downsampling for the current slice. When slice_alternative_filters_enabled_flag is not present, it is inferred to be equal to 0.

slice_alternative_filters_used_for_motion_compensation_flag equal to 1 specifies that alternative filters are used for regular motion compensation interpolation (scaling ratio is 1) for the current slice. slice_alternative_filters_used_for_motion_compensation_flag equal to 0 specifies that the alternative filters are not used for the regular motion compensation interpolation (scaling ratio is 1) for the current slice. When slice_alternative_filters_used_for_motion_compensation_flag is not present, it is inferred to be equal to 0.

slice_filter_aps_id specifies the adaptation_parameter_set_id of the FILTER APS that the slice refers to.

A new APS type FILTER_APS is introduced which carries luma filter coefficients.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = FILTER_APS ) |  |
| filter_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS or FILTER_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 7-2. When aps_params_type is equal to 1 (LMCS_APS), the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Table 7-2 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | FILTER_APS | Filter parameters |
| 4..7 | Reserved | Reserved |

When filter coefficients are signalled, it is assumed that the filter norm is 64, luma filter length is 8 with 16 fraction positions. Since the filter norm is known, one coefficient is derived from the norm and the sum of the signalled coefficietns, filter symmetry is considered and only unique coefficients are signalled.

The ALF coefficient signaling is reused, e.g Exp-Golomb code of order 3 is used to signal absolute value of a filter coefficient and one bit is spent for coefficient sign.

| filter_data( ) { | Descriptor |
| --- | --- |
| for( fIdx = 0; fIdx < 8; fIdx++ ) |  |
| for ( j = 0; j < 8; j++ ) |  |
| if( j ! = 3) { |  |
| luma_coeff_abs[ fIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ fIdx ][ j ] ) |  |
| luma_coeff_sign[ fIdx ][ j ] | u(1) |
| } |  |
| for ( j = 0; j < 4; j++ ) { |  |
| luma_coeff_abs[ 8 ][ j ] | uek(v) |
| if( luma_coeff_abs[ 8 ][ j ] ) |  |
| luma_coeff_sign[ 8 ][ j ] | u(1) |
| } |  |
| } |  |

luma_coeff_abs[ fIdx ][ j ] specifies the absolute value of the j-th coefficient of the signalled luma filter indicated by fIdx. When luma_coeff_abs[ fIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is set equal to 3.

luma_coeff_sign[ fIdx ][ j ] specifies the sign of the j-th luma coefficient of the filter indicated by fIdx as follows:

- If luma_coeff_sign[ fIdx ][ j ] is equal to 0, the corresponding luma filter coefficient has a positive value.

- Otherwise (luma_coeff_sign[ fIdx ][ j ] is equal to 1), the corresponding luma filter coefficient has a negative value.

The variable LumaFiltCoeff[ fIdx ][ j ] with fIdx = 0..15, j = 0..7 is initialized as follows:

skipCoeffIdx = 3, norm = 64

LumaFiltCoeff[ 0 ][ j ] = 0

LumaFiltCoeff[ 0 ][ skipCoeffIdx ] = norm

for( fIdx = 0; fIdx  <  8; fIdx++ ) {

sum = 0

for ( j = 0; j < 8; j++ )

if( j  ! =  skipCoeffIdx) {

LumaFiltCoeff[fIdx][j] = luma_coeff_abs[fIdx][j] *( 1 – 2* luma_coeff_sign[fIdx][j] )

sum += LumaFiltCoeff[fIdx][j]

}

LumaFiltCoeff[fIdx][ skipCoeffIdx ] = norm – sum

}

sum = 0

for ( j = 0; j < 3; j++ ) {

LumaFiltCoeff[8][j] = luma_coeff_abs[8][j] * ( 1 – 2 * luma_coeff_sign[8][j] )

sum += LumaFiltCoeff[8][j]

}

LumaFiltCoeff[8][ skipCoeffIdx ] = norm / 2 – sum

for ( j = 4; j < 8; j++ )

LumaFiltCoeff[8][j] = LumaFiltCoeff[8][7 – j]

for( fIdx = 9; fIdx  <  16; fIdx++ )

for ( j = 0; j < 8; j++ )

LumaFiltCoeff[fIdx][j] = LumaFiltCoeff[16-fIdx][7 – j]

Then, LumaFiltCoeff is used for motion compensation.

        - 8.5.6.3.2 Luma sample interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- a luma location in full-sample units ( xSbIntL, ySbIntL ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left luma sample of the reference picture,

- the luma reference sample array refPicLXL,

- the half sample interpolation filter index hpelIfIdx,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture,

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

- The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

- The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fL[ p ] are specified in Table 8-12.

- Otherwise, if slice_alternative_filters_used_for_motion_compensation_flag is equal to 1, or hori_scale_fp or vert_scale_fp is not equal to (1<<14), i.e. scaling ratio is not equal to 1x

  - fL is set equal to LumaFiltCoeff

- Otherwise, the luma interpolation filter coefficients fL[ p ] are specified in Table 8-11 depending on hpelIfIdx.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 )	(8-771)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )	(8-772)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(8-773) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-774)

The luma locations in full-sample units are further modified as follows for i = 0..7:

xInti = Clip3( xSbIntL − 3, xSbIntL + sbWidth + 4, xInti )	(8-775)

yInti = Clip3( ySbIntL − 3, ySbIntL + sbHeight + 4, yInti )	(8-776)

The predicted luma sample value predSampleLXL is derived as follows:

- If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-777)

- Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-778)

- Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-779)

- Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fL[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-780)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fL[ yFracL ][ i ] * temp[ i ]   >>  shift2	(8-781)

Table 8-11 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 (hpelIfIdx = = 0) | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 8 (hpelIfIdx = = 1) | 0 | 3 | 9 | 20 | 20 | 9 | 3 | 0 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

Table 8-12 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p for affine motion mode.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | 0 | 1 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | 0 | 2 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | 0 | 3 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | 0 | 3 | −11 | 52 | 26 | −8 | 2 | 0 |
| 6 | 0 | 2 | −9 | 47 | 31 | −10 | 3 | 0 |
| 7 | 0 | 3 | −11 | 45 | 34 | −10 | 3 | 0 |
| 8 | 0 | 3 | −11 | 40 | 40 | −11 | 3 | 0 |
| 9 | 0 | 3 | −10 | 34 | 45 | −11 | 3 | 0 |
| 10 | 0 | 3 | −10 | 31 | 47 | −9 | 2 | 0 |
| 11 | 0 | 2 | −8 | 26 | 52 | −11 | 3 | 0 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 3 | 0 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 2 | 0 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 1 | 0 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).