| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0594-v1 |
| --- | --- |

| Title: | Crosscheck of CE6-2.2a and CE6-2.2d: An Orthogonality Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck report |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this cross-check report, the orthogonality of 4x4, 8x4 and 4x8 LFNSTs proposed in CE6-2.2a and 8x8 LFNSTs in CE6-2.3a (used in CE6-2.2d test) are analyzed. In the analysis, an SNR metric is used to evaluate orthogonality, and SNR results for LFNSTs in VTM-6.0 are used as benchmarks.

# Analysis of Orthogonality of LFNST matrices

To evaluate orthogonality, the following formula is used for a given NxM transform matrix T

SNR(T) = 20log10(1/MSE(T))                                               (1)

MSE(T) =|| TTT - I ||F2N2

where the basis vectors are in the rows of T,  TT is its transpose, I is the NxN identity matrix, and || ⋅ ||F2 denotes the Frobenius norm.

SNR values are calculated for the following set of LFNST matrices (T):

- The set of eight 4x4 LFNST matrices in VTM-6.0 (VTM-6.0: 4x4)

- The set of eight 8x8 LFNST matrices in VTM-6.0 (VTM-6.0: 8x8)

- The set of eight 8x8 LFNST matrices in CE6-2.3a (CE6-2.3a: 8x8)

SNR values are also calculated for 4x4, 8x4 and 4x8 LFNSTs, which are derived from 8x8 LFNSTs (in VTM-6.0) as described in JVET-P0051 for CE-6.2.2a. The corresponding results are presented in the last three rows of Table 1 for the set of all (in total 8) LFNST matrices, where in lfnst[i][j], i and j are associated with the transform set index and LFNST index, respectively.

All available 16 basis vectors are used to calculate SNR values, and corresponding results are presented in Table 1. Table 2 further evaluates the orthogonality based on top 8 basis vectors, which are the ones used when the zero-out scheme normatively restricts the number of coefficients to 8.

Table 1. Orthogonality analysis based on 16 basis vectors

| SNR | lfnst[0][0] | lfnst[0][1] | lfnst[1][0] | lfnst[1][1] | lfnst[2][0] | lfnst[2][1] | lfnst[3][0] | lfnst[3][1] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-6.0: 8x8 | 49.63 | 50.53 | 49.32 | 48.81 | 49.96 | 49.44 | 49.37 | 50.03 |
| CE6-2.3a: 8x8 | 49.08 | 49.33 | 50.00 | 49.68 | 49.54 | 49.77 | 50.09 | 49.46 |
| VTM-6.0: 4x4 | 49.60 | 49.18 | 50.36 | 49.44 | 48.74 | 50.40 | 49.71 | 49.60 |
| CE6-2.2a: 4x4 | 20.98 | 23.32 | 20.51 | 17.40 | 18.03 | 17.71 | 20.54 | 18.03 |
| CE6-2.2a: 8x4 | 24.02 | 35.15 | 20.89 | 17.64 | 18.07 | 17.77 | 21.97 | 18.11 |
| CE6-2.2a: 4x8 | 24.03 | 23.84 | 35.22 | 38.64 | 38.10 | 37.69 | 30.53 | 45.17 |

Table 2. Orthogonality analysis based on 8 top-most basis vectors

| SNR | lfnst[0][0] | lfnst[0][1] | lfnst[1][0] | lfnst[1][1] | lfnst[2][0] | lfnst[2][1] | lfnst[3][0] | lfnst[3][1] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-6.0: 8x8 | 50.24 | 49.99 | 48.36 | 48.50 | 50.60 | 48.69 | 49.45 | 49.48 |
| CE6-2.3a: 8x8 | 49.59 | 50.23 | 48.45 | 48.82 | 49.61 | 49.15 | 51.77 | 49.22 |
| VTM-6.0: 4x4 | 49.61 | 48.03 | 51.00 | 49.84 | 49.43 | 50.38 | 49.63 | 49.16 |
| CE6-2.2a: 4x4 | 46.60 | 41.62 | 26.39 | 18.94 | 18.08 | 19.25 | 25.30 | 19.15 |
| CE6-2.2a: 8x4 | 48.20 | 45.87 | 26.62 | 19.01 | 18.08 | 19.27 | 26.54 | 19.18 |
| CE6-2.2a: 4x8 | 48.95 | 43.83 | 46.67 | 48.75 | 50.50 | 47.35 | 41.08 | 49.18 |

Both Tables 1 and 2 show that some of the LFNST designs in CE6-2.2a can significantly deviate from orthogonality. As a fair comparison point with the same transform sizes (highlighted in bold letters), 4x4 LFNSTs in CE6-2.2a can lead up to 30dB SNR loss with respect to 4x4 LFNSTs in VTM-6.0.

The results also show that similar SNR values are obtained for the matrices in CE6-2.3a and VTM-6.0 (see first two rows of Table 1 and 2).

# Analysis Tool

For the analysis of orthogonality, the attached “code/cross_check_ce622.m” MATLAB script is used. The LFNST matrices in VTM-6.0 are provided in “code/vtm_lfnst_4x4.m” and “code/vtm_lfnst_8x8.m”. The matrices for the 8x8 LFNST used in CE6-2.3 are provided in “code/vtm_lfnst_8x8_ce623.m”.

# References

- X. Zhao, X. Li, S. Liu, “CE6-2.2a/c: LFNST with unified kernels”, JVET-P0051, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- T. Zhou, T. Hashimoto, T. Ikai, “CE6-2.3a: Simplification with new LFNST transform basis”, P0065, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct.2019.

- H. E. Egilmez, M. Salehifar, X. Zhao, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-O2026, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- X. Zhao, H. E. Egilmez, M. Salehifar, “CE6: Summary Report on Transforms and Transform Signalling”, JVET-P0026, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

# Patent rights declaration

Qualcomm Inc. does not have current or pending patent rights relating to the technology described in this contribution.