| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0596-v3 |
| --- | --- |

| Title: | Non-CE3: Removal of chroma 2xN blocks in CIIP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Ted Hsieh, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, thsieh, aramasub, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal proposes to restrict chroma 2xN blocks for CIIP blocks. In the first method, the CIIP mode is disabled for 4xN CUs. In the second approach, when a 4xN CU is encoded using CIIP mode, the combination of inter-intra prediction is only applied for luma component while the chroma components using the inter prediction. (Version 3: Updated results).

The average (Y, U, V) BD-rate change of these proposed methods are reported as follows:

Proposed method 1: (0.02%, 0.09%, 0.02%) and (0.02%, -0.05%, -0.34%) in RA and LDB.

Proposed method 2: (0.01%, 0.08%, 0.00%) and (-0.04%, 0.06%, 0.20%) in RA and LDB.

# Introduction

In the combination inter-intra prediction mode (CIIP), the inter prediction of a merge index is blended with the intra prediction of Planar mode to get the final prediction of the block. The blending weight is derived based on the prediction mode the neighbour blocks. The weight of the chroma components and the luma component is identical. It should be noted that, the 2xN chroma intra blocks lead to an inefficient hardware implementation, because it causes pipeline delay and requires 2xN pixels process in the hardware architecture.

In this document, two restrictions to simplify the CIIP mode are proposed.

# Proposed method

This document presents two restrictions to simplify the CIIP mode that avoid 2xN processing in CIIP mode.

- Disable CIIP mode for 4xN CU

In this restriction, the CIIP mode is disabled for 4xN CUs.

- Disable CIIP for 2xN chroma blocks

In this restriction, the CIIP mode is allowed for 4xN CUs. In this case, the combination inter-intra prediction is applied for only luma component while the chroma components use only inter prediction. On the other hands, there is no inter-intra blending for the 2xN chroma blocks; hence the 2xN intra prediction is not needed for CIIP mode.

# Experimental results

The proposed method has been implemented on top of VTM6.0 and has been tested under the test conditions defined in JVET-N1010. Table 1 and 2 show the test results in RA and LD configuration for method 1 and method 2, respectively.

Table 1 Experimental results of method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | -0.08% | 100% | 102% |
| Class A2 | 0.04% | 0.06% | 0.13% | 101% | 100% |
| Class B | 0.00% | 0.08% | 0.03% | 102% | 103% |
| Class C | 0.04% | 0.16% | 0.00% | 99% | 89% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.09% | 0.02% | 101% | 98% |
| Class D | -0.01% | 0.07% | -0.09% | 104% | 111% |
| Class F | 0.03% | 0.02% | 0.00% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.05% | -0.47% | 104% | 106% |
| Class C | -0.01% | -0.14% | -0.17% | 99% | 88% |
| Class E | 0.06% | 0.07% | -0.34% | 102% | 101% |
| Overall | 0.02% | -0.05% | -0.34% | 102% | 98% |
| Class D | 0.04% | 0.40% | -0.05% | 109% | 123% |
| Class F | 0.07% | -0.25% | 0.13% | 102% | 100% |

Table 2 Experimental results of method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | -0.01% | 101% | 103% |
| Class A2 | 0.03% | 0.13% | 0.04% | 101% | 100% |
| Class B | 0.01% | 0.07% | 0.00% | 102% | 103% |
| Class C | -0.01% | 0.05% | 0.00% | 99% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.00% | 101% | 98% |
| Class D | -0.04% | -0.13% | -0.18% | 107% | 114% |
| Class F | 0.01% | 0.00% | 0.00% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.10% | 105% | 109% |
| Class C | -0.10% | -0.14% | -0.16% | 101% | 93% |
| Class E | -0.03% | 0.42% | 0.83% | 102% | 102% |
| Overall | -0.04% | 0.06% | 0.20% | 103% | 102% |
| Class D | -0.07% | -0.10% | -0.16% | 109% | 123% |
| Class F | -0.06% | -0.09% | 0.38% | 103% | 101% |

# Conclusion

This proposal proposes to restrict chroma 2xN blocks for CIIP blocks. In the first method, the CIIP mode is disabled for 4xN CUs. In the second approach, when 4xN CU is encoded using CIIP mode, only combination of inter-intra is applied for luma components while the chroma components using only inter prediction. It is recommended to adopt the proposed technique 2 into the next version of VTM.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).