| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0597 |
| --- | --- |

| Title: | AHG6/AHG17: Generalized cubemap projection syntax for 360-degree videos |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Ying-Jui Chen, Chi-Cheng Ju (MediaTek) No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Jill Boyce, Max Dmitrichenko (Intel) | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 { louise.lee, jl.lin, yr.chen, cc.ju}@mediatek.com jill.boyce@intel.com, max.dmitrichenko@intel.com |
| Source: | MediaTek Inc., Intel |  |  |

_____________________________

- Abstract

This contribution proposes the syntax for signalling a generalized cubemap projection which is flexible to adjust the sampling distribution by setting different coefficients of the mapping function. The proposed syntax also allows to designate CMP and EAC. An approximation of HEC can also be realized by setting the coefficients. In addition to the regular cubemap projections with six faces, hemisphere cubemap projection, which consists of five faces representing 180⁰ x 180⁰ content, is also supported by the proposed syntax.

Introduction

To send a cubemap projection, the HEVC SEI messages [1] of cubemap projection and region-wise packing are used. However, it only supports the conventional CMP which is not well-designed for compression due to unequal sampling distribution, and lots of bits are also required for signalling.

In the Gothenburg meeting, the syntax for a generalized cubemap projection [2] was proposed, and BoG [3] recommended to adopt it. The proposed syntax design has the ability to cover many kinds of cubemap projections such as CMP, EAC [4] and HEC [5]. Hemisphere cubemap projection [6] which has five faces, (one full and four half faces) representing 180⁰ x 180⁰ content is also included. The number of bits used to send a padded cubemap projection only needs nearly one-tenth of bits comparing to the current HEVC SEI messages.

Syntax and semantics

The cubemap-based projection has six square faces which are defined as Front, Back, Right, Left, Top and Bottom. Each face is defined by a two-dimensional (u, v) plane as illustrated in Figure 1 with u and v in the range of [-1, 1].

Figure 1. Coordinates definition for a cubemap projection

To compress 360 videos, the video content on the sphere is projected to the cube faces. Adjustment functions, such as EAC and HEC, can be applied to achieve more uniform sampling distribution. The cube are then unfolded, and its faces, with or without rotation, are arranged by some pre-defined packing type to form a 2D picture for coding. The proposed syntax such as face index and rotation is aligned with 360Lib [7].

However, there may have discontinuous edges inside the cubemap projected picture which will create seam artifacts when reconstructing 360 videos. To deal with this issue, padding can be applied on the discontinuous edges and/or frame boundaries to protect the content.

Syntax

| generalized_cmp_projection( payloadSize ) { | Descriptor |
| --- | --- |
| generalized_cmp_cancel_flag | u(1) |
| if( !generalized_cmp_cancel_flag ) { |  |
| generalized_cmp_persistence_flag | u(1) |
| cmp_packing_type | u(3) |
| cmp_mapping_function_type | u(2) |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| numFaces = ( cmp_packing_type = = 4 ¦ ¦ cmp_packing_type = = 5 ) ? 5 : 6 |  |
| for( i = 0; i < numFaces; i++ ) { |  |
| face_index[ i ] | u(3) |
| face_rotation[ i ] | u(2) |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| if( cmp_mapping_function_type = = 2 ) { |  |
| cmp_x_axis_coeff[ i ] | u(7) |
| cmp_x_axis_y_position_related_flag[ i ] | u(1) |
| cmp_y_axis_coeff[ i ] | u(7) |
| cmp_y_axis_x_position_related_flag[ i ] | u(1) |
| } |  |
| } |  |
| cmp_padding_enabled_flag | u(1) |
| if( cmp_padding_enabled_flag ) { |  |
| cmp_padding_type | u(1) |
| cmp_padding_width_minus1 | u(4) |
| } |  |
| while( !byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Semantics

Cubemap projection semantics

generalized_cmp_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous generalized cubemap projection SEI message in output order. generalized_cmp_cancel_flag equal to 0 indicates that generalized cubemap projection information follows.

generalized_cmp_persistence_flag specifies the persistence of the generalized cubemap projection SEI message for the current layer.

generalized_cmp_persistence_flag equal to 0 specifies that the generalized cubemap projection SEI message applies to the current decoded picture only.

Let picA be the current picture. generalize_cmp_persistence_flag equal to 1 specifies that the generalized cubemap projection SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a generalized cubemap projection SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

cmp_packing_type specifies the packing type and position index of the cubemap packing as specified in Table 1. When the value of cmp_packing_type is in the range of 0 to 3, cubemap packing with six faces is used. When cmp_packing_type is 4 or 5, hemisphere cubemap packing with one full face and four half faces is used. By identifying cmp_packing_type, the width and height of a cubemap face in luma samples can be determined by pic_width_in_luma_samples, pic_height_in_luma_samples, and conformance window offsets. The value of cmp_packing_type shall be in the range of 0 to 5, inclusive. Other values for cmp_packing_type are reserved for future use by ITU-T | ISO/IEC.

Table 1 – Specification of packing type, position index, face width and height in luma samples based on cmp_packing_type

| cmp_packing_type | Packing type and position index | Face width in luma samples | Face height in luma samples |
| --- | --- | --- | --- |
| 0 |  | pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) | ( pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) ) / 6 |
| 1 |  | ( pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) ) / 2 | ( pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) ) / 3 |
| 2 |  | ( pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) ) / 3 | ( pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) ) / 2 |
| 3 |  | ( pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) ) / 6 | pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) |
| 4 |  | For position 0, 1, 3 and 4, ( pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) ) / 6 For position 2, ( pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) ) / 3 | pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) |
| 5 |  | pic_width_in_luma_samples - ( SubWidthC * ( conf_win_left_offset + conf_win_right_offset ) ) | For position 0, 1, 3 and 4, ( pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) ) / 6 For position 2, ( pic_height_in_luma_samples - ( SubWidthC * ( conf_win_top_offset + conf_win_bottom_offset ) ) ) / 3 |

cmp_mapping_function_type specifies the mapping function used in the cubemap projection. When cmp_mapping_function_type is equal to 0, conventional CMP is used. When cmp_mapping_function_type is equal to 1, EAC is used. When cmp_mapping_function_type is equal to 2, other mapping functions are used, and cmp_x_axis_coeff[ i ], cmp_x_axis_y_position_related_flag[ i ], cmp_y_axis_coeff[ i ], and cmp_y_axis_x_position_related_flag[ i ] shall be signalled. The value of cmp_mapping_function_type shall be in the range of 0 to 2, inclusive.

face_index[ i ] specifies the face index for position index i in cmp_packing_type and the relationship between the global coordintes 3D (X, Y, Z) and the local coordinate 2D (u, v) as specified in Table 2. When cmp_packing_type is 4 or 5, the faces on position index 0, 1, 3, and 4 should be the half faces that connect to the face on position index 2 in the 3D coordinate. The value of face_index[ i ] shall be in the range of 0 to 5, inclusive.

Table 2 – Specification of the cube faces based on face_index[ i ]

| face_index[ i ] | Corresponding cube face | X | Y | Z |
| --- | --- | --- | --- | --- |
| 0 | Front face with positive X axis value | 1.0 | −v | −u |
| 1 | Back face with negative X axis value | −1.0 | −v | u |
| 2 | Top face with positive Y axis value | u | 1.0 | v |
| 3 | Bottom face with negative Y axis value | u | −1.0 | −v |
| 4 | Right face with positive Z axis value | u | −v | 1.0 |
| 5 | Left face with negative Z axis value | −u | −v | −1.0 |

face_rotation[ i ] specifies the rotation to be applied to the face on position index i as specified in Table 3. The value of face_rotation[ i ] shall be in the range of 0 to 3, inclusive.

Table 3 – Specification of counterclockwise rotation angle based on face_rotation[ i ]

| face_rotation[ i ] | Rotation angle (counterclockwise) |
| --- | --- |
| 0 | 0 |
| 1 | 90 |
| 2 | 180 |
| 3 | 270 |

cmp_x_axis_coeff[ i ] specifies the coefficient used in the cubemap adjustment function in x-axis. The value of cmp_x_axis_coeff[ i ] shall be in the range of 0 to 27 – 1, inclusive.

When cmp_x_axis_coeff[ i ] is not present, it is inferred to be equal to 0.

cmp_x_axis_y_position_related_flag[ i ] equal to 1 specifies that the sampling in x-axis referes to the value of y position. When cmp_x_axis_y_position_related_flag[ i ] is equal to 0, the value of y position is not used.

When cmp_x_axis_y_position_related_flag[ i ] is not present, it is inferred to be equal to 0.

cmp_y_axis_coeff[ i ] specifies the coefficient used in the cubemap adjustment function in y-axis. The value of cmp_y_axis_coeff[ i ] shall be in the range of 0 to 27 – 1, inclusive.

When cmp_y_axis_coeff[ i ] is not present, it is inferred to be equal to 0.

cmp_y_axis_x_position_related_flag[ i ] equal to 1 specifies that the sampling in y-axis referes to the value of x position. When cmp_y_axis_x_position_related_flag[ i ] is equal to 0, the value of x position is not used.

When cmp_y_axis_x_position_related_flag[ i ] is not present, it is inferred to be equal to 0.

Given a position (u, v) on a cube face after projecting from the sphere to the cube, the adjusted sampling location (u’, v’) is derived as follows:

cmpCoeffX[ i ] = ( cmp_x_axis_coeff[ i ] + 1 ) / 27
t = cmpCoeffX[ i ] * u * ( cmp_x_axis_y_position_related_flag[ i ] * v’2 – 1 )
if( t = = 0 )	(1)
      u’ = u
else
      u’ = ( 1 – Sqrt( 1 – 4 * t ( u – t ) ) ) / ( 2 * t )

cmpCoeffY[ i ] = ( cmp_y_axis_coeff[ i ] + 1 ) / 27
t = cmpCoeffY[ i ] * v * ( cmp_y_axis_x_position_related_flag[ i ] * u’2 – 1 )
if( t = = 0 )	(2)
      v’ = v
else
      v’ = ( 1 – Sqrt( 1 – 4 * t ( v – t ) ) ) / ( 2 * t )

cmp_padding_enabled_flag equal to 0 specifies that no padding is applied on the cubemap projection. When cmp_padding_enabled_flag is equal to 1, the cubemap projected picture includes padding areas.

cmp_padding_type specifies the padding type in the cubemap projected picture. When cmp_padding_type is equal to 0, padding is applied on the discontinuous edges. When cmp_padding_type is equal to 1, padding is applied on both the discontinuous edges and frame boundaries. The location of the padding shall refer to cmp_packing_type as specified in Table 4.

Table 4 – Specification of padding location based on cmp_packing_type and cmp_padding_type

| cmp_packing_type | cmp_padding_type | Location of padding |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |
| 2 | 0 |  |
| 2 | 1 |  |
| 3 | 0 |  |
| 3 | 1 |  |
| 4 | 0 |  |
| 4 | 1 |  |
| 5 | 0 |  |
| 5 | 1 |  |

cmp_padding_width_minus1 plus 1 specifies the padding width in luma samples used in the cubemap projected picture. The value of cmp_padding_width_minus1 shall be in the range of 0 to 15, inclusive.

Projection for one sample location

Inputs to this process are:

–	lw and lh, which are the width and height, respectively, of a generalized cubemap projected face without padding in luma samples, and

–	the centre point of a sample location (hPos, vPos) along the horizontal and vertical axes, respectively, in a generalized cubemap projected face, where hPos and vPos may have non-integer real values.

Outputs of this process are:

–	sphere coordinates (ϕ, θ) for the sample location in degrees relative to the local coordinate axes.

The projection for a sample location is derived as follows:

–	If cmp_mapping_function_type is equal to 0, which is the conventional CMP, the following applies:

u = ( 2 * hPos ÷ lw ) − 1
v = ( 2 * vPos ÷ lh ) – 1

Table 2 is then used to derive X, Y, Z for each face.
ϕ = Atan2( -Z, X ) * 180 ÷ π
θ = Asin( Y ÷ Sqrt( X2 + Y2 + Z2) ) * 180 ÷ π

–	If cmp_mapping_function_type is equal to 1, which is EAC, the following applies:

tmpU = ( 2 * hPos ÷ lw ) − 1
tmpV = ( 2 * vPos ÷ lh ) – 1
u = tan( tmpU * π / 4)
v = tan( tmpV * π / 4)

Table 2 is then used to derive X, Y, Z for each face.
ϕ = Atan2( -Z, X ) * 180 ÷ π
θ = Asin( Y ÷ Sqrt( X2 + Y2 + Z2) ) * 180 ÷ π

–	If cmp_mapping_function_type is equal to 2, the following applies:

tmpU = ( 2 * hPos ÷ lw ) − 1
tmpV = ( 2 * vPos ÷ lh ) – 1
cmpCoeffX[ i ] = ( cmp_x_axis_coeff[ i ] + 1 ) / 27
cmpCoeffY[ i ] = ( cmp_y_axis_coeff[ i ] + 1 ) / 27
u = tmpU / ( 1 + cmpCoeffX[ i ] * ( 1 – cmp_x_axis_y_position_related_flag[ i ] * tmpV2 ) * ( 1 – tmpU2 ) )
v = tmpV / ( 1 + cmpCoeffY[ i ] * ( 1 – cmp_y_axis_x_position_related_flag[ i ] * tmpU2 ) * ( 1 – tmpV2 ) )

Table 2 is then used to derive X, Y, Z for each face.
ϕ = Atan2( -Z, X ) * 180 ÷ π
θ = Asin( Y ÷ Sqrt( X2 + Y2 + Z2) ) * 180 ÷ π

Cubemap-based Projection

The forward adjustment function of the generalized cubemap projection is as follows:

|  | fxu, v=u1+ax(1-bxv2)(1-u2)fyu, v=v1+ay(1-byu2)(1-v2) | (3) |
| --- | --- | --- |

There are two coefficients in each axis that can make the sampling distribution have different behavior. ax and ay shall be in the range of [0, 1], and bx and by are binary numbers.

CMP

When cmp_mapping_function_type is equal to 0, the conventional CMP is applied. The forward and backward functions are described in (4) and (5), respectively.

|  | fxu=ufyv=v | (4) |
| --- | --- | --- |
|  | gxu=ugyv=v | (5) |

EAC

When cmp_mapping_function_type is equal to 1, EAC is applied. The forward and backward functions are described in (6) and (7), respectively.

|  | fxu=tan⁡(u*π4)fyv=tan⁡(v*π4) | (6) |
| --- | --- | --- |
|  | gxu=4π*tan-1(u)gyv=4π*tan-1(v) | (7) |

Approximated HEC

To implement HEC by the proposed syntax, cmp_mapping_function_type is equal to 2, and the following coefficients shall be set. The adjustment function of EAC is applied on the Top and Bottom faces in HEC. By setting ax = ay = 0.28 and bx = by = 0 in the forward (8) and backward (9) functions, EAC can be simulated. Figure 2 shows the curves of the forward and backward adjustment functions comparing between EAC and approximated EAC using the above coefficients.

|  | fxu=u1+0.28(1-u2)fyv=v1+0.28(1-v2) | (8) |
| --- | --- | --- |
|  | gxu=u , if t=01-1-4t(u-t)2t, otherwise , t=-0.28ugyv=v , if t=01-1-4t(v-t)2t, otherwise , t=-0.28v | (9) |

Figure 2. The curves of the forward and backward adjustment functions of EAC and approximated EAC

For the Left, Front, Right, Back faces, ax = 0.28 and bx = 0 are set to approximate EAC in x-axis while in y-axis, ay = 0.4 and by = 1 are set to perform the same mapping function of HEC. The forward and backward adjustment functions are as follows:

|  | fxu=u1+0.28(1-u2)fyu,v=v1+0.4(1-u2)(1-v2) | (10) |
| --- | --- | --- |
|  | gxu=u , if t=01-1-4t(u-t)2t, otherwise , t=-0.28ugyu,v=v , if t=01-1-4t(v-t)2t, otherwise , t=-0.4v(gx(u)2-1) | (11) |

Hemisphere cubemap projection

A 180⁰ x 180⁰ hemisphere cubemap projection consists of one full cubemap face placing on the middle of the layout and four half faces which comes from the faces surrounding the middle face in the 3D coordinate as illustrated in Figure 3.

Figure 3. An example of the hemisphere cubemap projection layout when cmp_packing_type is equal to 4

When cmp_packing_type is equal to 4 or 5, hemisphere cubemap projection is applied. Adjustment functions, such as EAC and HEC, can also be applied on the faces according to the value of cmp_mapping_function_type to achieve more uniform sampling distribution.

Conclusion

In this contribution, the syntax of signalling a generalized cubemap projection is proposed. With this format, many types of cubemap projections can be supported. Padding of the cubemap projection is also designed. With the proposed syntax, less bits are required to send a cubemap projection compared to the current HEVC SEI messages (cubemap projection and region-wise packing).

References

- “Text of ISO/IEC FDIS 23008-2 (4th edition),” Joint Collaborative Team on Video Coding of ITU-T SG16 and ISO/IEC JTC1/SC29/WG11, w18277, Jan. 2019, Marrakech, MA.

- Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju, “AHG6/AHG17: Cubemap-based projection syntax for 360-degree videos”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O0891, July. 2019, Gothenburg, SE.

- Y.-K. Wang, J. Boyce, “Report of BoG on high-level syntax”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O1076, July. 2019, Gothenburg, SE

- M. Zhou, “AHG8: A study on Equi-Angular Cubemap projection (EAC),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0056, Jul. 2017, Torino, IT.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju, “CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, Jul. 2018, Ljubljana, SI.

- J. Boyce, M. Dmytrychenko, “AHG8: Hemisphere cubemap projection format”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M0452, Jan. 2019, Marrakech, MA.

- Y. Ye, and J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1004, Jan. 2019, Marrakech, MA.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).