| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0601-v1 |
| --- | --- |

| Title: | Non-CE4: Support slice level disabling for PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo Masaru Ikeda 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to support slice level disabling for PROF. And, it’s reported that PROF brings visual artifact in the specific frame. In current VTM design, PROF doesn’t have a block level control flag, and it is used implicitly when Affine enable flag and PROF enable flag are enabled in SPS. It makes difficult to avoid the visual artifact in slice level. In this contribution, slice level control flags are proposed to avoid the visual artifact.

Problem statement

A technique, prediction refinement with optical flow (PROF) was adopted in last JVET meeting [1]. In the PROF process, a pixel level refinement is applied with optical flow after subblock level Affine motion compensation. Here, this refinement is not applied for chroma to reduce complexity. Therefor, a misalignment exists between luma and chroma.

Figure 1 is an example that it is cropped from the test sequence “CatRobot”. The frame number is 16.

Figure 1: An example of visual artifact (CatRobot, frame number 16)

The chroma components in Figure 1, Cb and Cr are shown in Figure 2 and Figure 3. As we can see, serrated visual artifacts are observed at the color boundary.

Figure 2: Cb component

Figure 3: Cr component

As similar as CatRobot, the visual artifact is observed in the other test sequence, SlideShow, frame number 72. It’s shown in Figure 4, and Figure 5 and Figure 6 show the chroma component.

Figure 4: An example for visual artifact (SlideShow, frame number 72)

Figure 5: Cb component

Figure 6: Cr component

Proposed method

To support PROF for also chroma component is a straightforward way to solve above problem. In last JVET meeting, also PROF for chroma was proposed in a contribution JVET-O0583 [2]. However, it was not adopted due to increase complexity.

If PROF is disabled using the SPS flag, it is expected that above visual artifacts disappear. However, also it will bring the coding efficiency loss.

This visual artifact occurs in a specific frame that the motion of object has rotation and a long-time distance such as the frame number 16 in RA case. This contribution proposes the other way that an encoder controls PROF on/off in slice level, and propose to add slice level flag for PROF. The syntax changes are described in below subclause. The detail draft text, the file is included in this contribution.

To control PROF on/off, this contribution proposes to add two flags that are  sps_affine_prof_slice_present_flag and slice_disable_affine_prof_flag. Those semantics are described in below subclause. This approach is similar to sps_bdof_dmvr_slice_present_flag  and slice_disable_bdof_dmvr_flag that was proposed in [3].

Text changes

Sequence parameter set RBSP semantics

sps_affine_prof_slice_present_flag equal to 1 specifies that slice_disable_affine_prof_flag is present in slice headers referring to the SPS. sps_affine_prof_slice_present_flag equal to 0 specifies that slice_disable_affine_prof_flag is not present in slice headers referring to the SPS. When sps_affine_prof_slice_present_flag is not present, the value of sps_affine_prof_present_flag is inferred to be equal to 0.

Slice header semantics

slice_disable_affine_prof_flag equal to 1 specifies that the prediction refinement with optical flow for affine motion compensation is disabled in the current slice. slice_disable_affine_prof_flag equal to 0 specifies that the prediction refinement with optical flow for affine motion compensation may or may not be enabled in the current slice. When slice_disable_affine_prof_flag is not present, the value of slice_disable_affine_prof_flag is inferred to be 0.

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_bdof_enabled_flag | u(1) |
| sps_smvd_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if( sps_bdof_enabled_flag ¦ ¦ sps_dmvr_enabled_flag) |  |
| sps_bdof_dmvr_slice_present_flag | u(1) |
| … | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_affine_prof_enabled_flag | u(1) |
| if( sps_affine_prof_enabled_flag ) |  |
| sps_affine_prof_slice_present_flag | u(1) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if ( slice_type != I ) { |  |
| … |  |
| if( sps_bdof_dmvr_slice_present_flag ) |  |
| slice_disable_bdof_dmvr_flag | u(1) |
| if( sps_affine_prof_slice_present_flag ) |  |
| slice_disable_affine_prof _flag | u(1) |
| … |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Conclusion

This contribution introduced that the visual artifacts occur when PROF is applied. To avoid the visual artifacts, it’s proposed to support slice level disabling for PROF.

References

- Jiancong (Daniel) Luo, Yuwen He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, Gothenburg, SE, July 2019.

- Yuwen He, Wei Chen, “CE4-related: Prediction refinement with optical flow for chroma components for affine mode”, JVET-O0583, Gothenburg, SE, July 2019.

- Yong He, Yuwen He, Ahmed Hamza, Han Huang, Wei-Jung Chien, Vadim Seregin, Muhammed Coban,Marta Karczewicz, Sriram Sethuraman , “Unified syntax for JVET-O0184/O0250/O0504 on DMVR and BDOF flag” , JVET-O1140, Gothenburg, SE, July 2019.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).