| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0604-v2 |
| --- | --- |

| Title: | Non-CE4: Motion field storage optimization for the line buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Sergey Ikonin, Alexander Karabutov, Chernyak Roman | Tel: Email: | +79250089652 solovyev.timofey@huawei.com sergey.ikonin@huawei.com karabutov.alexander@huawei.com chernyak.roman@huawei.com |  |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes modification of motion vector and reference index inheritance across the CTU boundaries. 8-pixel grid is used for accessing motion information located in top CTU. The proposed changes allow to halve the amount of stored motion information. For 4K video the line buffer size is reduced by 5 Kbytes, for 8K video – by 10 Kbytes. The simulation results reportedly show 0.01% / 0.xx% BD-rate change for RA/LDB configurations respectively.

# Introduction

In VVC draft 6 motion information inheritance is used for merge, AMVP, affine and IBC modes. Due to 4x4 grid is used for motion information storage, one motion information unit per 4 luma pixels should be stored in the line buffer. One motion information unit comprise two 18-bit motion vectors (list0 and list1) and two 4-bit reference indices. In total, size of one motion information unit is equal to (18 + 18 + 4) * 2 = 80 bits. This means that, for example, for 4K video ( 4096 / 4 ) * 80 = 81920 bits (10 Kbytes) of motion information is needed to be stored , for 8K video 20 Kbytes is needed.

# Proposed Solutions

It’s proposed to use 8-pixel grid instead of 4-pixel grid for accessing motion information located in top CTU. This means that, for example, if bottom line of top CTU consists of 4x4 blocks, only motion information of every second block is stored. Location of stored and skipped motion information blocks is depicted in Figure 1.

Implementation of this method for the regular merge, AMVP and IBC is straightforward. Only for affine mode some tiny changes should be done. In particular, it’s proposed slightly change deriving of inherited affine candidates for affine merge and affine AMVP processes, when used neighboring affine block is located in top CTU.

Figure 1. Location of stored and skipped motion information blocks

Firstly, affine inheritance is not used across CTU boundaries if top affine block width is equal to 8. With this restriction it’s guaranteed that x coordinate of the top affine block is multiple of 8.

Secondly, for affine model parameters derivation, instead of subblocks with coordinates x and x + w – 1 (where x and w is a horizontal coordinate and width of the neighboring affine block respectively) it’s proposed to use the motion information of subblocks with coordinates x and x + w/2.

The reason is that after converting to 8-pixel aligned coordinates ( ( x >> 3 ) << 3 ) distance between x and x + w – 1 is equal to w – 8, as depicted in Figure 2, and w – 8 can be not a power of two. Meanwhile, the proposed method (usage of coordinates x and x + w/2) allows to use w/2 (which is a power of two) in model derivation equations, so division-free implementation is preserved.

Figure 2. Affine model parameters derivation for inherited affine candidates

# Simulation Results

The proposed modification is evaluated for random-access (RA) and Low-delay-B (LDB) configurations using VTM-6.0 software [1] according to the common test conditions as described in JVET-N1010 [2]. The simulation results for the proposed solution are presented in the tables below.

Coding performance of the proposed solution on top of VTM-6.0 for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.14% | 0.00% | 102% | 102% |
| Class A2 | 0.05% | 0.09% | 0.05% | 102% | 102% |
| Class B | 0.00% | 0.06% | -0.02% | 102% | 102% |
| Class C | 0.01% | 0.04% | -0.03% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.00% | 102% | 103% |
| Class D | -0.03% | -0.08% | 0.05% | 102% | 106% |
| Class F (optional) | 0.01% | 0.06% | -0.01% | 103% | 104% |

Coding performance of the proposed solution on top of VTM-6.0 for LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.04% | 0.12% | 0.04% | 102% | 102% |
| Class E | 0.03% | 0.17% | 0.63% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | 0.17% | 0.46% | 102% | 101% |
| Class F (optional) | 0.01% | -0.41% | 0.65% | 102% | 102% |

# Conclusion

This contribution describes modification of motion information inheritance across the CTU boundaries. 8 - pixel grid is used for accessing motion information located in top CTU. The amount of motion information that need to be stored in the line buffer decreases twice. Notably, for 4K video the line buffer can be reduced by 5 Kbytes, for 8K video – by 10 Kbytes. The performance change is 0.01% / 0.0x% for RA/LDB configurations.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team, JVET-O2001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).