| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0605-v1 |
| --- | --- |

| Title: | Non-CE4: On triangular merge list size signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Sergey Ikonin, Alexander Karabutov, Chernyak Roman | Tel: Email: | +79250089652 solovyev.timofey@huawei.com sergey.ikonin@huawei.com karabutov.alexander@huawei.com chernyak.roman@huawei.com |  |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution describes method of signalling triangular merge candidate list size in picture parameter set.

# Introduction

JVET-O0238 [1] adopted into VVC draft 6 [2] introduces picture parameter set syntax element pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 with the following semantics.

pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice header of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 greater than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is not present in slice header of slices referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 shall be in the range of 0 to MaxNumMergeCand − 1.

Corresponding slice level syntax element is max_num_merge_cand_minus_max_num_triangle_cand having the following semantics.

max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slice subtracted from MaxNumMergeCand.

When max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 + 1.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand = 
	MaxNumMergeCand − max_num_merge_cand_minus_max_num_triangle_cand

When max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive.

When max_num_merge_cand_minus_max_num_triangle_cand is not present, and (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the current slice.

## Problem definition

According to the current design, to avoid slice level syntax element max_num_merge_cand_minus_max_num_triangle_cand signalling, pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 should be in range [1, MaxNumMergeCand – 1]. If the following conditions are true:

- sps_triangle_enabled_flag is equal to 1

- MaxNumMergeCand greater than or equal to 2,

max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 + 1. This means that max_num_merge_cand_minus_max_num_triangle_cand is in range [2, MaxNumMergeCand]. This in turn means that MaxNumTriangleMergeCand (equal to MaxNumMergeCand − max_num_merge_cand_minus_max_num_triangle_cand) is in range [0, MaxNumMergeCand – 2]. So triangular merge list size equal to regular merge list size or less by one can not be signalled. Herein MaxNumTriangleMergeCand can be derived as 1, that is useless because triangular prediction mode is used only when MaxNumTriangleMergeCand is greather or equal to 2.

# Proposal

In order to remove redundand signalling and keep possibility to signal triangular merge list size equal to regular merge list size and thereby support the original spirit of triangular merge list size derivation the following specification changes are proposed. With such changes MaxNumTriangleMergeCand can be signalled in range [2, MaxNumMergeCand] that corresponds the triangular prediction mode merge list design.

## Proposed changes to the VVC specification

The proposed changes on top of the VVC draft (JVET-O2001-v14) are marked in yellow

pps_max_num_merge_cand_minus_max_num_triangle_cand_minusplus1 equal to 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is present in slice header of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 greater than 0 specifies that max_num_merge_cand_minus_max_num_triangle_cand is not present in slice header of slices referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 shall be in the range of 0 to MaxNumMergeCand − 1.

…

max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slice subtracted from MaxNumMergeCand.

When max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 + 1plus1 – 1.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand = 
	MaxNumMergeCand − max_num_merge_cand_minus_max_num_triangle_cand

When max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive.

When max_num_merge_cand_minus_max_num_triangle_cand is not present, and (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the current slice.

# References

[1] M. Pettersson, P. Wennersten, R. Sjöberg, M. Damghanian, “AHG17: Parameters in PPS or slice headers”, Document of Joint Video Experts Team, JVET-O0238.

[2] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6)”, Document of Joint Video Experts Team, JVET-O2001.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).