| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0609 |
| --- | --- |

| Title: | AHG12: On slice address signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the VVC specification related to the slice address signaling in case of subpictures:

- Firstly, it is proposed to signal a subpicture ID in the slice header to specify which subpicture the current slice belongs to, conditioned on the presence of subpictures in the CVS.

- Secondly, it is proposed to signal the slice address in the slice header relative to the subpicture position.

- Thirdly, it is proposed to use address indirection for subpicture ID and remove address indirection for slice addresses. For each subpicture, the slice addresses are fixed relative to the subpicture and can be reused during sub-bitstream extraction and merge processes.

Proposed specification text relative to JVET-O2001-vE is provided with the contribution.

# Introduction

In the current VVC specification draft in JVET-O2001-vE, subpictures are supported and targets simplifying the sub-bitstream extraction and merge processes. However, the address signaling mechanisms for subpictures with regards to other defined hierarchical partitions such as picture and slices can be improved.

In the current VVC specification draft, slice addresses are signaled in the slice header and are used to derive the spatial position of the slice in the picture. However, there are a few issues with the current slice address signaling scheme when subpictures are used and when there is more than one slice in the subpictures:

- The spatial position of the slice in the subpicture cannot be derived directly from the slice_address syntax in the slice header and it requires a multi-steps process:

  - the spatial position of the slice in the picture needs to be derived first

  - then in a second step it needs to be derived which subpicture that spatial position in the picture belongs to

  - then in a third step the spatial position of the slice in that subpicture can be derived.

- From the slice header it cannot be derived which subpicture this slice belongs to. This information would be useful for the sub-bitstream merge and extraction process.

- The fixed relative spatial position of slices in subpicture is not exploited when subpictures are being extracted or merged (as in sub-bitstream extraction and merge).

- The indirection mechanism used for mapping the slice_address to slice_id might be suboptimal for sub-bitstream extraction and merge processes since in case of multiple slices in a subpicture, sub-bitstream extraction using the current VVC design may require several address indirections: one indirection for slice_address values in each slice.

# Proposal

This contribution proposes a solution to solve the above issues and to simplify multi-step process for deriving the relative position of slices in a subpicture. This contribution proposes following changes related to the slice address signaling in case of subpictures:

- Firstly, it is proposed to signal a subpicture ID in the slice header to specify which subpicture the current slice belongs to, conditioned to the presence of subpictures in the CVS.

- Secondly, it is proposed to signal the slice address in the slice header relative to the subpicture position.

- Thirdly it is proposed to use address indirection for subpicture ID and remove address indirection for slice addresses. For each subpicture the slice addresses are fixed relative to the subpicture and can be reused during sub-bitstream extraction and merge processes.

The proposed syntax and semantics are marked in yellow with removed text in red strike-through on top of the current VVC draft (JVET-O2001-vE).

Below are the proposed syntax and semantics changes in the SPS on top of JVET-O2001-vE:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( subpics_present_flag ) { |  |
| max_subpics_minus1 | u(8) |
| subpic_grid_col_width_minus1 | u(v) |
| subpic_grid_row_height_minus1 | u(v) |
| for( i = 0; i < NumSubPicGridRows; i++ ) |  |
| for( j = 0; j < NumSubPicGridCols; j++ ) |  |
| subpic_grid_idx[ i ][ j ] | u(v) |
| for( i = 0; i <= NumSubPics; i++ ) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| signalled_subpic_id_flag | u(1) |
| if( signalled_subpic_id_flag ){ |  |
| signalled_subpic_id_length_minus1 | ue(v) |
| for( i = 0; i < NumSubPics; i++ ) |  |
| subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

signalled_subpic_id_flag equal to 1 specifies that the subpicture ID for each subpicture is signalled. signalled_subpic_id_flag equal to 0 specifies that subpic IDs are not signalled. When subpics_present_flag is equal to 0, the value of signalled_subpic_id_flag is inferred to be equal to 0.

signalled_subpic_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element subpic_id[ i ] when present, and the syntax element slice_subpic_id in slice headers. The value of signalled_subpic_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_subpic_id_length_minus1 is inferred to be equal to Ceil( Log2( Max( 2, max_subpics_minus1 + 1 ) ) ) − 1.

subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the subpic_id[ i ] syntax element is signalled_subpic_id_length_minus1 + 1 bits.  When not present, the value of subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to NumSubPics minus 1, inclusive.

Below are the proposed syntax and semantics changes in the PPS on top of JVET-O2001-vE:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

signalled_slice_id_flag equal to 1 specifies that the slice ID for each slice is signalled. signalled_slice_id_flag equal to 0 specifies that slice IDs are not signalled. When rect_slice_flag is equal to 0, the value of signalled_slice_id_flag is inferred to be equal to 0.

NOTE – For a bitstream that is a result of a sub-bitstream extraction, when each picture in the bitsream contains a true subset of the slices included in the picture of the "original" bitstream, and the subset of the included slices does not included the top-left corner slice of the pictures in the "original" bitstream, the value of signalled_slice_id_flag in the PPSs in the extracted bitstreams has to be equal to 1.

signalled_slice_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( Max( 2, num_slices_in_pic_minus1 + 1 ) ) ) − 1.

slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits.  When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive.

Below are the proposed syntax and semantics changes in the slice header on top of JVET-O2001-vE:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ){ |  |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| slice_address | u(v) |
| } |  |
| … |  |
| } |  |

slice_subpic_id specifies the value of subpic_id  for the sub-picture the slice is spatialy located in. When not present, the value of slice_subpic_id is inferred to be equal to 0. The length of the syntax element is Ceil( Log2(max_subpics_minus1 + 1) ) bits.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0. If subpics_present_flag is equal to 0, slice_address represents the slice adress of the slice relative to the picture, else, if subpics_present_flag is equal to 1, slice_adress represents the slice adress of the slice relative to the sub-picture with sub-picture ID equal to slice_subpic_id.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the brick ID as specified by Equation (7-59).

- The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice ID of the slice.

- The length of slice_address is signalled_slice_id_length_minus1 + 1 Ceil( Log2 ( max (1,  NumBricksInPic-NumSubpics ) ) ) bits.

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- The value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each brick, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded brick(s).

num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0. When single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0.

The variable NumBricksInCurrSlice, which specifies the number of bricks in the current slice, and SliceBrickIdx[ i ], which specifies the brick index of the i-th brick in the current slice, are derived as follows:

if( rect_slice_flag ) {
	sliceIdx = 0
	while( slice_address  !=  slice_id[ sliceIdx ] )
		sliceIdx++
	subpicIdx = 0
	while( slice_subpic_id  !=  subpic_id[ subpicIdx ] )
		subpicIdx++
	sliceIdx = subpic_id[ subpicIdx ] + slice_address
	NumBricksInCurrSlice = NumBricksInSlice[ sliceIdx ]
	brickIdx = TopLeftBrickIdx[ sliceIdx ]
	for( bIdx = 0; brickIdx  <=  BottomRightBrickIdx[ sliceIdx ]; brickIdx++ )	(7-92)
		if( BricksToSliceMap[ brickIdx ]  = =  sliceIdx )
			SliceBrickIdx[ bIdx++ ] = brickIdx
} else {
	NumBricksInCurrSlice = num_bricks_in_slice_minus1 + 1
	SliceBrickIdx[ 0 ] = slice_address
	for( i = 1; i < NumBricksInCurrSlice; i++ )
		SliceBrickIdx[ i ] = SliceBrickIdx[ i − 1 ] + 1
}

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).