| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0613-v1 |
| --- | --- |

| Title: | Non-CE5: On the average calculation for luma adaptive deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Dmytro Rusanovskyy, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com dmytror@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify the average luma calculation value for luma average dependent deblocking filter, by utilizing the information of maximum filter length from side P and side Q. The results for SDR-CTC and HDR-CTC conditions are reported below.

Table-1: Average BD rate results for SDR-CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.00% | 0.00% | 102% | 99% |
| RA | 0.00% | -0.02% | 0.08% | 100% | 100% |
| LD | -0.02% | -0.20% | -0.10% | 99% | 101% |

Table-2: Average BD rate results for HDR-CTC

| Config. | DE-100 | PSNR-L100 | wPSNR-Y | wPSNR-U | wPSNR-V | PSNR-Y | PSNR-U | PSNR-V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | -0.00% | 100% | 100% |
| RA | 0.01% | 0.01% | 0.01% | -0.25% | -0.03% | 0.01% | -0.21% | -0.01% | 100% | 100% |

# Introduction

In last Macao meeting, luma adaptive deblocking filter (JVET-L0414) was adopted [4]. The proposed deblocking filter strength is dependent on the level of non-inloop filtered reconstructed samples. The main motivation is that the distortion of decoded pictures is caused by a quantization process and is displayed through nonlinear process called Electro-Optical Transfer Function (EOTF) in the linear light domain. Additionally, the scene referred video system such as conventional SDR format and one of the HDR formats, Hybrid Log-Gamma (HLG), requires an additional nonlinear process called Opto-Optical Transfer Function (OOTF). The displayed subjective distortion is enhanced by the nonlinear transfer function depending on the video signal level. Such mechanism makes block distortion visible at brighter areas, even if the quantization noise is uniformly distributed at all video level. Especially, for HDR formats, this enhanced distortion becomes salient due to its strong nonlinearity.

In JVET-L0414, the strong deblocking filter is applied for both brighter and dark areas [4]. In essence, for every deblocking boundary average luma level is calculated using:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as shown in Table 1 [1].

|  | p7,0 | p6,0 | p5,0 | p4,0 | p3,0 | p2,0 | p1,0 | p0,0 | q0,0 | q1,0 | q2,0 | q3,0 | q4,0 | q5,0 | q6,0 | q7,0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | P |  |  |  |  |  |  |  |  | Q |  |  |  |  |
|  | p7,3 | p6,3 | p5,3 | p4,3 | p3,3 | p2,3 | p1,3 | p0,3 | q0,3 | q1,3 | q2,3 | q3,3 | q4,3 | q5,3 | q6,3 | q7,3 |  |

Table 1: Red colored samples are used for the calculation of average luma value in VTM6.

The filter strength of the deblocking filter is controlled by the variables β and tC, which are derived from the averaged quantization parameters qPL. The proposed deblocking filter controls the strength of the deblocking filter by adding offset to qPL according to the luma level of the reconstructed samples, as shown below, and in Table 2.

qPL = ( ( QpQ + QpP +1 ) >> 1 ) + qpOffset

| transfer_characteristics == 1 (conventional SDR format) |  | transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL≤0.326 | 1 | LL≤0.608 | 2 | LL≤0.504 | 2 |
| 0.326<LL≤0.878 | 0 | 0.608<LL≤0.685 | 1 | 0.504<LL≤0.612 | 1 |
| 0.878 < LL | 1 | 0.685<LL≤0.837 | 0 | 0.612<LL≤0.822 | 0 |
|  |  | 0.837<LL≤0.910 | 1 | 0.822<LL≤0.926 | 1 |
|  |  | 0.910< LL | 2 | 0.926 < LL | 2 |

Table 2: qpOffset as a function of LL, defined in JVET-L0414

# Proposal

Currently, in VVC, the deblocking filtering can use various filter lengths. It has the possibility to modify 1, 2, 3, 5 and 7 samples on each side. Currently, for LADF, the average luma level is always calculated using the nearest pixels from the boundary on each side, as shown in Fig. 1, which is independent of the filter length. For the long deblocking filter, where 7 samples may be modified from either side, the average luma level calculated using this way may not always provide a robust estimate.

In VVC, before actual deblocking filtering, for each transform and subblock edge, maximum filter length (for side P and side Q) is derived, which specifies the maximum filtering length that can be applied without compromising the parallel processing capability. In this proposal, we derive LL after the maximum filter length is decided for side P and side Q, as defined in the following, where maxP and maxQ are the maximum filter length for side P and side Q.

LL= ( ( p0,0 + pmaxP,0 + p0,3 + pmaxP,3 + q0,0 + qmaxQ,0 + q0,3 + qmaxQ,3) >> 3 ) / ( 1 << iBitDepth ),

|  | p7,0 | p6,0 | p5,0 | p4,0 | p3,0 | p2,0 | p1,0 | p0,0 | q0,0 | q1,0 | q2,0 | q3,0 | q4,0 | q5,0 | q6,0 | q7,0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | P |  |  |  |  |  |  |  |  | Q |  |  |  |  |
|  | p7,3 | p6,3 | p5,3 | p4,3 | p3,3 | p2,3 | p1,3 | p0,3 | q0,3 | q1,3 | q2,3 | q3,3 | q4,3 | q5,3 | q6,3 | q7,3 |  |

Table 3: Red colored samples are used for the calculation of average luma value in proposed method.

As from the Table 3, for each side (side P and side Q), we are using the samples from the farthest corner of the filtered area (marked in red), which may provide better estimate of the average luma values for the filtered area, particularly when the filtered area is large.

# Experimental results

The proposed technique has been implemented on top of VTM-6.0 following SDR and HDR CTC conditions, with LADF enabled both for the anchor and the proposed method. The experimental results are reported for QP values {22, 27, 32, 37}. For SDR, simulations are performed for AI, RA and LDB configurations [2], while HDR simulations are performed for AI and RA configurations [3].

In VTM-6.0, the default LADF parameters, which are signalled at SPS level, are defined for SDR content. So, for HDR simulations, additional configuration parameters are provided, which overwrites the default LADF parameters defined in VTM-6.0.

For Class H1 (HDR-PQ):

LadfNumIntervals		: 5

LadfQpOffset			: 2 1 0 1 2

LadfIntervalLowerBound	: 622 701 856 931

For Class H2 (HDR-HLG):

LadfNumIntervals		: 5

LadfQpOffset			: 2 1 0 1 2

LadfIntervalLowerBound	: 516 626 841 947

The LadfIntervalLowerBound are calculated using scaling (multiplication by 1023) the normalized threshold values defined in Table 2.

Table 4: SDR-CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6 + LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 100% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access |  |  |  |  |
|  | Over VTM6 + LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.07% | 100% | 101% |
| Class A2 | 0.02% | -0.04% | 0.03% | 100% | 101% |
| Class B | 0.01% | -0.06% | 0.06% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.16% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | 0.08% | 100% | 100% |
| Class D | 0.05% | -0.08% | -0.07% | 99% | 100% |
| Class F | 0.00% | -0.01% | 0.05% | 101% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6 + LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.20% | 0.02% | 100% | 102% |
| Class C | 0.05% | -0.14% | -0.19% | 100% | 99% |
| Class E | -0.16% | -0.27% | -0.16% | 101% | 101% |
| Overall | -0.02% | -0.20% | -0.10% | 100% | 101% |
| Class D | 0.02% | -0.31% | 0.11% | 101% | 101% |
| Class F | -0.03% | 0.18% | -0.78% | 100% | 102% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5: HDR-CTC results

From Table 4 and Table 5, it can be observed that for AI and RA configuration, BD Rate performance impact is negligible. For LD in Table 4, for ClassE, BD rate savings can be seen.

Additional results are provided with ALF off in Table 6 and Table 7.

Table 6: SDR-CTC (ALF off)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6+LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 106% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access |  |  |  |  |
|  | Over VTM6+LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | 0.04% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.01% | 0.06% | 0.04% | 100% | 100% |
| Class C | 0.01% | -0.08% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.04% | -0.03% | -0.03% | 100% | 100% |
| Class F | -0.01% | 0.05% | 0.03% | 100% | 100% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B |  |  |  |  |
|  | Over VTM6+LADF |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.01% | 0.18% | 0.02% | 100% | 101% |
| Class E | -0.09% | -0.17% | -0.07% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.07% | -0.51% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 7: HDR-CTC (ALF off)

# References

[1] Bross et al., “Versatile Video Coding (Draft 6),” Joint Video Experts Team, doc. JVET-O2001, Gothenburg, SE, Jul. 2019.

[2]  F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

[3]   A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

[4]  A. Ichigaya, S. Iwamura, S. Nemoto, “CE11: Luma-adaptive deblocking filter (CE11.2.2)”, Document of Joint Video Experts Team, JVET-L0414, Geneva, CH, Oct. 2018.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).