| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0615-v1 |
| --- | --- |

| Title: | Non-CE3: Intra chroma mode coding cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a cleanup of intra chroma mode index coding by using bypass coding of the first bin of cclm_mode_idx. The results for CTC condition are reported below.

CTC results: without context retraining

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.01% | -0.13% | -0.13% | 101% | 102% |
| RA | 0.01% | -0.19% | -0.14% | 101% | 101% |

CTC results: with context retraining

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI |  |  |  |  |  |
| RA |  |  |  |  |  |

# Introduction

In current VTM-6.0, the chroma intra prediction mode coding syntax and specification of Intra chroma prediction mode is shown in the following [1].

| if( CclmEnabled ) |  |
| --- | --- |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |

Table 8-2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode

| cclm_mode_flag | cclm_mode_idx | intra_chroma_pred_mode | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | - | 0 | 66 | 0 | 0 | 0 | 0 |
| 0 | - | 1 | 50 | 66 | 50 | 50 | 50 |
| 0 | - | 2 | 18 | 18 | 66 | 18 | 18 |
| 0 | - | 3 | 1 | 1 | 1 | 66 | 1 |
| 0 | - | 4 | 0 | 50 | 18 | 1 | X |
| 1 | 0 | - | 81 | 81 | 81 | 81 | 81 |
| 1 | 1 | - | 82 | 82 | 82 | 82 | 82 |
| 1 | 2 | - | 83 | 83 | 83 | 83 | 83 |

While the cclm_mode_index uses TR binarization (cMax=2, cRiceParam=0), the binarization of the intra_chroma_pred_mode is done in the following way.

Table 9-80 – Binarization for intra_chroma_pred_mode

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |
| 4 | 0 |

The context selection for the syntax elements are shown in the following.

| Syntax Element | binIdx=0 | 1 | 2 | 3 | 4 | >=5 |
| --- | --- | --- | --- | --- | --- | --- |
| cclm_mode_flag | 0 | na | na | na | na | na |
| cclm_mode_idx | 0 | bypass | na | na | na | na |
| intra_chroma_pred_mode | 0 | bypass | bypass | na | na | na |

Where currently binIdx=0 of cclm_mode_idx and intra_chroma_pred_mode syntax elements share a single context.

# Proposal

In this proposal, we propose to remove the sharing of context for cclm_mode_idx and intra_chroma_pred_mode syntax elements, by using bypass coding for binIdx=0 for cclm_mode_idx, as shown in the following.

| Syntax Element | binIdx=0 | 1 | 2 | 3 | 4 | >=5 |
| --- | --- | --- | --- | --- | --- | --- |
| cclm_mode_flag | 0 | na | na | na | na | na |
| cclm_mode_idx | bypass | bypass | na | na | na | na |
| intra_chroma_pred_mode | 0 | bypass | bypass | na | na | na |

# Experimental results

The proposed technique has been implemented on top of VTM-6.0. The experimental results under CTC condition for AI, and RA configurations are reported [2]. The coding runtime is not reliable.

We have run simulations for two cases:

Case 1: without context re-training of intra_chroma_pred_mode syntax.

Case 2: with context re-training of intra_chroma_pred_mode syntax.

Case 2 is tested, because for the proposed method the context does not rely on combined statistics of intra_chroma_pred_mode and cclm_mode_index anymore. So, the context model may need to change.

CTC results (Case 1)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.16% | -0.25% | 100% | 100% |
| Class A2 | 0.01% | -0.10% | -0.08% | 101% | 103% |
| Class B | 0.01% | -0.12% | -0.14% | 101% | 99% |
| Class C | 0.01% | -0.19% | -0.20% | 101% | 103% |
| Class E | -0.01% | -0.05% | 0.03% | 101% | 107% |
| Overall | 0.01% | -0.13% | -0.13% | 101% | 102% |
| Class D | 0.02% | -0.04% | -0.18% | 103% | 105% |
| Class F | 0.05% | -0.21% | -0.23% | 100% | 100% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.51% | -0.37% | 101% | 100% |
| Class A2 | 0.01% | -0.03% | -0.01% | 102% | 103% |
| Class B | -0.02% | -0.11% | -0.12% | 100% | 100% |
| Class C | 0.02% | -0.15% | -0.10% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.19% | -0.14% | 101% | 101% |
| Class D | -0.03% | -0.35% | -0.12% | 104% | 107% |
| Class F | 0.05% | -0.36% | -0.21% | 101% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

CTC results (Case 2)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.17% | -0.22% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.10% | -0.13% | 101% | 102% |
| Class C | 0.00% | -0.18% | -0.03% | 101% | 103% |
| Class E | 0.00% | -0.08% | 0.04% | 101% | 106% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.05% | -0.13% | 102% | 106% |
| Class F | 0.03% | -0.26% | -0.24% | 101% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.11% | -0.16% | 101% | 101% |
| Class C | -0.03% | 0.00% | -0.09% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.48% | -0.34% | 104% | 107% |
| Class F | -0.04% | -0.34% | -0.27% | 101% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

It can be seen that both the cases provide negligible coding performance impact.

# References

[1] Bross et al., “Versatile Video Coding (Draft 6),” Joint Video Experts Team, doc. JVET-O2001, Gothenburg, SE, Jul. 2019.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).